3GPP TSG-RAN4 Meeting #112bis	R4-2415476

Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Views on adjacent channel co-existence for 14800 - 15350 MHz

Source:	Qualcomm Incorporated

Agenda item:	6.2.4.1

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#112, a WF was agreed in [2] and [3] where several agreements were captured for the 14800 – 15350 MHz frequency range. In this contribution, we provide our preliminary results and views on the different system aspects, RF parameters for BS and UE items for the 14800 – 15350  MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

## 2.1.1	Channel bandwidth

In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB width and availability of the spectrum. Channel bandwidth of 100 MHz has been considered as a representative channel bandwidth in TR 38.921 for 6GHz and 10GHz. Accordingly, it is proposed that higher channel bandwidth (e.g., 200 MHz) should be considered as a baseline for this frequency range.

Proposal 1: RAN4 to consider 200 MHz as a baseline in the adjacent channel coexistence framework.

## 2.4 	UE parameters

### 2.4.1		UE beamforming

In [3] the following was agreed on the adopted model for UE beamforming

- FR1-like UE: - Option 1: 1TX assumption with omnidirectional antenna - Option 2: Two discrete elements following Table 1-1 below with maximum-gain across elements selection criterion. - Option 3: For [N] Tx/Rx, [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA. - Companies to clearly state what value of N and G assumed. - FR2-like UE: - FR2-likUE: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels criterion.

The “FR1-like” option has three options, firstly isotropic assumption, which follows that of TR 38.921 where an isotropic omnidirectional antenna with 0 dBi. Second option considers two discrete antenna elements, each with a maximum antenna gain, and then antenna selection diversity is considered to select the antenna with the maximum gain for a given link between a UE and its serving BS.  Third assumption represents a simplified model to account for the digital beamforming gain. On the other hand, the “FR2-like” option represents a similar model that was adopted in NR Release 15 studies (for more info, reader is kindly referred to TR 38.803).

In this contribution, we will investigate the following options for UE beamforming model:

- Isotropic case  1TX assumption with omnidirectional antenna (i.e., Option 1 in RAN4#112 WF)

- 2 elem. + ASD  Two discrete elements with antenna selection diversity (ASD) (i.e., Option 2 in RAN4#112 WF)

- Isotropic + Digital BF gain model  [G] dBi gain for link of interest regardless the AoA, and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA (i.e., Option 3 in RAN4#112 WF). G is assumed in our contribution to be equal 2dB.

- 2 elem. + ASD + Digital BF gain  Two discrete elements with ASD + [G] dBi gain for link of interest regardless the AoA and 0dB for signals from all non-serving co-channel and adjacent-channel links regardless the AoA (i.e., Option 2 + Option 3 in RAN4#112 WF)

- Analog BF: Two panels with 1x2 array at each panel following Table 1-2 below with maximum gain across panels criterion (i.e., “FR2-like UE” option in RAN4#112 WF)

## 2.2	BS parameters

For the BS AAS parameters, the number of elements considered for the 4400 - 4800 MHz is based on TR 38.803, subclause 5.2.3.2.4, where the total number of antenna elements for macro suburban and macro urban is set to be 96 elements (4x8 subarrays with 3 elements per subarray). Assuming the same coverage metrics for the 14800 - 15350 MHz frequency range as FR1 deployments apply, it is anticipated that the number of antenna elements scale four times with doubling the frequency. 192 elements are typically considered for FR1 deployments, while discussions in the 7125 – 8400 MHz frequency range are in the ballpark of 384 antenna elements. To maintain the same physical area of the antenna array in the 14800 – 15350 MHz frequency range, the number of elements is expected to be larger than 1k elements. Accordingly, RAN4 has agreed in [2] on >1k elements with different AAS configurations.

In [2], 46 dBm for dual polarization for Urban macro was agreed as BS maximum output power. Additionally, the following was agreed regarding the AAS configurations for UMa scenario:

- 3072 AEs dual polarization: - 16x24 array and 4x1 sub-array - 4096 AEs dual polarization: - 8x32 array and 8x1 sub-array - 16x32 array and 4x1 sub-array - 2048 AEs dual polarization: - 8x32 array and 4x1 sub-array - 1536 AEs dual polarization: - 8x16 and 6x1 sub-array - Other options are not precluded.

Assuming that the antenna element gain is set to 6.4 dBi, TRP equals 46 dBm (i.e., antenna losses are captured in the element gain), the EIRP for a given channel bandwidth is given for the different configurations in Table 1. It can be observed that the ballpark of achieved EIRP is similar apart from the 1536 option. Accordingly, we propose for RAN4 to consider one of the options as follows: 3072, 4096, or 2048 total antenna elements, with the subarray configuration and element/ subarray spacing to be further discussed in RAN4.

Table 1 EIRP values for different AAS configurations

| BS AAS configuration | AAS configuration | Maximum EIRP / CHBW [dBm/MHz] |
| --- | --- | --- |
| 3072 | 16x24 array and 4x1 sub-array | 84.3 |
|  | 8x32 array and 8x1 sub-array | 85.5 |
| 4096 | 16x32 array and 4x1 sub-array | 85.5 |
| 2048 | 8x32 array and 4x1 sub-array | 82.5 |
| 1536 | 8x16 and 6x1 sub-array | 81.2 |

Proposal 2: RAN4 to consider one of the options as follows: 3072, 4096, or 2048 total antenna elements, with the subarray configuration and element/ subarray spacing to be further discussed in RAN4.

## 2.5 Preliminary Coexistence results

In this contribution we present our preliminary results based on the set of parameters given in Table 1 below. In addition, we follow the simulation methodology described in Section 4 in [3], where the RF parameters (i.e., ACLR/ACS) are determined based on the degradation cause by adjacent channel interference (ACI).

Table 1 Adjacent channel coexistence for 14Ghz

| ## System Parameters | ## Parameter | ## Value |
| --- | --- | --- |
|  | ## Carrier frequency | ## 14 GHz |
|  | ## Deployment | ## Urban Macro |
|  | ## Channel BW | ## 200 MHz |
|  | ## Number of active UE per TTI | ## 1 |
|  | ## ISD | ## 450 meters |
|  | ## Grid shift | ## Un-coordinated scenario |
|  | ## UE to BS minimum distance | ## 35 meters |
| ## BS | ## BS Antenna Configuration ## (Mg, Ng, M, N, P) | ## (1,1,32, 32,2) |
|  | ## (dv, dh) | ## (0.5λ, 0.5λ) |
|  | ## Antenna element gain | ## 5.5 dBi |
|  | ## Maximum array gain | ## 36 dBi |
|  | ## Antenna element radiation pattern | ## ITU-R M.2101 AAS model |
|  | ## Max Tx conducted power (per polarization) | ## 43 dBm |
|  | ## EIRP | ## ~81.5 dBm |
|  | ## Noise figure | ## 8 dB |
| ## UE | ## UE antenna characteristics | Several UE beamforming settings considered (See Section 2.4.1). |
|  | ## Antenna element gain | ## 5 dBi |
|  | ## Max conducted power | ## 23 dBm |
|  | ## Noise figure | ## 11 dB |
|  | ## Target uplink SNR | ## 15 dB |
|  | ## Uplink power control | ## Adjusted to meet UL SNR target |

Our main primarily objective is to derive ACLR/ACS values based on the adjacent channel coexistence. In this regard, we investigate the impact of different UE beamforming models/ assumptions on the experienced throughput loss for the cell-edge and mean user performance in UMa scenarios. In our view, UMa scenario is the bottleneck and so whatever observations and conclusions derived from this scenario, will be applicable to the indoor and dense urban scenarios.

Figure 3 shows the perceived UE antenna gain considering the different UE beamforming options. We can observe the delta of 2 dB between the isotropic case and the isotropic + digital BF model, based on the assumed value of G. Additionally, the delta of around 2 dB between the 2 elements + ASD model and that of the 2 elements + ASD + Digital BF gain model, which are capped by 5 dBi and 7 dBi, respectively. Finally, it can be observed the dominance of the analog beamforming (i.e., FR2 like) option a since it has the higher beamforming gain. In addition, we can see that the two extremes are the isotropic case and the analog BF (i.e., FR2 like) options, which be regarded as the two extreme cases for the case of UE without beamforming and UE with beamforming. Accordingly, we believe that the two proposed models (i.e., blue and red) for FR1 and FR2 provide a better compromise between feasibility, performance, and complexity.

Observation 1: The isotropic 0 dBi model and the analog BF (i.e., FR2 like) model represents the two extremes for the UE beamforming models with other FR1 variants provide performance falling between those two extremes.

Figure 1 CDF of UE gain [dBi] for different UE beamforming models

The SINR CDF for the coordinated and uncoordinated case for different UE beamforming models for ACIR = 15 dB is shown in Figure 2 and Figure 3, for downlink and uplink, respectively. Note that the ACIR value is an arbitrary value that has been selected to investigate the system level performance for a given ACLR/ACS. Firstly, one can observe the impact of the deployment scenario on the UE models with directionality modelled, i.e., blue, green, and red curves, where the solid SINR curves (i.e., uncoordinated) is higher than that of the coordinated case, attributing to the fact of the higher adjacent channel interference form the coordinated case for such scenarios. Additionally, the performance difference observed from the UE gains CDF is preserved in the SINR performance, where we see the superiority of the analog BF (i.e., FR2 like) case in comparison with other models. However, one should observe that there is around 1 dB difference between the analog BF case and the 2 elem+ASD+Dig BF model, which shows that even for FR1 like implementations, the performance of FR1-like UE will be comparable to that of the FR2 like UE.

Figure 2 DL SINR (co-channel plus adjacent channel) CDF assuming ACIR = 15 dB. Solid (dashed) present uncoordinated (coordinated) deployment.

Figure 3 UL SINR (co-channel plus adjacent channel) zoomed-in CDF assuming ACIR = 15 dB. Solid (dashed) present uncoordinated (coordinated) deployment.

Observation 2: From system level performance, FR1-like UE beamforming models can provide comparable performance to the FR2-like UE beamforming model.

In Figure 4 we plot the required ACIR to meet the 5% throughput loss performance for the different UE beamforming settings and both coordinated and uncoordinated deployments. As observed, the isotropic case provides the highest DL (UL) ACIR value of 27 (18) dB while the analog BF case (i.e., FR2 like) provides the lowest ACIR  around 23 dB (14 dB) (option a). We can also observe the system level gains achieved in the different FR1-like beamforming models, where we can see that the 2 elem.+ASD+DigBF model can even result in lower ACIR values compared toe FR2 coordinated case. Based on the below figures, we can observe that having directionality at the UE considered, system level gains in term of adjacent channel coexistence can be realized.

Figure 4 DL ACIR values to meet 5%TP loss for different UE beamforming models.

Figure 5 UL ACIR values to meet 5%TP loss for different UE beamforming models.

Observation 3: There is an observed performance enhancement when considering directionality at the UE via single elements compared to the isotropic 0 dBi assumption.

Observation 4: With isotropic model, DL and UL ACIR are observed to be around 27 and 18 dB while for analog BF model (i.e., FR2 like), DL and UL ACIR are observed to be around 23 and 14 dB.

Observation 5: Different FR1-like UE beamforming model (two elements + antenna selection + digital BF model) can provide close performance to FR2-like UE beamforming model.

It is worth noting that the lower ACIR values are a byproduct of the enhanced performance resulting from large antenna arrays deployed at the BS. In comparison to Rel-17 work done on 10GHz, number of antennas at the BS side was assumed to be 16x8, whereas in [2], at least 1k element was considered, resulting in around 9 dBi difference in the directivity.

Observation 6: Large antenna arrays at the BS enables feasible adjacent channel coexistence via directing the wanted signal to the target destination and reducing the adjacent channel interference.

Focusing on the provided ACIR performance for the uplink scenario given in Figure 5, the worst-case performance provides around ACIR = 18 dB, which represents the combined effects of the UE ACLR and the gNB ACS. Additionally, UEs in the 14800 – 15350 MHz frequency range will anyway need to comply with the regulatory occupied bandwidth requirement, that specifies the maximum bandwidth within which emissions do not exceed a certain percentage of the total emissions. For NR, it is equal to the channel bandwidth with a maximum of 1% of emissions allowed outside of it (0.5% on each side), which result in around 24 dBc UE ACLR requirement.

Observation 7: Due to the regulatory occupied bandwidth requirement, required UE ACLR is technically equal to 24 dBc.

# Conclusion

In this paper we have shared our views on the RF parameters for 14800 - 15350 MHz frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to consider 200 MHz as a baseline in the adjacent channel coexistence framework.

Proposal 2: RAN4 to consider one of the options as follows: 3072, 4096, or 2048 total antenna elements, with the subarray configuration and element/ subarray spacing to be further discussed in RAN4.

Observation 1: The isotropic 0 dBi model and the analog BF (i.e., FR2 like) model represents the two extremes for the UE beamforming models with other FR1 variants provide performance falling between those two extremes.

Observation 2: From system level performance, FR1-like UE beamforming models can provide comparable performance to the FR2-like UE beamforming model.

Observation 3: There is an observed performance enhancement when considering directionality at the UE via single elements compared to the isotropic 0 dBi assumption.

Observation 4: With isotropic model, DL and UL ACIR are observed to be around 27 and 18 dB while for analog BF model (i.e., FR2 like), DL and UL ACIR are observed to be around 23 and 14 dB.

Observation 5: Different FR1-like UE beamforming model (two elements + antenna selection + digital BF model) can provide close performance to FR2-like UE beamforming model.

Observation 6: Large antenna arrays at the BS enables feasible adjacent channel coexistence via directing the wanted signal to the target destination and reducing the adjacent channel interference.

Observation 7: Due to the regulatory occupied bandwidth requirement, required UE ACLR is technically equal to 24 dBc.

# Reference

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2414283, “WF on 15GHz BS parameters”, ZTE, RAN4#112, Maastricht, Netherlands.

[2] R4-2414284, “WF on 15GHz UE parameters”, Qualcomm, RAN4#112, Maastricht, Netherlands.

[4]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.