3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415477

Hefei, China, 14th – 18th October 2024

Agenda Item:	5.21.3

Source: 	Nokia

Title: 	BS RF specification impact for MmWave UE spurious emission

Document for:	Approval

# 1.	Introduction

The new WI on mmWave UE spurious emission was approved at TSG RAN#105 [1]. One of the objectives of this WI is:

- For BS, conduct necessary modifications on protection of Earth Exploration Satellite Service requirements if any

This contribution provides the necessary changes of the BS RF requirements in TS 38.104 [2] and TS 38.141-2 [3] on mmWave UE spurious emission.

# 2.	Specification Impact

The following tables are included in TS 38.104.

Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; -9 dBm (Note 2); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027.. Table 9.7.5.3.3.1-1: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027..

And similar tables are included in TS 38.141-2.

Table 6.7.4.5.2.4.1-1: BS radiated limits for protection of EESS Frequency range; Measurement filter centre frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; 23.7 – 23.9 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; 23.7 – 23.9 GHz; -9 dBm (Note 2); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027.. Table 6.7.5.4.5.2-1: BS spurious emissions test limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027. NOTE 2: This limit applies to BS brought into use after 1 September 2027.. Table 7.7.5.2-3: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 September 2027.NOTE 2: This limit applies to BS brought into use after 1 September 2027..

EU Decision 2020-590 [4] has decided to implement −39 dBW/200 MHz (equivalent to -9dBm/200MHz) on 23.6 – 24 GHz from 1 January 2024 (instead of 1 September 2027). Therefore, the dates of the notes in the tables above need to be changed from 1 September 2027 to 1 January 2024 to align with EU Decision 2020-590.

# 3.	Conclusion

This contribution provides the necessary changes of the BS RF requirements in TS 38.104 and TS 38.141-2 on mmWave UE spurious emission. The necessary changes are provided below.

The following tables need to be changed in TS 38.104.

Table 9.7.4.3.4.1-1: OBUE limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; -9 dBm (Note 2); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use on or before 1 September 2027 inr other countries. NOTE 2: This limit applies to BS brought into use after 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use after 1 September 2027 in other countries.. Table 9.7.5.3.3.1-1: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use on or before 1 September 2027 inr other countries. NOTE 2: This limit applies to BS brought into use after 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use after 1 September 2027 in other countries..

And similar tables need to be changed in TS 38.141-2.

Table 6.7.4.5.2.4.1-1: BS radiated limits for protection of EESS Frequency range; Measurement filter centre frequency range; Limit; Measurement Bandwidth. 23.6 – 24 GHz; 23.7 – 23.9 GHz; -3 dBm (Note 1); 200 MHz. 23.6 – 24 GHz; 23.7 – 23.9 GHz; -9 dBm (Note 2); 200 MHz. NOTE 1: This limit applies to BS brought into use on or before 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use on or before 1 September 2027 inr other countries. NOTE 2: This limit applies to BS brought into use after 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use after 1 September 2027 in other countries.. Table 6.7.5.4.5.2-1: BS spurious emissions test limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use on or before 1 September 2027 inr other countries. NOTE 2: This limit applies to BS brought into use after 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use after 1 September 2027 in other countries.. Table 7.7.5.2-3: Limits for protection of Earth Exploration Satellite Service Frequency range; Limit; Measurement Bandwidth; Note. 23.6 – 24 GHz; -3 dBm; 200 MHz; Note 1. 23.6 – 24 GHz; -9 dBm; 200 MHz; Note 2. NOTE 1: This limit applies to BS brought into use on or before 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use on or before 1 September 2027 inr other countries. NOTE 2: This limit applies to BS brought into use after 1 January 2024 in countries following EU Decision 2020-590 and applies to BS brought into use after 1 September 2027 in other countries..

# References

[1]	RP-242371, “New WID on mmWave UE spurious emission”, China Unicom, Deutsche Telekom, Vodafone, CATT, Huawei, HiSilicon, OPPO, Qualcomm, Xiaomi, ZTE.

[2]	3GPP TS 38.104 v18.7.0: “NR; Base Station (BS) radio transmission and reception”.

[3]	3GPP TS 38.141-2 v18.7.0: “NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing”.

[4]	EU Decision 2020-590, “Commission Implementing Decision (EU) 2020/590 of 24 April 2020 amending Decision (EU) 2019/784 as regards an update of relevant technical conditions applicable to the 24,25-27,5 GHz frequency band”.