3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415482

Hefei, China, 14th – 18th October 2024

Agenda Item:	6.2.4.2

Source: 	Nokia

Title: 	BS antenna parameters for 14800 to 15350 MHz frequency range

Document for:	Approval

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

This topic was discussed at TSG RAN4#112 and the WF was agreed [2].

This contribution further discusses the BS antenna and simulation parameters based on the agreed WF and the simulation results provided in [3]and [4].

# 2.	Discussion

2.1		BS antenna array size in urban macro scenario

It is stated in the agreed WF.

Issue 2-1: BS antenna array and sub-array size - Proposals - Option 1: BS array size as 1536 and the sub-array size as 4 (CATT) - Option 2: At least 1k elements (Qualcomm) - Option 3: 1536=16x24 (Sub Array size 4) per polarization. (Nokia) - Option 4: Two options based on the simulation assumptions (dual polarization): (Ericsson) - 2048 AEs: 8x32 array and 4x1 sub-array - 1536 AEs: 8x16 and 6x1 sub-array - Recommended for further discussions - 3072 AEs dual polarization : - 16x24 array and 4x1 sub-array - 4096 AEs dual polarization: - 8x32 array and 8x1 sub-array - 16x32 array and 4x1 sub-array - 2048 AEs dual polarization: - 8x32 array and 4x1 sub-array - 1536 AEs dual polarization: - 8x16 and 6x1 sub-array - Other options are not precluded.

The simulation results in [3] and [4] have shown that the 16x24 array and 4x1 sub-array can provide satisfactory DL and UL coverage in urban macro scenario with 450 m inter-site distance and 20% indoor UE ratio, either with 1 or 3 UE per slot. Smaller BS antenna array and sub-array size would reduce the inter-site distance (meaning existing sites in 3.5 GHz may not be reused) and degrade indoor UE coverage. On the other hand, larger BS antenna array with the similar sub-array size would provide enhanced coverage but at the expense of implementation complexity with the larger number of transceivers in the antenna array. Therefore, it is proposed to adopt 16x24 array and 4x1 sub-array as a trade-off between cell coverage and implementation complexity.

2.2		BS antenna array size in indoor scenario

It is stated in the agreed WF.

Issue 2-2: BS antenna array size - Proposals - Option 1: Prioritize 4x4 array for Indoor scenarios (Ericsson) - Recommended WF - Further discuss the proposals

Simulation results from previous coexistence studies as recorded in TR 38.803 [5] have shown that indoor scenario is not the dominant case for the required ACIR to achieve the below 5% average and 5%-tile throughput losses caused by the interfering system in the adjacent bands. Therefore, there is no need to prioritize 4x4 array for indoor scenario in the coexistence studies, as the required simulation time for indoor scenario is not as long as that for urban macro scenario.

# 3.	Conclusion

This contribution further discusses the BS antenna and simulation parameters based on the agreed WF at TSG RAN4#112 and the obtained simulation results.

Proposal 1: Adopt 16x24 array and 4x1 sub-array as the BS antenna array size in urban macro scenario.

Proposal 2: No need to prioritize 4x4 array for indoor scenario in the coexistence studies.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2414283, “WF on 15GHz BS parameters”, ZTE.

[3]	R4-2412589, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range”, Nokia.

[4]	R4-2415480, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 3 UE per slot”, Nokia.

[5]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.