3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415483

Hefei, China, 14th – 18th October 2024

Agenda Item:	6.2.5

Source: 	Nokia

Title: 	On additional information required by ITU WP5D

Document for:	Discussion

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. One of the requests from ITU-R WP5D in [2] is:

d) IMT base station (inclusive of its AAS-related parameters) and IMT user equipment technical parameters for analyzing adjacent band compatibility of terrestrial IMT systems with other co-primary services in the ITU Radio Regulations. WP 5D also seeks information about how the AAS performs in adjacent bands i.e. does its performance drops down to that of a single element or still some array features remain. If the latter is true how far in frequency should one assume the continuity of array features even with some degradation?

5 contributions related to this topic [3-7] were submitted to TSG RAN4#112 but they were not discussed in detail due to time constraint. This contribution provides further discussion on this topic based on the contributions in last meeting.

# 2.	Discussion

The 5 contributions related to this topic were summarized in the topic summary [8] as follows:

| T-doc number | Company | Title | Proposals / Observations |
| --- | --- | --- | --- |
| R4-2411021 | Spark NZ Ltd | Text Proposals on MIMO models and PA nonlinearity impacts and ACLR | TP to Clause 7.1.4, 7,2 and 8 Observation 1: Is the method to compute an array ACLR as defined currently in 3GPP specification still correct? If not, what improvements are needed? Observation 2a: There is no definition of an array correlation factor. Is it the correlation of the in-band signal with out of band signal. But the in-band signal, Pinband (θ,φ), is spatially sensitive and as discussed below the out of band signal , P oob (θ,φ), may also be spatially sensitive. Then the array correlation  is also spatially sensitive may be defined as: Observation 2b: This also means that θφ) defined above is a function of frequency separation from the band centre. In this case best to denote this as: Where f is the frequency separation from band centre Observation 3: In order to do this, we would therefore need to characterise and investigate the constituent components of P oob (θ,φ) as a function of f Observation 4: The change in array performance due to a different vertical and horizontal spacing arising from the lower and upper adjacent bands is not large enough to warrant a conclusion that the array drops down to a single element in adjacent bands. Observation 5: The choice of a PA behavioural model validated by measurements is needed. Ref [10,11] provide a good framework to model these non-linearities via polynomial models. Observation 6: The differences in the linear responses of the parallel PAs have a large impact on the system performance. The nonlinear terms do not necessarily adopt the digital predistortion as easily and efficiently. Therefore, the impact of nonlinear power amplifiers is nontrivial both in band and out of band. However, to do this we would need a model for the differences in the PA models in an AAS. Observation 7: PA non linearities may result in the array beamforming in different spatial directions than the desired signal. The impact of Power Amplifier non linearities when MIMO arrays are used vary with single user and multiuser systems. In multiuser beamforming case the distortion resulting from non-linearities is beamformed into distinct spatial directions that are different from the desired signal direction. Observation 8: It is important to specify a bandwidth and a model for the BPF. Observation 9: Array ACLR and antenna ACLR are different quantities. As given in observation 1, there is a need to determine if the computation of array ACLR as given in the specifications is still accurate. Furthermore, when the number of users increases, the array ACLR becomes almost isotropic as the channel is now beamforming in many directions. Therefore, out of band distortions are also influenced by MU MIMO; this needs the numbers of simultaneous users. Proposal 1: The text proposals in this document be included in TR 38 922, section 7 Proposal 2: AS WP 5D has started the process of technical calculations, it is best to advise them of how to calculate ethe BF weights only rather than the results as they would be scenario specific. A text on BF weights is included in section 3 of this contribution. This text could be sent in an LS to WP 5D. Proposal 3: There is a need to define array correlation factor so that it may be computed. Equation (1) may be used. Proposal 4: Define the constituent components of P oob (θ,φ) and their dependence on f Proposal 5: There is a need to investigate array ACLR and see that its computation method in the standards is still the correct way. Proposal 6: An array will not drop down to a single element in adjacent bands. Nor its performance will drop down to a sub array. Proposal 7: Power Amplifier non linearities should be carefully modelled when considering adjacent channel impacts when MIMO arrays are used. The choice of a PA model should be agreed (say via polynomial models), then it may be validated by measurements and also agree on a model for the differences in the parameters for the distributed PAs. Proposal 8: We may assume for the sake of simplicity there are no differences in the PA model parameters in a distributed PA AAS. Proposal 9: A model for the BPF needs to be agreed Proposal 10: MU-MIMO precoder type and numbers of simultaneous users need to be agreed and could impact out of band distortions to fall in un-intended directions. |
| R4-2411093 | CATT | TP for other issues (Adjacent channel modelling) | TP to Clause 7.x Adjacent channel modelling |
| R4-2411873 | Ericsson | On the topic of additional information requested by ITU-R WP 5D | TP to Clause 7.2 Observation 1: Support for modelling array antenna gain adjacent carrier have been documented since Rel-13 in TS 37.840, while relevant parameters for the adjacent channel regions have not been defined. Observation 2: The array antenna average gain varies between peak sub-array gain and peak full array gain (including array factor) depending on signal correlation properties. Observation 3: The modelled array average gain characteristics will depend on the carrier bandwidth. Observation 4: When adjacent channel gain is modelled in sharing studies it is essential to use correct information for power fed to the array antenna in conjunction with information on array correlation properties. In a sharing situation both power and gain will affect the produced EIRP level. Observation 5: Based on our simulation results, there is no need to change the beamforming assumptions in Recommendation ITU-R M.2101. The differences introduced by ZF beamforming technique, compared to the model in M.2101, are minor. The simulation results show minimal impact on the overall AAS BS gain towards a number of potential interfered-with receivers and on the BS antenna gain integrated over parts of or the entire sphere around the antenna. Proposal 1: In TR 38.922 capture information on how to model adjacent channel EIRP using the array antenna model and capture relevant model for correlation factor in the adjacent channel regions. Proposal 2: Based on outcome of beamforming modelling evaluation, the current modelling approach remains effective for its intended purposes. Modifying the existing beamforming model is not necessary, as the variations observed are insignificant and do not justify the complexity of updating the model. Capture simulation results and corresponding conclusion in TR 38.922. Proposal 3: Include information relevant for adjacent channel EIRP modelling and beamforming evaluation in scheduled LS responses to ITU-R WP 5D. |
| R4-2412592 | Nokia | Study of AAS performance in adjacent bands | Observations from simulation results: 1) With 400 MHz channel bandwidth at the 15 GHz carrier frequency, the composite array radiation patterns formed by the AAS in the adjacent bands are similar to that formed at the designed carrier frequency, even with the adjusted horizontal/vertical radiating sub-array spacing and vertical element separation in sub-array. Therefore, the array will continue to perform in the adjacent bands, and it is not correct to assume that its performance will drop down to that of a single element or that of a sub-array. 2) The average and 5%-tile downlink and uplink throughput losses are similar for the three simulated cases with the composite array radiation patterns formed using the designed or adjusted horizontal/vertical radiating sub-array spacing and vertical element separation in sub-array. |
| R4-2412713 | ZTE Corporation, Sanechips | Discussion on other issues in ITU-R LS | In this contribution, we share some initial views on other questions raised in the coming ITU-R LS. |

It is proposed in [3] that power amplifier non-linearities should be carefully modelled when considering adjacent channel impacts when MIMO arrays are used, while it is pointed out in [7] that the overall radiation pattern of the IMD3 products due to the non-linearization characteristics of the transmitters has been studied and record in clause 5.3.2 of TR 37.842 [9] as follows:

### 5.3.2 UE specific beamforming simulation assumptions In case of UE specific beamforming, the AAS BS may generate beams, each pointing to a specific UE. The overall radiation pattern of the beams can be derived as: where is the element pattern defined in table 5.3.3.1-1, and and are the array factors defined in table 5.3.2-1. Due to the non-linearization characteristics of the transmitters, the 3rd order intermodulation productions generated by the beams may be pointed to different directions as the weighting vector for each of the beams are involved in the intermodulation process, and combinations of any three of the weighting vectors generated a different direction for the 3rd order intermodulation product. The overall radiation pattern of the IMD3 products can be derived as: where is number of IMD3 beams generated, and . Please be noted that the triplet (p, q, l) is order sensitive and they represent any of the three UE specific beams with an intermodulation relation of p+q- l. Please also be noted that p, q, l can take the same value. Therefore: = and is the weighting vector for the IMD3 products generated by the three beams and: = The weighting factor above is for the IMD3 products which is generated by+-, where , , and represent any frequency component in beam p, q, l correspondingly. Based on the radiation pattern of wanted signal and IMD3 products, the ACLR pattern can be derived as: + where is the ACLR performance of each transmitter.

Therefore, power amplifier non-linearities have been included in the current AAS model for UE specific beamforming with multiple active UE in the cell. The simulation assumptions with 3 UE per slot have been agreed for 14800 to 15350 MHz frequency range [10], and simulation results using the UE specific beamforming simulation assumptions above are provided in [11].

Besides, it is proposed in [5] to capture information on how to model adjacent channel EIRP using the array antenna model and capture relevant model for correlation factor in the adjacent channel regions, while it is pointed out in [7] that the impact on the system performance degradation with different correlation levels between transmitters has been evaluated and recorded in Annex B of TR 37.840 [12] which is summarized in clause 6.3.1.1 as follows:

2. The impact of correlation level to the system coexistence is evaluated. Simulation results in Case 1a(AAS to Legacy) and Case 1b(AAS to AAS) show that different correlation levels have little impact on the throughput loss due to the fact that the dominant source of adjacent channel interference is due to UE ACS.

Therefore, impact of correlation level to the system coexistence has been evaluated in the AAS SI, and simulation results have shown that different correlation levels have little impact on the throughput loss.

# 3.	Conclusion

This contribution has provided further discussion on AAS performance in adjacent bands based on the contributions in last meeting.

Observations:

1)	Power amplifier non-linearities have been included in the current AAS model for UE specific beamforming with multiple UE.

2)	Impact of correlation level to the system coexistence has been evaluated in the AAS SI, and simulation results have shown that different correlation levels have little impact on the throughput loss.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP5D.

[3]	R4-2411021, “Text Proposals on MIMO models and PA nonlinearity impacts and ACLR”, Spark NZ Ltd.

[4]	R4-2411093, “TP for other issues (Adjacent channel modelling)”, CATT.

[5]	R4-2411873, “On the topic of additional information requested by ITU-R WP 5D”, Ericsson.

[6]	R4-2412592, “Study of AAS performance in adjacent bands”, Nokia.

[7]	R4-2412713, “Discussion on other issues in ITU-R LS”, ZTE.

[8]	R4-2412822, “Topic summary for [112][120] FS_NR_IMT_part2”, Moderator (Nokia).

[9]	3GPP TR 37.842 v13.3.0, “Radio Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)”.

[10]	R4-2414303, “TP to TR 38.922: Revisions of system level simulation assumptions for study on IMT parameters for 14800 to 15350 MHz frequency range”, Nokia, Fujitsu.

[11]	R4-2415480, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 3 UE per slot”, Nokia.