3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415484

Hefei, China, 14th – 18th October 2024

Agenda Item:	6.4.2

Source: 	Nokia

Title: 	UE RF requirements for inter-band NR CA/DC with 3MHz CBW

Document for:	Approval

# 1.	Introduction

The revised WI for NR channel BW less than 5MHz for FR1 Phase 2 was approved at TSG RAN#104 [1]. One of the objectives of this WI is to:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101

This topic was discussed at TSG RAN4#112, and the Way Forward was agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

Two issues were FFS in the agreed WF as follows:

### Issue 2-2: UE capability signaling - Options for further discussion - Option 1: Request RAN2 to introduce UE capability signaling to enable less than 5MHz CBW operation for CA/DC - The following Rel-18 capabilities for NR less than 5MHz CBW operation need to be extended or new capabilities defined to enable CA/DC operation scenario with one or more component carriers with less than 5MHz CBW - [support12PRB-CORESET0-r18] - [support3MHz-ChannelBW-Asymmetric-r18] - [support3MHz-ChannelBW-Symmetric-r18] - [support5MHz-ChannelBW-20PRB-CORESET0-r18] - [support12PRB-CORESET0-GSCN-41637-r18] - UE capability signalling granularity is FFS - The respective UE capabilities shall be early-implementable from Rel-18 - The details including whether to update existing Rel-18 capabilities or introduce new Rel-19 capabilities are up to RAN2 - Option 2: Do not introduce new UE capability signaling to enable less than 5MHz CBW operation for CA/DC - RAN4 to define BCSs for band combination CA_n100A-n101A or combinations with other band where 3 MHz is supported as usual where 3MHz / 5MHz channel bandwidths are listed for each band. BCS5 with supportedMinBandwidthUL-r17 set to 5 MHz can be used to indicate that a UE supports CA channel BWs other than 3 MHz for a given band. - RAN2 specs need to remove “This feature is only applicable to single-carrier operation from Rel-19. RAN2 to confirm no NBC issues. - No new RAN2 UE capability signaling is required.” - Other options are not precluded ### Issue 2-4: Sync raster applicability - Agreement - For band n100 - SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-3 - SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-3 - Whether and how to capture the respective notes in the specification can be decided subject to the decisions on UE capability signaling in the WI RF maintenance stage. - The candidate CR text - Option 1: “SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel bandwidth or 20 PRB transmission bandwidth configuration within 5 MHz channel bandwidth can be configured only in band n100 with SS Block frequency position defined in Table 5.4.3.1-3.” - Other options not precluded

## Issue 2-2: UE capability and network signalling

Currently, UE capability signalling related to less than 5 MHz as well as 12 PRB or 20 PRB transmission bandwidth for 3MHz or 5MHz CBW are specified in TS 38.306 [3] as follows:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |
| support5MHz-ChannelBW-20PRB-CORESET0-r18 Indicates whether the UE supports short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS, and the reception of 20 PRB CORESET0. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 [2]. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this feature supports configuration of 20 PRB BWP operation. | UE | No | FDD only | FR1 only |
| support12PRB-CORESET0-GSCN-41637-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 [2]. NOTE: The UE supporting this FG supports configuration of 12 PRB BWP operation. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. | UE | No | FDD only | FR1 only |

It can be seen from the yellow highlighted texts above that the current UE capability signalling are only applicable to single-carrier operation. Therefore, UE capability signalling for less than 5 MHz CBW operation for CA/DC need to be defined in TS 38.306, which was added to the impacted specification list in the revised WID in [1]. Extended Rel-18 UE capability signalling for less than 5 MHz CBW operation for CA/DC is an alternative to allow the respective UE capabilities to be early-implementable from Rel-18. The UE capability signalling details should be discussed and decided in RAN2.

Since there already exists UE capability signalling related to less than 5 MHz, if RAN4 is to define BCSs for band combination CA_n100A-n101A or combinations without new RAN2 UE capability signaling, there would create ambiguity due to the mix-and match approach between RAN2 signaling and RAN4 BCSs.

## Issue 1-7: Sync raster applicability for Scell with < 5MHz CBW for CA_n100-n101

The candidate CR text in option 1 should be slightly modified as follow because there are currently two SS Block frequency positions defined in Table 5.4.3.1-3.

“SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel bandwidth or 20 PRB transmission bandwidth configuration within 5 MHz channel bandwidth can be configured only in band n100 with SS Block frequency positions defined in Table 5.4.3.1-3.”

# 3.	Conclusion

This contribution has provided proposals to progress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: UE capability signalling for less than 5 MHz CBW operation for CA/DC should be defined in TS 38.306. The UE capability signalling details should be discussed and decided in RAN2.

Proposal 2: Adopt a slightly modified version of the candidate CR text: “SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel bandwidth or 20 PRB transmission bandwidth configuration within 5 MHz channel bandwidth can be configured only in band n100 with SS Block frequency positions defined in Table 5.4.3.1-3.”

# References

[1]	RP-241622, “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel Corporation.

[2]	R4-2414451, “WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2”, Intel.

[3]	3GPP TS 38.306 v18.3.0: “NR; User Equipment (UE) radio access capabilities”.