3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415486

Hefei, China, 14th – 18th October 2024

Agenda Item:	6.23.3

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#105 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#112, and the Way Forward proposals were agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

2.1	Manufacture declaration on LP-WUS

It is stated in the agreed WF as follows:

- FFS on following options: - Option 1: Set the LP-WUS power boosting a complete manufacture declaration feature, including whether supporting LP-WUS power boosting and the supported boosting level. - Option 2: Minimum power boosting level in core specification together with manufacturer declaration in the conformance test specification.

Currently, the RE power control dynamic range requirement for NR BS is specified in TS 38.104 [3] as follows:

### 6.3.2 RE power control dynamic range #### 6.3.2.1 General The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) for a specified reference condition. For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band. #### 6.3.2.2 Minimum requirement for BS type 1-C and BS type 1-H RE power control dynamic range: Table 6.3.2.2-1: RE power control dynamic range Modulation scheme used; RE power control dynamic range (dB). on the RE; (down); (up). QPSK (PDCCH); -6; +4. QPSK (PDSCH); -6; +3. 16QAM (PDSCH); -3; +3. 64QAM (PDSCH); 0; 0. 256QAM (PDSCH); 0; 0. 1024QAM (PDSCH); 0; 0. NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station..

In table 6.3.2.2-1 above, the RE power control dynamic range requirement is specified depending on the modulation schemes and physical channels. The RE power up and down range for each modulation scheme was derived by considering the relation between RE power boosting/de-boosting and other RF requirements like UEM, ACLR and EVM, as explained in TR 38.817-02 [4].

Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new core requirement shall be added for the dynamic range for LP-WUS/LP-SS. In table 6.3.2.2-1 above, the RE power up and down range for modulation order higher than 16QAM is 0 dB considering the impact on PA with the higher PAPR of higher order modulation. It is shown in Figure 1 below that the PAPR of LP-WUS/LP-SS is more like 64QAM than 16QAM. Therefore, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB.

(a) LP-WUS (bit sequence 1101) using ZC sequence with M=1

(b) LP-WUS (bit sequence 1101) using ZC sequence with M=2

(c) LP-WUS (bit sequence 1101) using ZC sequence with M=4

(d) LP-WUS (bit sequence 1101) using M sequence with M=1

(e) LP-WUS (bit sequence 1101) using M sequence with M=2

(f) LP-WUS (bit sequence 1101) using M sequence with M=4

(g) Typical QAM signals

Figure 1: PAPR comparison of LP-WUS and typical QAM signals

On the other hand, support of higher levels of LP-WUS power boosting will increase BS implementation complexity, like NB-IoT power boosting in TS 36.141 [5] and TS 38.141-1 [6], BS manufacturers should be allowed to declare the supported LP-WUS power boosting level in the manufacturer declarations for BS supporting LP-WUS operation to differentiate their product performance.

2.2	Concept of LP-WUS dynamic range/power boosting

It is stated in the agreed WF as follows:

- FFS on following options: - Introduce EPRE ratio to define LP-WUS power boosting, since it has the merit that power boosting does not vary with different CBWs. - Option 1: The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs). - Option 2: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of an LP-WUS RE (averaged power per RE based on all LP-WUS REs within a NR transmission bandwidth configuration) and the average power of a non-LP-WUS RE (averaged power from all REs from the NR carrier excluding the LP-WUS REs). - Adopt following definition rather than introduce EPRE ratio. - Option 3: The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

The main issue of Option 1 or 2 is that there may not be any non-LP-WUS RE in the NR carrier for small channel bandwidth, and in this case the concept cannot be applied as the average power of non-LP-WUS RE is zero. Therefore, the definition of LP-WUS dynamic range/power boosting in RAN4#110bis approved WF R4-2406140 should continue to be used to avoid potential confusion among the various requirements related to power boosting, unless the issue above can be solved.

2.3	Whether to preclude small CBW for consideration of LP-WUS power boosting

It is stated in the agreed WF as follows:

- FFS on following options: - Option 1: Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting. - Option 1-1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting. - Option 1-2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded. - Option 1-3: other option does not excluded. - Option 2: No need to preclude any CBW, only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement.

On the other hand, it was agreed at RAN1#117 as follows [7]:

Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2

Therefore, RAN1 has not precluded LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz. Besides, as discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to preclude small CBW for LP-WUS power boosting if BS manufacturers declare to support power boosting with small CBW for LP-WUS operation.

2.4	On applicable BS type for LP-WUS

It is stated in the agreed WF as follows:

- FFS whether all available BS types can be considered for LP-WUS due to the support of LP-WUS is optional and the power boosting level could be up to manufacturer declaration. - FFS whether only conductive based BS RF requirements should be defined for LP-WUS:

As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to set restriction on applicable BS types for LP-WUS if BS manufacturers declare to support power boosting with LP-WUS operation for their BS types.

2.5	On minimum value for LP-WUS power boosting

It is stated in the agreed WF as follows:

- FFS on the following options: - Option 1: If power boosting will be defined as EPRE ratio, use 3dB as minimum requirement. - Option 2: If power boosting will be defined as dynamic range, based on existing specification for OOK-1 waveform, the core requirement for the dynamic range should be limited to 3dB. - Option 3: Not to introduce minimum value for LP-WUS power boosting

As discussed in section 2.3 above, it was agreed at RAN1#117 to support 11 PRBs for LP-WUS and LP-SS with SCS 30kHz. The power degradation of RBs other than LP-WUS signal for this setting with different LP-WUS power boosting levels are provided in Table 1 below, which shows that the power reduction for other RBs because of LP-WUS power boosting varies between 0.1 dB to 1.1 dB depending on the power boosting level and channel bandwidth.

Table 1: Impact of LP-WUS power boosting

| LP-WUS power boosting (dB) | SCS (KHz) | Number of RBs | LP-WUS RBs | Rated carrier output power (dBm) | Channel bandwidth (MHz) | Power reduction for other RBs (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 30 | 162 | 11 | 46 | 60 | -0.32687277 |
| 3 | 30 | 189 | 11 | 46 | 70 | -0.27568025 |
| 3 | 30 | 217 | 11 | 46 | 80 | -0.237165576 |
| 3 | 30 | 245 | 11 | 46 | 90 | -0.208095127 |
| 3 | 30 | 273 | 11 | 46 | 100 | -0.185374077 |
| 4 | 30 | 162 | 11 | 46 | 60 | -0.506770566 |
| 4 | 30 | 189 | 11 | 46 | 70 | -0.425992238 |
| 4 | 30 | 217 | 11 | 46 | 80 | -0.365577629 |
| 4 | 30 | 245 | 11 | 46 | 90 | -0.32017752 |
| 4 | 30 | 273 | 11 | 46 | 100 | -0.284811572 |
| 6 | 30 | 162 | 11 | 46 | 60 | -1.063293018 |
| 6 | 30 | 189 | 11 | 46 | 70 | -0.884288268 |
| 6 | 30 | 217 | 11 | 46 | 80 | -0.752987437 |
| 6 | 30 | 245 | 11 | 46 | 90 | -0.655701573 |
| 6 | 30 | 273 | 11 | 46 | 100 | -0.580711194 |

As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level.

2.6	Whether a cap for LP-WUS power boosting should be considered

It is stated in the agreed WF as follows:

- FFS on the following options: - Option 1: It should be considered as 3dB. - Option 2: It should be considered as 6dB - Option 3: Not to introduce maximum value for LP-WUS power boosting.

As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to cap the LP-WUS power boosting if BS manufacturers declare to support higher levels of power boosting with LP-WUS operation.

2.7	Transmitted signal quality

It is stated in the WF as follows:

- Option 1: Legacy transmitted signal quality requirements apply, no impact when BS transmitting LP-WUS signal. - Option 2: The OOK-based EVM should be defined for LP-WUS. FFS on how to specify this requirement with consideration of RAN1 waveform design.

Currently, the EVM requirement for NR BS is specified in TS 38.104 [3] as follows:

### 6.5.2 Modulation quality #### 6.5.2.1 General Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector. Details about how the EVM is determined are specified in Annex B. For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. #### 6.5.2.2 Minimum Requirement for BS type 1-C and BS type 1-H For BS type 1-C and 1-H, the EVM levels of each NR carrier for different modulation schemes on PDSCH outlined in table 6.5.2.2-1 shall be met using the frame structure described in clause 6.5.2.3. Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier Modulation scheme for PDSCH; Required EVM. QPSK; 17.5 %. 16QAM; 12.5 %. 64QAM; 8 %. 256QAM; 3.5 %. 1024QAM; 2.5 %1 2.8 %2. Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz..

In table 6.5.2.2-1 above, the EVM requirement is specified depending on the modulation schemes and physical channels. The EVM for each modulation scheme was derived by considering the EVM impacts on overall link performance, as explained in TR 38.817-02 [4].

Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new core requirement shall be added for the modulation quality for LP-WUS/LP-SS. The EVM requirement for LP-WUS/LP-SS should be derived by considering the EVM impacts on overall link performance according to RAN1 waveform design.

On the other hand, it was agreed at RAN1#117 as follows [7]:

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

The equation between EVM and SNR is as follows:

SNR=-[PAPR+20logEVM100]							(1)

It has been shown in section 2.1 that the PAPR of LP-WUS/LP-SS is more like 64QAM than 16QAM, and thus the PAPR of 64QAM (equal to 3.7) could be used in equation (1) to obtain the required EVM when the SNR is decided in RAN1.

2.8	Multi-band operation

It is stated in the WF as follows:

- FFS on multi-band operation.

As discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such the same LP-WUS/LP-SS power boosting requirement and declaration can be applicable to multi-band BS.

# 3.	Conclusion

This contribution has provided proposals to progress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: The core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level.

Proposal 2: The definition of LP-WUS dynamic range/power boosting in RAN4#110bis approved WF R4-2406140 should continue to be used to avoid potential confusion among the various requirements related to power boosting.

Proposal 3: A new EVM requirement shall be added for LP-WUS/LP-SS and it should be derived by considering the EVM impacts on overall link performance according to RAN1 waveform design.

If proposal 1 is agreed, then:

Proposal 4: There is no need to preclude small CBW for LP-WUS power boosting.

Proposal 5: There is no need to set restriction on applicable BS types for LP-WUS.

Proposal 6: There is no need to cap the LP-WUS power boosting.

Proposal 7: The same LP-WUS/LP-SS power boosting requirement and declaration can be applicable to multi-band BS.

# References

[1]	RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

[2]	R4-2413512, “Way Forward for [112][309] NR_LPWUS”, Moderator (Huawei).

[3]	3GPP TS 38.104 v18.7.0: “NR; Base Station (BS) radio transmission and reception”.

[4]	3GPP TR 38.817-02 v15.11.0: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”.

[5]	3GPP TS 36.141 v18.4.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[6]	3GPP TS 38.141-1 v18.7.0: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.

[7]	RP-241039, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo.