3GPP TSG-RAN WG4 Meeting #112bis                            R4-2415498

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.17.2.1

Source:	Apple

Title:	Study for CSI Compression: Testability and Interoperability issues

Release:       Rel-19

Document for:	Discussion

# Introduction

Rel-19 Work Item (WI) was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface (WID in [1]). The application of AI/ML techniques to NR air interface has been studied in FS_NR_AIML_Air.

This work item provides normative support for the general framework of AI/ML concerning air interfaces. It also enables the implementation of recommended use cases outlined in the previous study. Furthermore, several study objectives within this project aim to address outstanding issues identified during the study, with the goal of enhancing understanding in preparation for future normative effort.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [2]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for AI/ML for NR air interface for CSI compression and prediction.

# Discussion

# 2.1 Testability and interoperability issues for CSI compression Option 3

In R4-2414447 it was agreed to perform another round of simulations based on some agreements on model alignment of model training and data set parameters for AI/ML-based CSI compression it was agreed to study the feasibility of the test decoder for Option 3.

2.1.1 Agreements about the parameters used in the simulation for checking the feasibility of testing options for CSI compression

The following agreement was reached: Parameters agreed are just for the feasibility study of testing options.

If/when RAN4 discusses requirement definition, RAN4 will define a new test decoder which may or may not reuse any of the parameters agreed in the feasibility study.

2.1.2 Agreements regarding the process for checking the feasibility of deriving a test decoder for Option 3

Test decoder derivation procedure:

Option 1

Companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

Option 2

Companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

RAN4 agrees to work on option 1.

Interested companies are invited to bring further feasibility analysis for Option 2

2.1.3 Simulation parameters for verifying the feasibility of aligning model among companies

The following simulation parameters have been agreed:

System-level simulation parameters (from TR):

| Parameter |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Dense Urban (Macro only) |
| Frequency Range |  | FR1 only, [2GHz, 4GHz] |
| Inter-BS distance |  | 200m |
| Channel model |  | According to TR 38.901 |
| Antenna setup and port layouts at gNB |  | Companies need to report which option(s) are used between - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for (rank 1-4) |
| BS Tx power |  | 44dBm for 20MHz |
| BS antenna height |  | 25m |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | Baseline: 15kHz for 2GHz; Optional: 30kHz for 4GHz |
| Simulation bandwidth |  | Baseline: 10 MHz for 15kHz Optional: 20 MHz for 30kHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| MIMO scheme |  | SU-MIMO |
| MIMO layers |  | Baseline: 1 Optional: 2 |
| CSI feedback |  | Feedback assumption at least for baseline scheme - CSI feedback periodicity (full CSI feedback): 5 ms (baseline) - Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms |
| Traffic load (Resource utilization) |  | Baseline: 50% Optional: 20/70% |
| UE distribution |  | CSI compression: 80% indoor (3 km/h), 20% outdoor (30 km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| Feedback assumption |  | Realistic |
| Channel estimation |  | Realistic or ideal channel estimation |

Based on the two rounds of discussions, the following parameters were agreed for the AI models:

Fig 1. Detailed model diagrams of encoder.

Fig 2. Detailed model diagrams of decoder.

System parameters:

| Frequency range | 2GHz (4GHz optional) |
| --- | --- |
| MIMO Layers | 1 layer (2 layers optional, companies to |
| Sub-bands | 13 |
| Channel estimation | ideal |
| Metrics | SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI. |

2.1.4 Simulation results for the agreed test decoder for Option 3

We implemented the test decoder in Pytorch. The training parameters (and channel type) are shown in the table below:

| Training parameters | Encoder/Decoder |
| --- | --- |
| Optimizer | Adam |
| Batch size | 256 |
| Loss function | 1-SGCS |
| Stopping Criterion | No change in metric over a given number of epochs (25) |
| Max Epochs | 150 |
| Learning Rate | 10^-4 |
| Dataset size | 600K |
| Quantization of Training Data | Not quantized (Float 32) |
| Channel Type | UMa |
| Weight Initialization | Default |
| RX Antenna pattern | Omni |

Table 1. RAN 4 agreed Training parameters

The complexity and size of both Encoder and Decoder were profiled with the Pytorch model in inference mode. The results of the profiling are shown below:

| Complexity (MACs) | Complexity (FLOPS) | Number of Parameters | Memory Size | Model |
| --- | --- | --- | --- | --- |
| 47MMacs | 93MFlops | 125795 | 491.3KBytes | Encoder |
| 740.3MMacs | 1480MFlops | 1806951 | 7058.4KBytes | Decoder |

Table 2. Computational Complexity and Memory Size

We perform quantization aware training by bypassing the quantization in the backward propagation to enable zero gradient (due to round) and enable learning.

We report the validation loss of the AI/ML model in terms of SGCS and we compare it with the SGCS loss of the e-typeII codebook with configuration 1. The results are shown below and captured as an observation:

Observation 1: Simulation results comparing SGCS between AI/ML and e-typeII are captured below:

| SGCS | Encoder/Decoder |
| --- | --- |
| 0.698 (Relative improvement 1.9%) | AI/ML (64 bits) |
| 0.685 | e-type II (configuration 1, 62 bits) |

Table 3. Simulation results for AI/ML and e-typeII codebook with Quantization

In the figures below we present the training evolution of the AI/ML across epochs and plot the training and validation loss for both floating point model and a model with quantization.

Fig 3. Training evolution of the AI/ML models

Observation 2: We observe a tendency for overfitting when we train the AI/ML model. This can be observed by the continuous improvement of the training error and the saturation of the testing error.

Proposal 1: Each company to report training/testing results across training epochs. If overfitting is consistently observed, investigate the sources of overfitting

According to agreements for aligning simulations results, it was encouraged to plot the cdf of the SGCS across different sub-bands. This is shown in the figure below:

Fig 4. SGCS across sub-bands for AI/ML model

Observation 3: Edge sub-bands seem to have the worst performance in terms of SGSS for the AI/ML model

Proposal 2: Each company to report cdf plots of the testing data per SGCS. Based on these results investigate if there is a tendency for the edge sub-bands to perform worse.

The CDF plot of the SGCS for etype-II is shown in the figure below.

Fig 5. SGCS across sub-bands for AI/ML model

2.1.5 Simulation study for evaluating different simulation parameter assumptions

Below we provide our simulation results across different simulation parameter assumptions.

Fig 6. Simulation Study for optimizing simulation parameters

Fig 7. Simulation Study for optimizing simulation parameters

Fig 8. Fast learning rate 0.025 causes instability

Observation 4: Significant performance differences are observed (in SGCS) across different simulation assumptions for Option 3 test decoder

Proposal 3: To better understand the relative gains of AI/ML compared to type-II SGCS in adapting optimized simulation parameters for feasibility analysis, we propose conducting a related study. This will help pave the way for simulation alignment and for the definition of performance requirements. We have identified two parameters for optimization:

- Learning rate (it could be adaptive)

- Phase normalization of the eigenvectors

- Increasing the data size remedies tendency to overfit

2.1.5 Updated diagram feasibility analysis for Option 3

For the two-sided model, two key questions arise:

- How can tests be defined in a way that ensures implementation on different test equipment is feasible and results are repeatable?

- How can we ensure that the requirements and test setup are sufficiently related to real network conditions, so that the tests are meaningful in relation to actual network operation? (importance of having a robust test decoder and a rich latent space for effective real-world operation)

To meet these criteria, according to option 3, a test decoder is standardized. By doing so, a consistent, repeatable test can be implemented across various test equipment, ensuring that test results are both reliable and applicable to real-world scenarios.

Feasibility analysis: Once the test decoder is standardized and established, all vendors can create practical product encoders that leverage the knowledge of the test decoder and its latent space. Before the final adoption of a test decoder, companies can validate its effectiveness by confirming that they can indeed develop encoders based on the decoder

We provide an updated flowchart diagram (from RAN4#112) for the procedures to follow for testing the feasibility of the test decoder for Option 3:

Figure 9. Flowchart of procedures for feasibility analysis of Option 3 Test decoder

For aiding the performance alignment of the simulation results reported by each company, each company should list the training parameters that the encoder, decoder pair was trained with.  As a next step we could employ data sharing as shown in step 8.

Proposal 4: For aiding the performance alignment of the simulation results reported by each company, each company should list the training parameters used for the encoder-decoder pair. An example of training parameters is shown below:

- Optimizer

- Batch size

- Loss function

- Stopping Criterion

- Max Epochs

- Learning rate

- Dataset size

- Quantization of training data

- Channel type

- Weight initialization

Additionally, data sharing (Step 8) could be employed:

- Company A trains its encoder/decoder with company’s B dataset.

- Company B tests the encoder decoder pair with company’s A testing data

Proposal 5: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 3. (It captures both Option 1 and Option 2)

We further elaborate on the steps described in the flowchart. We need to agree on the criteria to determine if feasibility is met. A test decoder that delivers marginal throughput performance compared to e-type II under ideal conditions (unquantized training data, high SNR) should be considered not feasible. A test decoder with better performance should be considered for feasibility analysis (loopback to step 1). Another aspect of feasibility concerns the alignment of all the UE preferred encoder with the chosen test decoder (step 6).

Proposal 6: The following criteria should establish the feasibility of the test decoder:

- The test decoder (along with the encoder) should deliver reasonable throughput performance gain compared to e-type II in ideal conditions. RAN4 to extend the feasibility analysis to higher ranks and different payload sizes to determine consistency of performance improvement. FFS on how to quantify the acceptable performance gain. (performance-complexity tradeoff)

- All UE preferred encoders should be aligned (work well) with the chosen test decoder

Therefore, the feasibility should entail both a performance and alignment aspect.

We will also need to agree on how to determine the alignment aspect.

Proposal 7: The following principles should guide the establishment of the alignment aspect of the feasibility analysis (steps 6-7):

- A well-trained UE encoder with the same or higher complexity compared to the Ref encoder that the test decoder was trained with, should have at least similar performance with the reference performance. (in throughput)

- FFS on how to define a criterion for the similarity/dissimilarity of all companies' performance results with respect to the reference performance, considering UE encoder complexity as well.

At step 5 of the flowchart, we will need to choose the test decoder (among all companies’ test decoders) to proceed with the feasibility analysis. We will need a criterion to select a test decoder

Proposal 8: RAN4 to define a criterion to select a test decoder among all the test decoders reported after performance alignment is achieved (step 5). The criterion could be based upon:

- Each company tests its trained pair of {Encoder, test decoder) with other companies testing data set. Chose the pair (and therefore test decoder) that has the best performance averaged across other’s companies datasets.

According to the agreed test decoder derivation procedure:

Option 1

Companies bring encoder + decoder set based on agreed parameters. RAN4 chooses one of the decoders and interested companies further check if an encoder can be trained with this decoder to obtain similar performance/complexity (or other evaluation criteria)

Option 2

Companies bring training data for encoder + decoder pair, interested companies train an encoder + decoder pair based on the aggregated dataset from all companies to check decoder derivation feasibility and performance/complexity (or other evaluation criteria)

We are merging the two options in one flowchart. First, we evaluate the feasibility analysis of Option 1 (steps 1-7) and if Option 1 is not feasible then we investigate Option 2 (steps 8-13).

Proposal 9: According to the flowchart, RAN4 should first evaluate feasibility analysis with Option 1 and proceed to Option 2 (aggregated dataset) if Option 1 does not produce a feasible test decoder

## 2.2 Testability and interoperability issues for CSI compression Option 4

Option 4 entails utilizing a reproducible test decoder for testing purposes, while the decoder itself is not explicitly detailed in the specification. The specification should include sufficient information to allow different test equipment (TE) vendors, and eventually network (NW) vendors, to design decoders that deliver consistent and reproducible test results.

The ultimate objective of both Option 3 and Option 4 is to establish a common and standardized decoder that facilitates reproducible and consistent testing for test equipment (TE). In Option 3, the decoder is explicitly defined, whereas in Option 4, a reference encoder or a dataset is specified , allowing the TE vendor to derive the decoder. The test decoder should also be interoperable across multi-vendor Encoders.

In order to achieve reproducible tests and guarantee interoperability, the specification will need to capture the following aspects:

- The latent space representation to be captured by the decoder

- An aggregate dataset from UE vendors, featuring UE-preferred encoders in the form of {V, c}, aiming to enrich the latent space and enhance interoperability.

For Option 4 test decoders, we require partially specified decoders to satisfy the principle of test repeatability and bounded variation across TE implementations.

Test repeatability should be guaranteed for Option 4 (bounded variations across various TE implementations). If the DUT passes or fails the test with one TE implementation, it should exhibit the same outcome with another TE implementation.

Based on the WF from RAN4 #111 [R4-2410571] and the ad-hoc meeting notes:

Option 4 for 2-sided model

Several companies brought proposal on how to further study/check the feasibility of option 4.

Issue 4-6: Option 4 for 2-sided model

- Proposals

  - Option 1: Qualcomm (R4-2407334)

- Option 4a-1 standardized dataset

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure.

  - Step 2: RAN4 uses this encoder/decoder pair and the generation procedure to generate a set of decoder input and output data and captures this dataset in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4a-2 standardized aggregated dataset

  - Step 1: RAN4 achieves some agreements (e.g., part of but not all the parameters in the test decoder parameter table in the previous meeting WF[1]) for the test decoder.

  - Step 2: Interested companies can design their own encoder/decoder pairs based on the agreements to contribute the (decoder input, decoder output) dataset to RAN4

  - Step 3: RAN4 aggregates the datasets from all the contributing companies, and capture the aggregated dataset in the specification

  - Step 4: RAN4 specifies a test decoder verification procedure based on the specified dataset.

- Option 4b reference encoder/decoder pair

  - Step 1: RAN4 agrees a pair of encoder and decoder with full details (same as fully specified decoder discussion) and an encoder input data generation procedure

  - Step 2: RAN4 capture the encoder/decoder as a reference encoder/decoder pair and the encoder input data generation procedure in the specification.

  - Step 3: RAN4 specifies a test decoder verification procedure based on the reference encoder.

- Option 2: Xiaomi (R4-2407847): Standardized data / dataset format + Dataset exchange between NW-side and UE-side

- Option 3: vivo(R4-2408294)

  - Option 4a: Reference encoder + test decoder model structure, and channel generation method are in the spec;

  - Option 4b: Test decoder model structure + reference encoder model structure, and dataset (PMI and corresponding channel) are in the spec;

  - Option 4 – Huawei (R4-2409003))

- Option 4a: Model structure is not specified in RAN4. Training dataset is specified, where each training sample consists of both the raw channel matric/precoding matrix and the bit stream forwarded to the test decoder.

- Option 4b: Model structure is specified in RAN4. Training dataset is not specified for verifying the encoder at DUT. The test decoder developed by TE vendor needs verification.

  - FFS: How to determine the test metric for test decoder developed by each TE vendor.

  - Option 5: Nokia (R4-2408659)

- Option 2 (Dataset based):

  - Option 2a: Freeze complete training data while leaving model architecture for implementation.

  - Option 2b: Freeze the important characteristics of training data, e.g., number of bits of latent message while leaving actual data samples and model architecture for implementation.

- Option 3: Freeze the important characteristics of training data, e.g., number of bits of latent message, and a backbone of model architecture while leaving actual data samples and architectural details for implementation.

  - Option 6: Ericsson (R4-2408492)

    - the latent space needs to be standardized

    - standardize a reference encoder in order to capture the latent space

  - Option 7: others

- Recommended WF

  - To be discussed

Likely multiple options need to be chosen/combined RAN4 should agree on a minimum set such that companies can continue the study.

We are making a proposal (Option 5) for carrying the feasibility analysis for test decoder for Option 4 based on the following concepts:

- The architecture of the test decoder is partially specified through some high-level parameters (to be agreed) while leaving room for vendor specific implementations

- Interested companies implement a test decoder according to the partially specified agreement

- The partially specified test decoder is trained from an aggregate data set of UE encoder latent space distributions (dataset is stored in a server). Multiple test decoders are trained from participating companies. The test decoders are stored in a database.

- Each UE vendor tests its own preferred Encoder with the test decoders stored in the database

Regarding the goal of Option 4 test decoder:

Test repeatability should be guaranteed for Option 4 (bounded variations across various TE implementations). If the DUT passes or fails the test with one TE implementation, it should exhibit the same outcome with another TE implementation.

The proposed procedures could determine the feasibility of the Option 4 test decoder goal since DUT => preferred UE encoder and {another TE implementation} => Test Decoder stored in database.

The flowchart of procedures is shown below:

Fig 10. Flowchart of procedures for feasibility analysis of Option 4 Test decoder.

Proposal 10: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option4 based on the aggregate dataset and database of test decoder models. (Option 5 for partially specified test decoder)

- The architecture of the test decoder is partially specified through some high-level parameters (to be agreed) while leaving room for vendor specific implementations

- Interested companies implement a test decoder according to the partially specified agreement

- The partially specified test decoder is trained from an aggregate data set of UE encoder latent space distributions (dataset is stored in a server). Multiple test decoders are trained from participating companies. The test decoders are stored in a database.

- Each UE vendor tests its own preferred Encoder with the test decoders stored in the database

- Analysis of results to determine feasibility of test decoder

## 2.3 Generalization/scalability aspects

The performance of generalization is of importance for a successful AI/ML feature deployment and the necessity and feasibility of defining requirements or test to verify the generalization of AI/ML has been studied. The content below from the TR discusses achieving minimum performance under certain conditions while ensuring that performance is not significantly degraded under other conditions Accordingly for the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signaling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

Proposal 11:

Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

# Conclusion

In conclusion the following observations and proposals were discussed in this contribution:

Observation 1: Simulation results comparing SGCS between AI/ML and e-typeII are captured below:

| SGCS | Encoder/Decoder |
| --- | --- |
| 0.698 (Relative improvement 1.9%) | AI/ML (64 bits) |
| 0.685 | e-type II (configuration 1, 62 bits) |

Observation 2: We observe a tendency for overfitting when we train the AI/ML model. This can be observed by the continuous improvement of the training error and the saturation of the testing error.

Observation 3: Edge sub-bands seem to have the worst performance in terms of SGSS for the AI/ML model

Observation 4: Significant performance differences are observed (in SGCS) across different simulation assumptions for Option 3 test decoder

Proposal 1: Each company to report training/testing results across training epochs. If overfitting is consistently observed, investigate the sources of overfitting

Proposal 2: Each company to report cdf plots of the testing data per SGCS. Based on these results investigate if there is a tendency for the edge sub-bands to perform worse.

Proposal 3: To better understand the relative gains of AI/ML compared to type-II SGCS in adapting optimized simulation parameters for feasibility analysis, we propose conducting a related study. This will help pave the way for simulation alignment and for the definition of performance requirements. We have identified two parameters for optimization:

- Learning rate (it could be adaptive)

- Phase normalization of the eigenvectors

- Increasing the data size remedies tendency to overfit

Proposal 5: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option 3. (It captures both Option 1 and Option 2)

Proposal 6: The following criteria should establish the feasibility of the test decoder:

- The test decoder (along with the encoder) should deliver reasonable throughput performance gain compared to e-type II in ideal conditions. RAN4 to extend the feasibility analysis to higher ranks and different payload sizes to determine consistency of performance improvement. FFS on how to quantify the acceptable performance gain. (performance-complexity tradeoff)

- All UE preferred encoders should be aligned (work well) with the chosen test decoder

Proposal 7: The following principles should guide the establishment of the alignment aspect of the feasibility analysis (steps 6-7):

- A well-trained UE encoder with the same or higher complexity compared to the Ref encoder that the test decoder was trained with, should have at least similar performance with the reference performance. (in throughput)

FFS on how to define a criterion for the similarity/dissimilarity of all companies' performance results with respect to the reference performance, considering UE encoder complexity as well

Proposal 8: RAN4 to define a criterion to select a test decoder among all the test decoders reported after performance alignment is achieved (step 5). The criterion could be based upon:

- Each company tests its trained pair of {Encoder, test decoder) with other companies testing data set. Chose the pair (and therefore test decoder) that has the best performance averaged across other’s companies datasets.

Proposal 9: According to the flowchart, RAN4 should first evaluate feasibility analysis with Option 1 and proceed to Option 2 (aggregated dataset) if Option 1 does not produce a feasible test decoder

Proposal 10: RAN4 to employ the procedures shown in the flowchart for studying the feasibility analysis of test decoder Option4 based on the aggregate dataset and database of test decoder models. (Option 5 for partially specified test decoder)

- The architecture of the test decoder is partially specified through some high-level parameters (to be agreed) while leaving room for vendor specific implementations

- Interested companies implement a test decoder according to the partially specified agreement

- The partially specified test decoder is trained from an aggregate data set of UE encoder latent space distributions (dataset is stored in a server). Multiple test decoders are trained from participating companies. The test decoders are stored in a database.

- Each UE vendor tests its own preferred Encoder with the test decoders stored in the database

- Analysis of results to determine feasibility of test decoder

Proposal 11:

Consider the following aspects regarding the different conditions for testing generalization for CSI AI/ML use:

- Various antenna port layouts, e.g., (N1/N2/P) and/or antenna port numbers (e.g., 32 ports, 16 ports)

- Various antenna spacings (e.g., 0.5 lambda, 0.8 lambda, etc)

- Various antenna virtualization (TxRU mapping)

- Various carrier frequencies and bands (e.g., 2GHz, 4.0GHz)

- Various outdoor/indoor UE distributions for UMa/Umi

- Various UE speeds.

Consider the following aspects regarding the scalability aspect for generalization testing for CSI AI/ML use:

- Various bandwidths (e.g., 20MHz, 50MHz) and/or frequency granularities, (e.g., size of subband), different layers

- Various sizes of CSI feedback payloads

# Reference

[1] RP-242399 - Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

[2] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”