# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415506

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.14.2

Source:	Apple

Title:	On Spatial Channel Modeling for Demodulation Requirements

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In RAN#104 a new SI for study of spatial channel modeling was approved [1].  In RAN4#112 the discussion on this topic was kicked off and way forward [2] was approved. In this contribution we share our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4.

# 2. Discussion

## General Aspects

In RAN4#112 the following agreement were made:

For SU-MIMO so far only 8-layer PDSCH TP test with 8 layers has been agreed but the precoder is FFS. In RAN4 requirements for PDSCH the standard practice is to use random PMI, and we should continue to adopt the same for the study. Fixed PMI is used in some cases with CQI reporting but might not be suitable for PDSCH throughput. We might need to find a suitable precoder for the channel realization in case fixed PMI is used and it might not be a trivial task to do so especially with large number of layers and antenna.

- RAN4 employs random PMI for PDSCH demod requirements.

- Finding suitable fixed PMI for channel realization would be time consuming and not trivial especially for larger number of TX/layers.

- RAN4 employ random precoder for PDSCH TPut test cases for alignment and comparison study of different channel modeling methodologies.

The test case for PMI reporting with 8 layers needs further discussion and justification. We don’t introduce requirements for PMI reporting for large number of layers, hence more justification is needed as to the purpose of this.

- Further justification is needed for considering PMI reporting with 8 layers for the study.

It would be insufficient to only evaluate for 8RX, including 4RX would also be needed as 8RX is only for a certain form factor UE currently and 4 RX would be more applicable to a wider range of devices.

- Include 4RX in study to cover more devices and applicable requirements introduced in RAN4.

For MU-MIMO RAN4 has defined requirements in Rel-17, Rel-18 with TDL channel model. The purpose of introducing requirements with MU-MIMO is to determine if the UE can reject or suppress the inter user interference. Employing TDL channels doesn’t limit testing of UE processing to cancel inter-user interference in case of MU-MIMO. Further justification is needed for studying impact of MU-MIMO performance with spatial channel model.

- UE processing is tested sufficiently for cancelling inter-user interference in MU-MIMO scenarios with TDL channel model.

- Further justification is needed for considering MU-MIMO for the study.

In case MU-MIMO test cases are agreed for the study, the test setup should be what RAN4 has used so far – with random PMI for target UE, orthogonal/random PMI for co-scheduled UEs. Scenarios with multiple UEs, PMI feedback should be out of scope. There is no use is the test setup in the study cannot be used for the actual test setup when requirements are defined.

- In case MU-MIMO is agreed the test setup is limited with what RAN4 has used so far – single UE with random PMI for target UE.

- In case MU-MIMO is agreed the test setup shall not include PMI feedback and/or multiple UEs.

If found necessary and feasible to introduce a spatial channel model for requirements in RAN4, we propose that RAN4 doesn’t define demodulation or CSI reporting requirements with it in Rel-19 for ongoing Rel-19 WIs or for features/ requirements introduced in previous WIs. This is a study and not a WI.

- If determined as necessary and feasible to introduce spatial channel model, RAN4 shall not define any new requirements with it in Rel-19 – for either Rel-19 WIs or earlier WIs.

## TDL based methodologies

On TDL methodologies there was a discussion on the channel models to consider for the evaluation and study:

The channel model and especially the MIMO correlation chosen should align with the test purpose. For example, if PDSCH performance for 8 layers is considered, we don’t see it being practical and feasible with ULA Medium B.

- The MIMO correlation for TDL channel models should be chosen based on test purpose.

In 38.901 spatial filtering for TDL channel is described. This is an option that could be used to include directional antennas in TDL channel model which otherwise assumes omni directional antennas. With CDL channel model, some antenna pattern typically directional is used. With the same assumptions and simplification to generate TDL channel model, we can use a directional antenna pattern and obtain spatially filtered TDL channel models.

RAN4 to also consider spatially filtered TDL channel models in the study.

- RAN4 to also consider spatially filtered TDL channel models described in 38.901 in the study.

## CDL based methodologies

In RAN4#112 it was agreed to prioritize the UMa-CDL-C channel model in TS 38.827. The following BS antenna parameters are defined in the TS and the highlighted one is what we would use for this study, based on agreement in [2].

The purpose of the channel model in 38.827 is for MIMO OTA, hence a large antenna array with 64 dual-polarized antennas is used and analog beam forming is employed.

- The purpose of channel model in 38.827 is for MIMO OTA and employs a large antenna array with analog beam forming.

Introducing analog beam forming in link level simulation analysis introduces unnecessary complications which we have not dealt with in any performance evaluation or requirements in demod. And how this would impact with any comparison of TDL based channel modeling methodologies is yet to be investigated.

- The necessity of analog beam forming for evaluation of CDL methodology is not justified.

- Study of channel model for demod requirements need not consider analog beam forming

For the purpose of evaluation of different methodologies, we shouldn’t enable analog beam forming for CDL based methodologies.

- Do not enable analog beam forming for CDL based methodology evaluation.

In our understanding the channel model simplification and definition in the TR is not dependent on the BS antenna array captured in table 7.2-7 but the antenna model for the study in MIMO OTA. For using any CDL channel model the antenna parameters at the BS and UE need to be specified and we can specify our own for this study and alignment purposes.

- The BS and UE antenna parameters need to be specified with CDL channel model, and we can choose our own in RAN4 for this study.

Our proposed values are:

Table 1: BS Antenna parameters for SCM study

| Parameter description | Symbol | Parameter value |  |
| --- | --- | --- | --- |
|  |  | For 4TX | For 8TX |
| Antenna panels in vertical dimension | Mg | 1 | 1 |
| Antenna panels in horizontal dimension | Ng | 1 | 1 |
| Elements per panel in vertical dimension | Me | 2 | 2 |
| Elements per panel in horizontal dimension | Ne | 1 | 2 |
| Number of polarizations per panel | P | 2 | 2 |
| Element spacing in horizontal dimension () | dH | 0.5 | 0.5 |
| Element spacing in vertical dimension () | dV | 0.5 | 0.5 |

For TX, the antenna element has ±45 polarization components and the radiation pattern parameters are 3dB = 65, 3dB = 65, Amax = 30dB, SLAv = 30dB, GE,max =8 dBi.

In the channel coefficient generation, there will not be beam forming weight factor.  The channel coefficient for time instant t, Rx antenna u, Tx antenna s, and cluster n is defined by the following equation for NLOS.

Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,θθn,m,ZOD,φn,m,AODFtx,s,φθn,m,ZOD,φn,m,AOD

expjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,sλ0expj2πrrx,n,mT.vλ0t

- Use CDL channel in 38.827 with the following BS antenna parameters and channel coefficient generation.
BS antenna equal to the number of TX ports: 4TX (2,1,2); 8RX (2,2,2)
Coefficient generation without analog beam forming.
Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,θθn,m,ZOD,φn,m,AODFtx,s,φθn,m,ZOD,φn,m,AOD

expjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,sλ0expj2πrrx,n,mT.vλ0t

The current models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions on RMS delay spread, UE velocity, etc. RAN4 defines band agnostic demodulation and CSI reporting requirements.

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

It would be very important to study if the spatial channel model can be defined for all applicable carrier frequencies/ bands for each FR.

- RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

In Rel-15 we defined simplified TDL channel models with maximum of 12 taps by taking the channel model in 38.901 and combining close by paths and having a resolution of 5ns. The CDL channel models in 38.901 and 38.827 are defined for a large number of clusters – 23/24 for NLOS cases. RAN4 needs to further study if they models can be simplified by taking a similar approach as described in 38.101-4 Annex B.2.1.

- RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

## Comparison of methodologies

In RAN4#112 we agreed on the comparison approach for methodologies comparison as captured below:

But we have not discussed the metric we will use to compare the methodologies to rationalize the need for a special channel model for demodulation requirements.

One of the objectives of the SI is to identify the limitations of current channel models and how MIMO performance is impacted:

We need to further discuss the metric used to compare methodologies that help identify limitations in the current methodology that impacts UE/BS demod/ MIMO performance. Without a concrete idea on how to this is done, it would be difficult to move ahead with the agreed comparison.

- RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts MIMO performance.

# 3. Conclusion

In this paper, we provide our views on scope of study for spatial channel models for defining demodulation and CSI reporting requirements in RAN4. Our observations and proposals are captured below:

General aspects

- RAN4 employs random PMI for PDSCH demod requirements.

- Finding suitable fixed PMI for channel realization would be time consuming and not trivial especially for larger number of TX/layers.

- RAN4 employ random precoder for PDSCH TPut test cases for alignment and comparison study of different channel modeling methodologies.

- Further justification is needed for considering PMI reporting with 8 layers for the study.

- Include 4RX in study to cover more devices and applicable requirements introduced in RAN4.

- UE processing is tested sufficiently for cancelling inter-user interference in MU-MIMO scenarios with TDL channel model.

- Further justification is needed for considering MU-MIMO for the study.

- In case MU-MIMO is agreed the test setup is limited with what RAN4 has used so far – single UE with random PMI for target UE.

- In case MU-MIMO is agreed the test setup shall not include PMI feedback and/or multiple UEs.

- If determined as necessary and feasible to introduce spatial channel model, RAN4 shall not define any new requirements with it in Rel-19 – for either Rel-19 WIs or earlier WIs.

TDL based methodologies

- The MIMO correlation for TDL channel models should be chosen based on test purpose.

- RAN4 to also consider spatially filtered TDL channel models described in 38.901 in the study.

CDL based methodologies

- The purpose of channel model in 38.827 is for MIMO OTA and employs a large antenna array with analog beam forming.

- The necessity of analog beam forming for evaluation of CDL methodology is not justified.

- Study of channel model for demod requirements need not consider analog beam forming

- Do not enable analog beam forming for CDL based methodology evaluation.

- The BS and UE antenna parameters need to be specified with CDL channel model, and we can choose our own in RAN4 for this study.

- Use CDL channel in 38.827 with the following BS antenna parameters and channel coefficient generation.
BS antenna equal to the number of TX ports: 4TX (2,1,2); 8RX (2,2,2)
Coefficient generation without analog beam forming.
Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕFtx,s,θθn,m,ZOD,φn,m,AODFtx,s,φθn,m,ZOD,φn,m,AOD

expjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,sλ0expj2πrrx,n,mT.vλ0t

- The CDL channel models in the TR 38.827 are defined for specific scenarios, carrier frequency, with some assumptions of RMS delay spread and UE velocity.

- RAN4 to study if the same channel model can be defined for all carrier frequencies/ bands in given frequency range – example FR1/ FR2.

- RAN4 to study if the CDL channel models can be simplified to smaller number of clusters.

Comparison of methodologies

- RAN4 to discuss test metrics to compare the methodologies that help identify limitations in the current methodology that impacts MIMO performance.

# Reference

- RP‑241848, “Revised SID: Study on spatial channel model for demodulation performance requirements for NR”, Nokia.

- R4-2413606, “Way Forward for [112][327] FS_NR_demod_SCM”, Nokia.