# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415513

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.2.1

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#112, which includes open issues for further discussion. In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

# Discussion

Assumption on searchers

Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier [Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT)

We can see the rationale behind each option.

- Option A/1a: Since it is assumed that there is only one target carrier in FR2-1 to be measured and UE is configured with single carrier on FR2-1 band, it is OK to assume both searchers are used for L3 measurement for either serving cell or target cell. Note that for the intra-freq. case, where target and serving carrier frequency is the same, it is possible for the UE to measure both serving cell and target cell at the same time and then process the measurement samples from both cells sequentially.

- Option B: To consider forward compatibility where the specified requirement can apply to CA case, one searcher can be assumed.

Proposal 1: Either option is OK. To consider forward compatibility where the specified requirement can apply to CA case, Option B, i.e., one searcher assumption, is slightly preferred.

SSB processing delay/time

Issue 1-2-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC [Way forward]: FFS: - Option 1 (CATT, vivo): No need to add additional processing time due to multiple SSBs within one SMTC. - Option 2 (Apple): allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing - Option 3 (Ericsson): Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement - Option 4 (Intel): - Consider UE baseband processing capabilities when specifying the L3 delay reduction for simultaneous receptions on multiple FR2 SSB-s. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Option 5 (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms

This issue was extensively discussed at the last meeting. In the meeting, we pointed out that when both panels are activated to make measurements, the measurement samples from each panel can be processed sequentially if only one searcher is assumed. Since the SMTC periodicity is 5ms or longer, it is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity. In that regard, Option 5 can be used as a starting point for further discussion.

Proposal 2: It is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity. In that regard, Option 5 can be used as a starting point for further discussion.

Power consumption issue (including conditions to trigger UE using FBS for L3 measurement)

Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement) - Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. - Option 2 (HW): - RAN4 shall firstly identify the promising scenario(s) for L3 measurement delay reduction enabled by multi-Rx with clear/significant benefits, which could help to converge the discussion. - One possible scenario to be considered is when there is strong demand of mobility performance (e.g. UE at cell edge or the link is about to break). - It shall not be assumed that UE supporting this feature shall activate multiple panels all the time for all L3 measurement. RAN4 shall discuss the entering and/or exiting conditions for enhanced L3 measurement enabled by multi-Rx taking the targeting scenario into account. - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. - Option 4 (vivo): Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx - Option 5 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions. - Option 6a (Nokia): Mobility Event triggering BSF reduction - Option 6b (Nokia): FBS is triggered by conditional Handover configuration - Option 6c (Nokia): BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC). - Option 7 (QC): RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements: - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. - FFS on the application delay of UE Rx beam sweeping factor switch. - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. - RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 9 (ZTE): Due to L3 measurement is long-term operation, power consumption issue could be considered, which may lead to some interaction signalling. But which would not be the applicability condition of applying fast beam sweeping. [Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed.

It is reasonable to consider UE location and/or mobility state as a trigger for activation/deactivation of FBS. Besides these two factors, UE QoS should be considered too. For example, if a serving cell cannot provide good QoS such as data rates or latency to a UE for a certain service, the network may need to hand over the UE to another cell even though there is no urgent need to trigger handover due to mobility or signal quality degradation. Meanwhile, UE power consumption should also be considered.

To consider all these factors, there are two alternatives:

Alt. 1: Network configures conditional activation/deactivation (in a way similar to conditional handover). The conditions are UE location/mobility state/QoS requirement. When the conditions are met, the UE activates FBS; and when the conditions are not met, the UE de-activates FBS. On top of this, the UE is allowed to use UAI to inform the network if it wants to get out of FBS due to power consumption.

Alt. 2: Network configures conditional activation/deactivation. The conditions are UE location/mobility state/QoS requirement. In addition, the network can assign an activation timer on FBS operation based on UE power state (or the level of need for power saving). UE power state can be indicated to the network by the UE. When the conditions are met, the UE activates FBS and starts the timer; and when the conditions are not met or the timer expires, the UE de-activates FBS.

We prefer the conditional activation/deactivation of FBS to the mechanism where NW indicates UE enabling/disabling FBS through L3 or lower layers signaling.

Proposal 3: Conditional trigger mechanism is preferred, in which UE power consumption, UE location, UE mobility and UE QoS requirements can be considered.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Issue 1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Proposal 1 (CATT): - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Proposal 2 (Apple, CTC): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Proposal 3 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Proposal 4 (LGE): - If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4. - RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception - Proposal 5 (Ericsson): - As a result of FBS, RX beam sweep factor can be defined as [4]. - Proposal 6 (Ericsson): - RAN4 to study the requirements relevant to switching between different operations, including: - Transition time period when enabling and disabling FBS in Rel-19. - Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18. - Proposal 7 (Samsung): - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline - Whether to/How to define the other values can be based on SLS results (if necessary)

Since RAN4 agreed at the last meeting that RAN4 would not consider simultaneous operation between L3 measurement and data reception on the same OFDM symbols, it is straightforward to consider the solution of reduced beam sweeping factor.

Proposal 4: For UE supporting R19 multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline.

Feature capability of L3 measurement delay reduction by optimizing Rx BSF

Issue 1-5: feature capability of L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Option 1(Apple, vivo, Nokia, Samsung): - RAN4 to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19. - Option 1a(Apple): The detailed definition can be postponed to the end of the core part discussion. - Option 2(Eircsson): - To support FBS, RAN4 to check if those capabilities for multi-Rx in Rel-18, e.g., faster RX beam sweeping, enhanced scheduling and measurement restrictions and multi-Rx preference indication, can be used directly. If the NW indicates FBS and multi-RX in Rel-18 is not enabled parallelly, we can observe the possibility of reusing at least part of such capabilities.

We share the same view with companies that a new capability is needed for this L3 Rx BSF reduction feature. The main reason is that different from R18 FR2 multi-Rx WI and R18 FR2 SCell activation enhancement, this R19 feature shall be used for both L3 serving and neighbor cell measurement. We are also fine to delay this capability discussion to the end of the core part as legacy procedure.

Proposal 5: RAN4 is to introduce a new individual capability for L3 BSF reduction due to multi-Rx operation in R19. The detailed definition can be postponed to the end of the core part discussion.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: Either option is OK. To consider forward compatibility where the specified requirement can apply to CA case, Option B, i.e., one searcher assumption, is slightly preferred.

Proposal 2: It is possible for the UE to process both samples (one from each panel) within one SMTC periodicity. Therefore, it is OK to only consider some additional processing time after the last SMTC periodicity. In that regard, Option 5 can be used as a starting point for further discussion.

Proposal 3: Conditional trigger mechanism is preferred, in which UE power consumption, UE location, UE mobility and UE QoS requirements can be considered.

Proposal 4: For UE supporting R19 multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline.

Proposal 5: RAN4 is to introduce a new individual capability for L3 BSF reduction due to multi-Rx operation in R19. The detailed definition can be postponed to the end of the core part discussion.

# References

[1] R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, RAN4#112