3GPP TSG-RAN WG4 Meeting #112bis                            			      				 R4-2415519

Hefei, Anhui, China, 14th – 18th Oct 2024

Source: 	Apple

Title: 	Discussion on RRM requirements for type 4 UE

Agenda Item:	6.5.3

Document for:	Discussion

# Introduction

RRM requirements for support of intra-band non-collocated EN-DC/NR-CA deployment have been discussed in the last meeting and a WF was agreed [1]. The following scopes remains open for further study.

Issue 1-3: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

Issue 1-4: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE

Issue 1-6: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario

Issue 1-8: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

Issue 1-10: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

Issue 1-11: SCell BFD/CBD requirements for Type 4 capable UE

This contribution will present our consideration on these open issues for type 4 UE.

# Discussion

While the discussion on type 4 UE is still not fully concluded in RF session, it is anticipated that the type 4 UE behaves quite similar as type 2 UE except for the maximum number of MIMO layers supported on each CC in both collocated condition and non-collocated condition. And it also likes type 2 UE that, a type 4 UE is assumed to be able to switch its RF between type 1 capability (Rx chain sharing) and type 4 capability (separate RF chain sets for each CC). It is anticipated the RRM requirements to be satisfied will differ for the 2 kinds of RF configurations. The discussion of this paper assumes similar UE capability reporting and network indication will be introduced for type 4 UE and the default UE behaviour is type 4 capability.

## Scenarios

Issue 1-3: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19

- Proposals

Option 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

The following was agreed in the WF from RAN4#112 meeting [1].

We think it’s reasonable to focus on synchronized case for intra-band non-collocated NR CA since this is also the same situation for type 2 UE.

Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

## Other RRM requirements impacted

The following was captured in the WF from RAN4#112 meeting [1].

Issue 1-4: Other RRM requirements (apart from MRTD/MTTD requirements) for Type 4 UE

- Proposals

  - Option 1(Apple, Nokia, Huawei, ZTE, Ericsson, Samsung, KDDI): The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE

- Interruption

- SCell activation delay

- Scheduling availability

- SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection)

We think there are no doubt that these requirements will be impacted given the similarity between type 2 UE and type 4 UE. And the requirement impact for non-collocated condition has been concluded in the last RAN4 meeting. What’s remaining open is the requirement impact when collocated condition is met.

Proposal 2: The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE (what’s open is when collocated condition is met)

- Interruption

- SCell activation delay

- Scheduling availability

- SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection)

## Interruptions (in collocated condition)

Issue 1-6: Interruption requirements for Type 4 capable UE when UE operates with shared RF chains in collocated scenario

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): when Type 4 capable UE operates with shared RF chains in collocated scenario, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied

- Interruption at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions during measurements on deactivated SCC

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

The following was agreed in the WF from RAN4#112 meeting [1].

For type 4 UE, since separate set of RF chains is used to handle the Rx signal for each CC, it behaves like an inter-band CA UE when receiving signals from 2CC in the same band. The legacy FR1 inter-band CA interruption requirements apply for interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC.

Some clarifications are needed with regard to the UE capability reporting signalling and network indication. Can be settled when the signalling name is confirmed.

Proposal 3: The legacy FR1 inter-band CA interruption requirements apply for type 4 UE in the aspects of interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC. Need to wait for the RF session conclusion on signalling.

## SCell activation delay requirements (in collocated condition)

Issue 1-8: SCell activation delay requirements for Type 4 capable UE when UE operates with shared RF chains in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): When Type 4 capable UE operates with shared RF chain in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

The following was agreed in the WF from RAN4#112 meeting [1].

The existing SCell activation delay requirements in clause 8.3.2 apply for type 4 UE if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA.

Some clarifications are needed with regard to the UE capability reporting signalling and network indication. Can be settled when the signalling name is confirmed.

Proposal 4: The existing SCell activation delay requirements in clause 8.3.2 apply for type 4 UE if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA. Need to wait for the RF session conclusion on signalling.

## Scheduling availability (in collocated condition)

Issue 1-10: Scheduling availability for Type 4 capable UE when UE operates with shared RF chain in collocated condition

- Proposals

  - Option 1(Apple, Huawei, Samsung, KDDI): When type 4 capable UE operates with shared RF chain in collocated condition, scheduling availability defined for type 2 UE in collocated condition in R18 can be applied

Note: Some clarifications are needed with regard to the UE capability reporting signalling and network indication

  - Option 2 (Nokia): wait for RF progress on UE type transition and design of new UE capabilities/NW signalling.

The following was agreed in the WF from RAN4#112 meeting [1].

Scheduling availability requirement should be introduced for type 4 UE performing beam failure detection with a different subcarrier spacing as PDSCH/PDCCH on FR1, L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1, measurement in TDD band, or measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1.

Proposal 5: Scheduling availability requirement should be introduced for type 4 UE performing beam failure detection with a different subcarrier spacing as PDSCH/PDCCH on FR1, L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1, measurement in TDD band, or measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1. Need to wait for the RF session conclusion on signalling.

## SCell BFD requirements

Similar as for type 2, it is suggested that that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers.

Proposal 6: It is suggested that that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band.

# Summary

This contribution presented further views on RRM requirements impact for type 4 UE. The following observation and proposal are concluded.

Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

Proposal 2: The following RRM requirements (apart from MRTD/MTTD requirements) are also impacted due to Type 4 UE (what’s open is when collocated condition is met)

- Interruption

- SCell activation delay

- Scheduling availability

- SSB based and CSI-RS based BFD (Beam failure detection) and CBD (candidate beam detection)

Proposal 3: The legacy FR1 inter-band CA interruption requirements apply for type 4 UE in the aspects of interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC. Need to wait for the RF session conclusion on signalling.

Proposal 4: The existing SCell activation delay requirements in clause 8.3.2 apply for type 4 UE if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA. Need to wait for the RF session conclusion on signalling.

Proposal 5: Scheduling availability requirement should be introduced for type 4 UE performing beam failure detection with a different subcarrier spacing as PDSCH/PDCCH on FR1, L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1, measurement in TDD band, or measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1. Need to wait for the RF session conclusion on signalling.

Proposal 6: It is suggested that that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band.

# References

[1] R4-2413973, WF on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Apple