3GPP RAN WG4 Meeting #112bis																R4-2415524

Hefei, China, 14th – 18th October 2024

Agenda item:	6.23.2.1

Source:	Apple

Title:	On Low-power Wake-up Receiver for NR

WI/SI:	NR_LPWUS-Core

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

LP-WUS/WUR shall be introduced to NR in Rel-19. The objectives are covered in the WID [1]. This contribution focuses on the RAN4 core requirements and the way forward [2] from last meeting. The WID lists RAN4 goals as:

# 2  Discussion

##### 2.1 On MDR and FAR metric

The WF [2] captures the state on MDR and FAR as shown below:

Issue 1-1-2: Performance metric for Rx RF requirements Agreement: - 1% MDR for evaluation [and core requirements] - FFS on the testability issue. Issue 1-1-3: Performance metric for Demodulation requirements Agreement: - [1% or 0.1%] FAR as assumption for MDR result calibration.

Last meeting an agreement was reached to set MDR to 1% for evaluation. Testability is FFS and the applicability to the core requirements need to be finalised. For FAR it was proposed to consider 1% and 0.1% for further discussion.

False alarm rate describes the quota of incorrectly recognizing an incoming signal as its wake-up signal. Any time the wake-up receiver incorrectly detects an incoming signal it would wake up the rest of the device (e.g. wake up main receiver and baseband) to process the expected message following the wake-up call. The device needlessly monitors the channel for a message from the network until time-out is reached and returns to sleep. Consequently, a high false alarm rate would drain device battery and is therefore not preferred. Device manufacturers have interest in keeping false alarm rate at a minimum level. However, we believe that due to the main goal of reducing the wake-up receiver power consumption there is a trade-off between power consumption, false alarm rate and missed detection rate. From network point of view there seems to be no downside from a UE monitoring the channel after waking up from a false positive. The performance of the network is not negatively impacted. Therefore, an overly tight false alarm rate does not seem justified. We propose to set the value for FAR not lower than 1%.

Proposal 1: From network point of view there seems to be no downside from a UE monitoring the channel after waking up from a false positive. The performance of the network is not negatively impacted. Therefore, an overly tight false alarm rate does not seem justified. We propose to set the value for FAR not lower than 1%.

##### 2.1 On noise figure for WUR types

Noise figure and SNR are critical parameters influencing the RF hardware design and network performance. RAN4 has decided that the noise figure for OFDM and OOK can be different. This is captured in the WF [2]:

Issue 2-2-4: whether different NF for OOK-based and OFDM-based Agreement: - Differentiate NF for OOK-based and OFDM-based receiver.

To achieve low noise figure the LNA is typically one of the first components in the receiver chain. Designing a LNA with low power consumption and low noise is immensely challenging. To achieve high linearity and high gain the LNA requires several stages translating into high power consumption. To effectively reduce the power consumption the gain and noise performance needs to be relaxed. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase for LP-WUR.

An OFDM type receiver is expected to re-use existing RF architecture from the main receiver. The battery lifetime of the device is enhanced by tweaking the LNA setup for reduced current consumption. This decreases the noise figure performance. Therefore, a delta to the typical 9dB of the main receiver is required if increased battery live for OFDM type receiver is targeted by this work item. We conducted measurements using an OFDM type WUR receiver and found that a delta of 7dB would be required to satisfy the expected improvements with respect to current consumption. Regarding implementation margin we propose to set the value no lower than 2.5dB.

Observation: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption gain and noise performance needs to be reduced. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase even for OFDM type receiver. Measurements indicate that approximately a 7dB delta to main receiver is required to satisfy the expected improvements with respect to current consumption. Regarding implementation margin a value of 2.5dB would be suitable.

##### 2.1 LP-WUR Channel Raster

There has been in-depth discussion on the channel raster applicability for WUR type receiver over the last meetings. The current state is captured below:

Issue 1-2-1: Channel raster for LP-WUR Agreements: - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS. - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

The answer to the channel raster seems to depend on several considerations:

- If the LP-WUR has no knowledge on the actual WUS frequency and needs to search for the signal itself then a dedicated sync and channel raster might be needed to optimise the search grid. However, this might mean that the actual MDR could be higher in the field as the search process for WUS might results into missed wake-up calls.

- The main receiver could configure the WUR for a certain carrier frequency based on the RB grid of the assigned main receiver channel. In this case no dedicated channel raster would be required for the WUR. However, this approach might have some issues if the raster of the main receiver uses 100kHz spacing use of the extended channel raster with 10kHz grid might be necessary for configuring the WUR.

- The network could indicate the WUS frequency location, e.g. via the SIB. If the WUR is required to the decode the SIB by itself then it can only be manageable for an OFDM type WUR as it can access the data via the FFT modulator. The OOK type WUR does not deploy any FFT and therefore cannot access this information. It would need to wake up the main receiver to check on the content to obtain the frequency location. Solutions should be considered which keep the WUR as simple as possible.

RAN4 should continue the discussion and consider checking on RAN1 and RAN2 to get a better understanding on the signalling and configuration aspect for the WUR.

# 4	Conclusions

This contribution provides a discussion based on last meetings way forward on LP-WUR. The following observations and proposals are made:

Proposal 1: From network point of view there seems to be no downside from a UE monitoring the channel after waking up from a false positive. The performance of the network is not negatively impacted. Therefore, a tight false alarm rate does not seem justified. We propose to set the value for FAR not lower than 1%.

Observation: To achieve high linearity and high gain the LNA requires more stages translating in more power consumption. To effectively reduce the power consumption gain and noise performance needs to be reduced. While typical NR FR1 receiver feature a noise figure of approximately 9 dB this value is expected to increase even for OFDM type receiver. Measurements indicate that approximately a 7dB delta to main receiver is required to satisfy the expected improvements with respect to current consumption. Regarding implementation margin a value of 2.5dB would be suitable.

# 4	References

- RP-240801, “Low-power Wake-up Signal and Receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, 3GPP TSG RAN Meeting #103

- R4-2414309, “WF on LP-WUS UE RF requirements”, vivo, 3GPP TSG-RAN WG4 Meeting #112