# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415534

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for UE initiated Beam reporting in R19 MIMO

Agenda Item:	6.19.3.1

Document for:	Discussion

Introduction

In RAN4 #112, the WF for R19 MIMO [1] is approved. In [1], the following table is agreed.

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

Based on all above information, we provide our views on RRM requirements for UE initiated beam reporting.

Discussion

The WID scope [2] for this is written as follows.

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

According to WID, the WI design is targeting FR2, but not precluding FR1. RAN1 has also agreed to support 2 Modes for the event-triggered beam reporting, i.e. Mode-A and Mode-B. The related agreements are cited as follows.

RAN1 #117 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This mode is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all modes For above procedures, cross-CC beam reporting is supported for both modes. FFS: Details. RAN1 #118 Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for regarding Mode-B, the pre-configured resource(s) for the second channel in Step-2 is at least type 1 CG-PUSCH. - FFS: PUCCH as the second channel - FFS: Whether the PUSCH can be with UL data On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the dedicated RRC signaling for first PUCCH channel configuration for Mode-A, down-select one of the following - Alt-1 (dedicated SR for mode-A): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR, corresponding to the one-bit indication in the first PUCCH channel - The RRC parameter is associated with the dedicated SchedulingRequestId. - Note: The detailed signaling is up to RAN2. - Alt 2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig-ModeA-UEIBR, for the periodic PUCCH resource configuration. - Note: The RRC parameter is NOT associated with SchedulingRequestId. - FFS: how to encode 1-bit to PUCCH resource, e.g., reuse encoding mechanism of positive/negative SR. - The dedicated RRC parameter at least comprises the following: - periodicityAndOffset - PUCCH-ResourceID - Above applies at least for the single CC case. Conclusion On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding Mode-A, there is no RAN1 consensus on additionally supporting that the DCI format in Step-2 comprises DL-grant DCI format, and the second channel in Step-3 is PUCCH.

The main difference between these 2 Modes for UE initiated beam reporting (UEIBR) is how UE reports the generated L1-RSRP/L1-SINR. As agreed in RAN1, Mode-A is the basic UE capability. In our view RRM requirements and test cases shall focus on Mode-A first. In RAN1 # 118, RAN1 has agreed that for both Mode-A and mode B, the uplink channel carrying UCI is at least PUSCH. However, even for Mode-A, RAN1 has not yet come to the conclusions whether the one-bit/multi-bit PUCCH is SR or not. The key point is that, if it is SR, then in case gNB has already provided uplink grant which is enough to carry UCI, there is probably no need for the first PUCCH. In this case, RAN4 may follow existing logic specified in 9.2.4 of TS 38.133 for defining delay requirements of event-triggered L1 reporting, i.e. RRM requirements are defined assuming UL resource for the reporting is always available. However, if multi-bit PUCCH is adopted in RAN1, it quite unlikely to be SR, and some additional information is provided, and the latency for sending step-1 PUCCH must be counted in the delay requirements for the event-triggered L1-RSRP/L1-SINR reporting.

Observation 1  For Mode-A, if the one-bit/multi-bit PUCCH is SR, then following legacy UE behaviour, UE needs not to send such SR, if gNB has already provides the uplink grant that is big enough to carry the UCI when UE generates the UCI. In this case requirements in 9.2.4 of TS 38.133 would be a starting point for event-triggered L1 reporting delay.

Regarding mode-B, as cited above, only CG-PUSCH has been agreed. However, CG-PUSCH is an optional UE feature since R15. Therefore, we are not sure whether RRM requirements are necessary for this case. In the field, as far as we know, most handheld UEs do not yet support CG-PUSCH, and there is no RRM test cases using CG-PUSCH. However, RAN1 may also come to further conclusions for mode-B, e.g. it can be also carried by DG-PUSCH, i.e. the PUSCH with additional UL data, which means an uplink grant indicating a variable TB size is available. In that case, RAN4 can further discuss RRM requirements on mode-B.

Proposal 1  RAN4 specifies RRM requirements at least for Mode-A UEIBR.

Proposal 2  RAN4 does not define RRM requirements for Mode-B if it can only be carried by CG-PUSCH. If RAN1 comes to further conclusion that it can be carried by PUCCH or DG-PUSCH, RAN4 may also define RRM requirements for Mode-B.

Proposal 3  To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not.

In last meeting, the following FFS is also listed in [1].

Issue 2-1-8: Whether to filter L1-RSRP? <Way forward> - FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured. Issue 2-1-9: Impact of beam on known/unknown conditions for TCI states? <Way forward> - FFS. Depends on RAN1 further progress.

In our view, these two issues are dependent to each other. In legacy L3 measurement and reporting, the measurement can deal with much lower RS SINR, and L1 and L3 filtering are performed, as illustrated in TS 38.300. However, for L1-RSRP reporting, so far TimeRestrictionForChannelMeasurement can be configured as True and in that case neither L1 nor L3 filtering is done. For this event triggered L1 reporting, at least based on following RAN1 conclusion, we think event-triggered L1 reporting shall follow legacy L1 measurement behaviour.

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

In our view, for beam management purposes, since the reported L1-RSRP/L1-SINR is one type of CSI which may also impact UL/DL scheduling, it would be OK if no filtering is applied before the reporting. Therefore, RAN4 shall focus on the basic UE feature.

Proposal 4  RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

In our view, network shall be responsible for the filtering of L1 measurement results reported by UE, which is the same as legacy. However, for event-triggered L1 reporting, the UE reporting behaviour could be different from legacy. For example, in legacy L3 measurement, UE shall also report when event-leaving condition is met. However, it is still not clear in RAN1 whether event-leaving is necessary for L1 based approach. On the other hand, after event is met, it is still not clear whether UE shall send one report or multiple reports.

In legacy known condition of TCI state switching or handover, it is assumed that if UE reports within a predefined period before switching happens, then the corresponding cell or TCI is considered as known. In some RRM requirements, for example, inter-cell L1 measurement in R17, RRM requirements are only applicable if the target cell is known. However, for event-triggered L1 reporting, it is still not clear either network or UE is responsible to make a robust decision based on the measured/reported results, and we are not sure whether it is necessary to specify RRM requirements on the cases where the report itself is not robust enough. On this part, RAN4 may wait for further RAN1/2 conclusions.

Proposal 5  Regarding the impact to requirement applicability conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions about the detailed UE reporting behaviour, e.g. whether single-shot or multi-shot reporting after event-entering condition is met, and whether UE reports when event-leaving condition is met, etc.

Conclusions

Based on above analysis, we have following observations and proposals.

Observation 1  For Mode-A, if the one-bit/multi-bit PUCCH is SR, then following legacy UE behaviour, UE needs not to send such SR, if gNB has already provides the uplink grant that is big enough to carry the UCI when UE generates the UCI. In this case requirements in 9.2.4 of TS 38.133 would be a starting point for event-triggered L1 reporting delay.

Proposal 1  RAN4 specifies RRM requirements at least for Mode-A UEIBR.

Proposal 2  RAN4 does not define RRM requirements for Mode-B if it can only be carried by CG-PUSCH. If RAN1 comes to further conclusion that it can be carried by PUCCH or DG-PUSCH, RAN4 may also define RRM requirements for Mode-B.

Proposal 3  To determine how to count for the uncertainty in acquiring UL resource for UEIBR, RAN4 waits further conclusion from RAN1 on whether the first PUCCH is SR or not.

Proposal 4  RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

Proposal 5  Regarding the impact to requirement applicability conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions about the detailed UE reporting behaviour, e.g. whether single-shot or multi-shot reporting after event-entering condition is met, and whether UE reports when event-leaving condition is met, etc.

- References

[1]  R4-2414035  WF on RRM requirements for NR_MIMO_Ph5, Samsung.  RAN4 #112

[2]  RP-242394   WID revision: NR MIMO Phase 5, Samsung.  RAN #105