# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415535

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on other RRM requirement impacts in R19 MIMO

Agenda Item:	6.19.3.2

Document for:	Discussion

Introduction

In RAN4 #112, the WF for R19 MIMO [1] is approved. In [1], the following table is agreed.

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

We have no strong view so far on CJT calibration reporting. In this contribution, we analyse the RRM requirement impacts from 3Tx and UL-only TRP.

Discussion

<On RRM requirement impacts from asymmetric DL sTRP/UL mTRP>

The WID scope [2] for this is written as follows.

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

According to WID, the two TA enhancement focus on the R17/R18 unified TCI framework. Regarding R18 2-TA, the following is concluded in last meeting [3].

Issue 1-2-1: For mDCI mTRP, whether to need additional DL RS tracking time for UL TCI state switching if UE supporting two TAs (RTD<CP and RTD>CP)? Agreement: - In Rel-18, no consensus on the requirement for separate UL state TCI switching if there is no active DL TCI-State associated with the same coresetPoolIndex. For the other cases, reuse Rel-17 requirements.

The above agreement is based on extensive discussion in many RAN4 meetings. For the case of UL TCI stat switching without DL TCI associated with the same coresetPoolIndex, which downlink timing reference UE shall follow was not clarified in R18 RRM requirements. However, in R19, similar scenario is prioritized again, and in our view RAN4 shall clarify DL timing reference and UL TCI state switch requirements if there is neither active DL TCI state nor coresetPoolIndex associated with the target UL TCI.

Proposal 1  RAN4 discuss and clarify the DL timing reference of the TA for a UL-only TRP in R19 MIMO

Proposal 2  RAN4 specify TCI state switching requirements for the case that target UL TCI is not associated with any coresetPoolIndex.

Regarding the DL timing reference, in our view, since there is no DL signal transmitted from the UL-only TRP, UE may not obtain an accurate DL timing of the TRP. To make the UE timing adjustment based on a more stable DL timing, so that gNB-based TA adjustment would work, we slightly prefer to use the cell-level timing reference instead of TRP level. The details of the cell level timing can be further discussed.

Proposal 3  Prefer to use cell level timing instead of TRP-level timing for the TAG associated to a UL TCI without DL TCI, i.e. the DL reference timing of this TAG is not derived from a single active DL TCI or any DL RS of a single UL TCI. The details of cell-level timing are FFS.

<On RRM requirement impacts from 3Tx>

The WID scope [2] for this is written as follows.

Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching.

Like 8 Tx discussion in R18, current requirements assuming SRS in the last few symbols already work for 3T6R. As RAN4 already agreed to preclude RRM requirements for the full slot SRS in R18, we think no further RRM requirements for 3T6R is needed. Note that this does not preclude to consider specifying full slot SRS transmission feature in R18 TEI or future release.

Proposal 4  Not to specify new SRS switching RRM requirements for 3Tx in the scope of Rel-19 MIMO evolution WI.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  RAN4 discuss and clarify the DL timing reference of the TA for a UL-only TRP in R19 MIMO

Proposal 2  RAN4 specify TCI state switching requirements for the case that target UL TCI is not associated with any coresetPoolIndex.

Proposal 3  Prefer to use cell level timing instead of TRP-level timing for the TAG associated to a UL TCI without DL TCI, i.e. the DL reference timing of this TAG is not derived from a single active DL TCI or any DL RS of a single UL TCI. The details of cell-level timing are FFS.

Proposal 4  Not to specify new SRS switching RRM requirements for 3Tx in the scope of Rel-19 MIMO evolution WI.

- References

[1]  R4-2414035  WF on RRM requirements for NR_MIMO_Ph5, Samsung.  RAN4 #112

[2]  RP-242394   WID revision: NR MIMO Phase 5, Samsung.  RAN #105

[3]  R4-2414036  WF on RRM requirements for NR_MIMO_evo_DL_UL, Samsung.  RAN4 #112