# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415537

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for on-demand SSB SCell operation in R19 NES

Agenda Item:	6.22.2.1

Document for:	Discussion

Introduction

In [1], the following objective is listed.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In RAN4 #112, the WF and Workplan for R19 NES [2] is approved. Moreover, one LS from RAN1 [3] is received for this WI.

Based on all above information, we provide our views on RRM requirements for on-demand SSB SCell operation in R19 NES.

Discussion

<On general aspects for OD-SSB >

In recent RAN1 meetings, the follow agreements are made.

RAN1 #118 (2024.08) Agreement - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB Agreement Support L3 measurement based on on-demand SSB - Further work on L3 measurement is up to RAN2/RAN4

RAN2 # 127(2024.08) Scenarios on OD-SSB: 1. RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B. OD-SSB transmission indication: 2. RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration. 3. New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A. Measurement on OD-SSB: 4. Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1). 5. For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated. 6. RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements. 7. RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

As RAN1 has prioritized the case that OD-SSB is not on the sync raster and is NCD-SSB, RAN4 may also prioritize the work on NCD-SSB. Moreover, we also see technical issues if OD-SSBs are on the sync raster, or are CD-SSB, hence prefer not to consider them in the 1st release of OD-SSB. The technical issues shall be discussed in RAN1.

Proposal 1  RAN4 to define RRM requirements firstly for the case that OD-SSBs are NCD-SSBs and are not on the sync raster.

RAN1 has also agreed to support L3 measurement based on OD-SSB, while details are discussed in RAN2/4. On the other hand, the measurement configuration of L3 measurements shall be discuss in RAN2 and RAN2 has not yet achieved conclusions. In our view, OD-SSB based L3 measurement shall be based on SSB periodicity, so as to achieve NW energy saving gain. In other word, our understanding is that gNB would not be able to always align SSB transmission in time domain between multiple intra-frequency OD-SSB-based cells. Therefore, to save RAN4 effort, RRM requirements are not specified for OD-SSB based L3 measurements on neighbour cells.

Proposal 2  RAN4 not to define RRM requirements for OD-SSB based L3 neighbour cell measurement, as feasibility and reliability are questionable and gain is not clear.

Even though OD-SSB based L3 measurement shall be performed based on SSB periodicity, UE would still need to perform OD-SSB based rough sync on this SCell in most cases. Therefore, OD-SSB based SCell identification requirements are still needed.

Proposal 3  Specify RRM requirements for OD-SSB based cell identification, i.e. SSB-based rough sync on the SCell, at least for the case when there is no inter-carrier synchronization indication provided by gNB.

For cell identification, normally searcher will be used. For an OD-SSB layer, where SSB based cell search is not always done once RRC configuration is provided, the searcher assumption would be much more complicated than legacy L3 measurements that semi-statically configured by MO. In our view RAN4 needs clarify the searcher assumption for the OD-SSB based rough sync. For example, whether UE count the OD-SSB frequency layer as one candidate layer for L3 cell search in calculating CSSF, either when OD-SSB is activated (by MAC CE) or when OD-SSB is configured (by RRC), would need more discussion.

Proposal 4  RAN4 to discuss the searcher assumption of OS-SSB based cell identification, which would impact the CSSF of intra-frequency L3 or L1 measurements.

Note that for the cell identification, RAN4 shall not only consider L3 measurement before SCell activation, but also after SCell activation, in which the legacy measurement period is based on SSB periodicity.

Moreover, after L3 measurement, the SCell could be activated based on UE’s L3 MR reporting. For case #1 defined in RAN1, where there is no ‘always-on’ SSB, UE shall also be able to perform rough sync on SSB for the SCell if needed, e.g. when DL SINR is low and UE somehow lost sync momentarily. In such case, no matter whether L3 measurement is configured or not, network shall ensure the availability of OD-SSB, which is the QCL-train source of the SCell in most cases. In other word, after SCell activation is completed, all RRM requirements for the OD-SSB based SCell, including TCI activation, L1/L3 RSRP reporting, and potential BFD/CBD, are only applicable if there is available OD-SSB transmission. Note that to ensure the performance of UE rough sync, the periodicity of OD-SSB shall be no larger than 160ms

Proposal 5  If OD-SSB is assumed to be the QCL-train source of the activated SCell, all RRM requirements, including TCI activation, L1/L3 measurement reporting, and potential BFD/CBD etc., are only applicable when there is available OD-SSB transmission as indicated by gNB, and the periodicity of the OD-SSB is not more than 160ms.

< OD-SSB based Deactivated SCell L3 measurement >

In legacy SSB-based L3 measurements on de-activated SCell, the UE’s L3 measurement is performed based on meascycleSCell configured by gNB. This provides significant power saving gain for the UE, when SCell is de-activated. However, in R19 OD-SSB based L3 measurements, for case #1, gNB may also want to save power, hence OD-SSB is only transmitted, or at least indicated as activated, for example, in 200ms (with periodicity as 20ms and includes 10 SSB busts) per 5 seconds. However, assuming more than one UEs are potentially in the coverage of SCell, and as discussed in proposal 5 that gNB shall ensure available OD-SSBs for activated SCells, our concern would be that gNB would not always de-activate OD-SSB transmission for a UE in de-activated SCell state, and UE would need to perform OD-SSB based SCell measurements based on SSB periodicity for quite a long time. In such case, UE power consumption will be huge.

Proposal 6  For OD-SSB based L3 measurements on deactivated SCell, RRM requirements are specified assuming UE perform L3 measurement based on the periodicity of OD-SSB, not periodicity of meascycleSCell, only within a predefined/configured period of time after OD-SSB based L3 measurements are indicated to be performed. After such period, UE could perform L3 measurements based on meascycleSCell if OD-SSBs are not de-activated.

< OD-SSB based SCell activation>

For SCell activation, in last meeting RAN4 has discussed whether to consider known or unknown scenarios for different RAN1 agreed scenarios. In our view, for SCell activation, there is no strong motivation to change known/unknown conditions for the OD-SSB cases, no matter whether scenario 2 or 2a are discussed. Even for scenario 2a, there could be known case. Therefore, we have the following proposal.

Proposal 7  RAN4 re-use legacy known/unknown SCell condition for SCell activation, no matter whether scenario 2 or 2a are considered.

Moreover, in legacy SCell activation, T_firstSSB_max is used in many scenarios. The definition of T_first_SSB in TS 38.133 is copied as follows.

TS 38.133 v18.6.0 Clause 8.3.2 TFirstSSB_MAX: Is the time to the end of the first complete SSB burst indicated by the SMTC, or within 5ms if SMTC is not configured, after slot n + THARQ+3msNR slot length, further fulfilling: - In FR1, in case of intra-band contiguous SCell activation or in case of intra-band non-contiguous SCell activation for UE not capable of intraBandNR-CA-non-collocated-r18, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot; in case of intra-band non-contiguous SCell activation for UE capable of intraBandNR-CA-non-collocated-r18 or in case of inter-band SCell activation, the first occasion when the SCell being activated is transmitting SSB burst. - In FR2, the occasion when all active serving cells and SCells being activated or released are transmitting SSB bursts in the same slot. TFirstSSB_MAX, enhanced: For a UE supporting shortMeasInterval-r18 in FR1 or FR2-1, if the SMTC for SCell being activated is only configured in the measObjectNR, TFirstSSB_MAX, enhanced is the time to the end of the first complete SSB burst indicated by the SSB periodicity of the SCell being activated, after slot n + THARQ+3msNR slot length. Otherwise, TFirstSSB_MAX, enhanced = TFirstSSB_MAX

As cited above, T_firstSSB_max consider the necessity of intra-band AGC adjustment over all active serving cell for the co-located CA case, which is normally based on SMTC. On the other hand, in R18 SCell activation delay reduction in RRM_enh3, TFirstSSB_MAX, enhanced is introduced while removing such consideration of AGC adjustment. In our view, for R19 OD-SSB based SCell activation, RAN4 may also consider similar UE behaviour as the UE supporting R18 UE feature on SCell activation delay reduction.

Proposal 8  For OD-SSB based SCell activation, for the T_firstSSB, RAN4 not to consider relaxation for AGC adjustment for all active serving cell and to-be-activated SCell in the same band, i.e. only the similar UE behaviour as supporting shortMeasInterval-r18 is considered

Moreover, in our view, for intra-band co-located CA case, how much NW energy saving gain can be achieved for the OD-SSB case would be unclear, and such scenario significantly complicates the UE RRM requirements. As discussed in the SI phase, most NW energy comes from gNB TxRUs to support large antenna array, and with intra-band collocated active serving cell, such TxRUs shall be already activated. Therefore, RAN4 can consider deprioritize such scenario.

Proposal 9  RAN4 may consider to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1, since the necessity of such feature and the network energy saving gain are not clear, but big spec impact is foreseen.

< On RAN1 LS for OD-SSB>

In RAN1’s coming LS, the following information is provided while a question is asked.

R1-2407565 RAN1 has made the following agreements for MAC-CE based indication of on-demand SSB (OD-SSB) transmission on SCell. Only the relevant agreements related to RAN1’s question are excerpted. In highlights in the agreements, RAN1 agreed working assumption that the value of T_min for MAC-CE based OD-SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command. RAN1 would like to ask RAN4 if the working assumption of T_min=. can be confirmed to agreement.

In our view, after decoding activation command for OD-SSB, there is no need for addition processing time unless if UE needs some time to adjust the scheduler of the searcher, as discussed in proposal 4. In our view, both types of UE implementation are considerable, and we slightly prefer that UE shall update the CSSF when OD-SSB configuration for SCell is provided during SCell addition. In such case, the working assumption could be confirmed.

Proposal 10  Confirm the working assumption from RAN1, if RAN4 agrees that the CSSF for SCell L3 measurement is updated when the OD-SSB configuration for SCell is provided during SCell addition.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  RAN4 to define RRM requirements firstly for the case that OD-SSBs are NCD-SSBs and are not on the sync raster.

Proposal 2  RAN4 not to define RRM requirements for OD-SSB based L3 neighbour cell measurement, as feasibility and reliability are questionable and gain is not clear.

Proposal 3  Specify RRM requirements for OD-SSB based cell identification, i.e. SSB-based rough sync on the SCell, at least for the case when there is no inter-carrier synchronization indication provided by gNB.

Proposal 4  RAN4 to discuss the searcher assumption of OS-SSB based cell identification, which would impact the CSSF of intra-frequency L3 or L1 measurements.

Proposal 5  If OD-SSB is assumed to be the QCL-train source of the activated SCell, all RRM requirements, including TCI activation, L1/L3 measurement reporting, and potential BFD/CBD etc., are only applicable when there is available OD-SSB transmission as indicated by gNB, and the periodicity of the OD-SSB is not more than 160ms.

Proposal 6  For OD-SSB based L3 measurements on deactivated SCell, RRM requirements are specified assuming UE perform L3 measurement based on the periodicity of OD-SSB, not periodicity of meascycleSCell, only within a predefined/configured period of time after OD-SSB based L3 measurements are indicated to be performed. After such period, UE could perform L3 measurements based on meascycleSCell if OD-SSBs are not de-activated.

Proposal 7  RAN4 re-use legacy known/unknown SCell condition for SCell activation, no matter whether scenario 2 or 2a are considered.

Proposal 8  For OD-SSB based SCell activation, for the T_firstSSB, RAN4 not to consider relaxation for AGC adjustment for all active serving cell and to-be-activated SCell in the same band, i.e. only the similar UE behaviour as supporting shortMeasInterval-r18 is considered

Proposal 9  RAN4 may consider to deprioritize the intra-band co-located CA for OD-SSB based SCell operation, at least in FR1, since the necessity of such feature and the network energy saving gain are not clear, but big spec impact is foreseen.

Proposal 10  Confirm the working assumption from RAN1, if RAN4 agrees that the CSSF for SCell L3 measurement is updated when the OD-SSB configuration for SCell is provided during SCell addition.

- References

[1]  RP-242354   Revised WID: Enhancements of network energy savings for NR, Ericsson.  RAN #105

[2]  R4-2413885  WF on Netw_Energy_NR_enh, Ericsson. RAN4 #112

[3]  R4-2414911  LS on timeline for On-demand SSB operation on SCell, Apple. RAN4 #112bis