# 3GPP TSG-RAN WG4 Meeting #112bis		R4-2415539

# Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on RRM requirements for R19 LTM

Agenda Item:	6.24.2.1

Document for:	Discussion

Introduction

In RAN4 #112, the WF and work plan for R19 Mobility enhancement phase 4 [1], [2] are approved.

Based on all above information, we provide our views on RRM requirement impacts for R19 mobility.

Discussion

In WID [3], the measurement related enhancement is stated as follows.

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

In our view, at least the two highlighted bullets will have RRM requirement impacts. Hence, we mainly focus on the RAN1/2 progress on above two bullets in this discussion.

<On event-triggered L1 reporting >

In RAN1 and RAN2, regarding event-triggered L1 reporting, the following related agreements have been achieved.

RAN2 Agreements (purpose of event-triggered L1 measurements) - Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure. (cell level or beam level for event evaluation) - For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. (LTM event definition) - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. - Event LTM1 is not defined. (Configurations for event) - Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5. - For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met. - R19 LTM event triggered measurement configuration can be taken as baseline: - LTM measurement resource configuration is provided in LTM-config. - Event triggered report config is provided in serving cell config. (Event Reporting) - Event-triggered L1-measurements are reported by the UE to the network via MAC CE. - MAC layer handles the event evaluation and measurement report triggering.

RAN1 Agreements - For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell - The RSs of the candidate cell(s) for event evaluation are explicitly configure - At least the following options are further studied in RAN1, where different options could apply to different LTM event - Option. 1: Derived from QCL (type-D) RS(s) of the indicated joint/DL TCI state for the serving cell - Option. 2: Derived from QCL RS(s) or SSB QCLed with the QCL RS of the indicated joint/DL TCI state for the serving cell - QCL RS or SSB is configured by the network - Option. 3: Measurement RS(s) is/are explicitly configured - Option. 4: Derived from QCL RSs of activated TCI states with the best quality, or SSB which is QCLed with the QCL RSs of activated TCI states with the best quality. - Option 6: Derived from QCL RSs of activated TCI states, or SSB which is QCLed with the QCL RSs of activated TCI states - Note: Companies are encouraged to take into account the RAN2 agreement (i.e current beam rather than best beam) for their further study.

The motivation of event-triggered LTM is for early sync or for LTM cell switch. Both RAN1 and RAN2 have agreed to use beam level results for event evaluation. Note that event-triggered L3 measurement reporting delay has already been specified in RAN4. From the perspective of L1 event evaluation, the main difference compared to L3 would be the beam level evaluation, especially for FR2-1. Note that as discussed in R18 LTM, L1 measurement reporting is not necessary for LTM related procedures in FR1, hence the priority of FR1 could be slightly lower.

Proposal 1  To investigate RRM requirement impacts for Event-triggered L1 reporting, RAN4 start from FR2-1. At a later stage, RAN4 to investigate whether FR2-1 conclusions achieved in the WI can also be applied to FR1.

In R18, multi-Rx based FR2 UE reception is discussed. In R19, RAN4 shall firstly agree that whether multi-Rx based UE reception is also considered in R19 L1 measurement-based mobility in FR2. In our view, if UE supports multi-Rx for 4-Layer DL MIMO, it shall also be able to support multi-Rx based beam-level measurement of neighbour cell if some conditions are met. However, considering the interest of multi-RX based implementation from companies, we are also open to discuss whether to specify RRM requirements for this case. Note that for SSB-based LTM L1 measurement with gNB scheduled reporting, multi-Rx based L1 measurement requirements are also missing in R18.

Proposal 2  For FR2-1, RRM requirements for multi-Rx based LTM L1 measurement shall be considered. RAN4 may make early decision on whether to specify RRM requirements for this case.

RAN2 have also agreed that any beam in candidate RS configuration can be used for LTM event evaluation. In RAN4, for R18 single-Rx based FR2 L1 measurement reporting, the down-selection indication derived from early-TCI activation for one candidate cell is already supported. Such mechanism shall also be re-used in R19 event-triggered L1 measurement reporting in FR2-1.

Proposal 3  For event triggered L1 reporting in FR2-1, regarding the neighbour cell which is prioritized to be measured, RAN4 re-use R18 solutions, i.e. if UE supports early TCI activation, UE prioritize the neighbour cell with pre-TCI activation in L1 measurement reporting.

Another issue would be whether to consider inter-frequency L1 measurement with gap for event triggered reporting. Our view is that this case is quite similar to L3 measurement reporting, even in FR2-1, and the gain compared to L3 measurement reporting for the gap-based case is limited. Therefore, RAN4 may deprioritize the gap-based event triggered reporting also in FR2-1. Note that in R18, RAN4 had a difficult discussion regarding the gap-based operation in LTM.

Proposal 4  RAN4 deprioritize gap-based inter-frequency event-triggered L1 reporting in FR2-1 and FR1.

Regarding the spec structure, our view is that the R18 spec still needs further polishing. LTM related L1 measurements shall be captured in LTM related clauses without impacting legacy L1 measurement for serving cell, i.e. without impacting clause 9.5 of TS 38.133. Our view is that, in case UE supports multiple FFT based L1 measurement for mobility, such UE behaviour shall be significantly different from legacy UE behaviour for serving cell, hence the results would be possibly not comparable. For LTM related reporting, serving cell L1 measurement shall also be captured in 9.14.

Therefore, although it would be slightly early to discuss the spec structure, our view is that, event triggered L1 measurement reporting for both serving cell and neighbour cells are captured in 9.14 and 9.15 (for inter-frequency without gap case).

Proposal 5  Event Triggered Reporting for LTM L1 measurement is to be introduced in section 9.14 and section 9.15.

<On CSI-RS based L1 reporting >

In RAN1, regarding CSI-RS based L1 reporting, the following related agreements have been achieved.

RAN1 Agreements - Support L1-RSRP measurement based on CSI-RS - CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting - Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported - Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting - For gNB scheduled reporting and event triggered reporting - At least periodic CSI-RS is supported for L1-RSRP measurement for candidate cell - At least CSI-RS for beam management is supported for L1-RSRP measurement for candidate cell

As the purpose of CSI-RS based L1 measurement and reporting is for LTM procedures and related beam management, in our view the definition of intra-f/inter-f of CSI-RS based L1 measurements is not an important issue. Note that RAN1 has already agreed to support R18 LTM CSI-RS for BM for R19 CSI-RS based L1 measurements, and in such configuration, there is no center frequency, but there is indication of QCL-ed SSB. Therefore, in our view, to simplify RAN4 design, R18 intra-frequency or inter-frequency definition shall be re-used.

Proposal 6  Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency.

- Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules

- No additional definition of intra-F/inter-F for CSI-RS based L1 measurements.

For SSB based L1 measurements, in R18 RAN4 had a long discussion on inter-frequency with gap. For CSI-RS based L1 measurements, in our view gap-based measurement is more difficult since it is only supposed to measure one cell at a time with the cost of interrupting current serving cell for several miliseconds every 40 or 80 ms. We do not think benefit of gap-based CSI-RS based measurement would match such huge cost. Hence, we think only CSI-RS based L1 measurement without gap, i.e. within active BWP of the UE, shall be considered in R18.

Proposal 7  For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases:

- CSI-RS based L1 measurement within active BWP of the UE

As RAN1 has agreed for periodic CSI-RS, RAN4 may also prioritize periodic CSI-RS for candidate cell

Proposal 8  RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS.

Conclusions

Based on above analysis, we have following observations and proposals.

Proposal 1  To investigate RRM requirement impacts for Event-triggered L1 reporting, RAN4 start from FR2-1. At a later stage, RAN4 to investigate whether FR2-1 conclusions achieved in the WI can also be applied to FR1.

Proposal 2  For FR2-1, RRM requirements for multi-Rx based LTM L1 measurement shall be considered. RAN4 may make early decision on whether to specify RRM requirements for this case.

Proposal 3  For event triggered L1 reporting in FR2-1, regarding the neighbour cell which is prioritized to be measured, RAN4 re-use R18 solutions, i.e. if UE supports early TCI activation, UE prioritize the neighbour cell with pre-TCI activation in L1 measurement reporting.

Proposal 4  RAN4 deprioritize gap-based inter-frequency event-triggered L1 reporting in FR2-1 and FR1.

Proposal 5  Event Triggered Reporting for LTM L1 measurement is to be introduced in section 9.14 and section 9.15.

Proposal 6  Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency.

- Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules

- No additional definition of intra-F/inter-F for CSI-RS based L1 measurements.

Proposal 7  For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases:

- CSI-RS based L1 measurement within active BWP of the UE

Proposal 8  RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS.

- References

[1]  R4-2414060  WF on NR mobility enhancements Phase 4, Apple.  RAN4 #112

[2]  R4-2411437  RAN4 Work Plan for R19 NR mobility enhancement phase 4, Apple.  RAN4 #112

[3]  RP-242356   Revised Work Item: NR mobility enhancements Phase 4, Apple, CTC.  RAN #105