# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415542

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.26.4.1

Source:	Apple

Title:	On R19 NTN (e)RedCap RRM requirements

Document for:	Discussion

# Introduction

The WF on RRM requirements for Rel-19 NR NTN phase3 [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112][226] NR_NTN_Ph3 [2].

In this contribution, we continue the discussion on the RRM requirement for R19 NTN (e)RedCap.

# RRM for support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

Regarding the R17 RedCap UE, the key features impacted RRM requirements are:

- 1Rx

- RRM relaxation

- Idle/Inactive mode eDRX (Inactive mode eDRX<=10.24s)

- NCD-SSB

- HD-FDD

Regarding the R18 eRedCap UE, the key features impacted RRM requirements are:

- Inactive mode eDRX>=10.24s

Issue 6-1-5: The network scenario considered for (e)Redcap UE with FR1-NTN

<Way Forward> - For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

Since both the (e)RedCap and NTN WIs in previous release didn’t consider CA or DC operation, here we think the scenario in this WI shall also only focus on NR SA operation with single carrier.

Proposal 1: For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

Issue 6-1-6: The SCS considered for (e)Redcap UE with FR1-NTN

<Way Forward> - For RRM requirements, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

In NTN we supports all FR1 SCS of 15/30/60kHz on band n256, n255 and n254 in TS38.101-5,

Table 5.3.5-1: Channel bandwidths for each NTN satellite band in FR1-NTN NTN satellite band; SCS kHz; UE Channel bandwidth (MHz). 5; 10; 15; 20; 30 (NOTE). 15; 5; 10; 15; 20. n256; 30; 10; 15; 20. 60; 10; 15; 20. 15; 5; 10; 15; 20. n255; 30; 10; 15; 20. 60; 10; 15; 20. 15; 5; 10; 15. n254; 30; 10; 15. 60; 10; 15. NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements..

However, there are some SCS limitation for R18 eRedCap when BB bandwidth reduction is used, as shown below,

RP-223544 WID for R18 eRedCap Complexity/cost reduction - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2] - UE peak data rate reduction - Relaxation of the constraint (vLayers·Qm·f ≥ 4) for peak data rate reduction - The relaxed constraint is, e.g., 1 (instead of 4). - The parameters (vLayers, Qm, f) can be as in Rel-17 RedCap. - Both 15 kHz SCS and 30 kHz SCS are supported. - Aim to define at most one Rel-18 RedCap UE type for further UE complexity reduction. - The existing UE capability framework is used, and changes to capability signalling are specified only if necessary. By default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified.

And in TS38.101-1, it was specified that,

5.3I Channel bandwidth for RedCap The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 with maximum channel bandwidth of 20MHz. The transmission bandwidth configuration in Table 5.3.2-1 shall be supported for each of the specified channel bandwidths up to 20 MHz. When UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 the requirements in this specification apply with maximum transmission bandwidth of 25RBs for 15 kHz SCS and 12 RBs for 30 kHz SCS for PDSCH and PUSCH. The channel bandwidths are specified for both the TX and RX paths.

That means, for NTN with eRedCap, if UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18, only need to consider 15kHz SCS and 30kHz SCS in FR1 NTN RRM requirement; and for all the other cases of RedCap and eRedCap, 15/30/60kHz shall be considered for FR1-NTN RRM requirement.

Proposal 2: for NTN (e)RedCap UE, if UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18, only need to consider 15kHz SCS and 30kHz SCS in FR1 NTN RRM requirement; otherwise for all the other cases, consider 15/30/60kHz for FR1-NTN (e)RedCap RRM requirement.

Issue 6-1-2: The bandwidth of (e)Redcap UE with FR1-NTN

<Way Forward> - Both bandwidth with and without reduction of R18 eRedCap UE should be supported for (e)RedCap UE with FR1-NTN bands. - Note: There is no RRM impact on the two different types of eRedCap UEs (bandwidth with and without reduction).

As stated in the R18 eRedCap WID below, it highlighted that “The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth”. The bandwidth reduction in R18 eRedCap has no extra impact on R18 RedCap RRM requirement, and it shall not either differentiate the NTN eRedCap requirement, since the RS for RRM will not be punctured on frequency or time domain.

RP-223544 WID for R18 eRedCap - Further reduced UE complexity in FR1 [RAN1, RAN2, RAN4] - UE BB bandwidth reduction - 5 MHz BB bandwidth only for PDSCH (for both unicast and broadcast) and PUSCH, with 20 MHz RF bandwidth for UL and DL - The other physical channels and signals are still allowed to use a BWP up to the 20 MHz maximum UE RF+BB bandwidth. - Support additional separate early indication(s) [RAN1, RAN2]

In general, we agree with the way forward from last meeting only except the 60kHz SCS case for UE supporting IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18 in proposal 2.

Proposal 3: Same RRM requirement shall be applied to the eRedCap NTN UE with and without BB bandwidth reduction, only except that RRM requirements with 60kHz SCS can only be applied for the UE without BB bandwidth reduction.

Issue 6-2-2: What RRM requirements are defined for (e)RedCap UE with FR1-NTN

<Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

The MDT feature is an optional feature from R16, and since NTN UE usually is equipped with GNSS for NTN connection, it shall not be a big challenge for such NTN (e)RedCap UE to support MDT feature, we are fine to discuss MDT requirement for NTN (e)RedCap UE. But it also makes sense to us to check with RAN1/2. For CHO and RACH-less HO for NTN (e)RedCap UE, we also agree to further check with RAN1/2 on the feasibility and necessity.

Regarding CG-SDT and RA-SDT, in the current UE capability of TS38.306, it was defined that,

These RA-SDT and CG-SDT can be optionally supported by NTN capable UE, and in current RAN4 spec, only RA-SDT requirement for RedCap has been defined. When considering NTN (e)RedCap UE, we didn’t see any technical feasibility issue to have RA-SDT and CG-SDT requirement for NTN (e)RedCap UE.

Proposal 4: RAN4 to define requirements for MDT, RA-SDT and CG-SDT requirement for NTN (e)RedCap UE.

Proposal 5: RAN4 to check with RAN1/2 about the feasibility of supporting NR Rel-17/18 Conditional Handover, and RACH-less Handover for NTN (e)RedCap UE.

Issue 6-3-2: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ?

<Way Forward> : Further discuss the following proposal - Proposal 1 (vivo, ZTE): - Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference. - Proposal 2 (Xiaomi): - For 1RX RedCap: Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy. - Proposal 3 (ZTE): - RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Qin shall be reused. - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN. - Proposal 4 (QC): - For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows: - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) - Handover delay Extension - Extended RLM OOS and BFD evaluation periods - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters - Measurement accuracy relaxation for SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP

In current TS38.133, the requirement for 1Rx RedCap has been relaxed from 2Rx requirement, e.g., increasing sample number, or relax the accuracy. In principle, the relaxation that RAN4 applied to 1Rx RedCap UE compared with 2Rx RedCap UE can be also applied to legacy FR1 NTN requirement for the FR1 NTN 1Rx (e)RedCap UE, and relaxation can be in terms of sample number for measurement/evaluation, offset to threshold for cell-reselection, measurement accuracy and so on. In last meeting, proposal 4 has provided a good example to list the relaxation, and we are open to further discuss the complete list.

Proposal 6: the relaxation that RAN4 applied to 1Rx RedCap UE compared with 2Rx RedCap UE can be also applied to legacy FR1 NTN requirement for the FR1 NTN 1Rx (e)RedCap UE, and relaxation can be in terms of sample number for measurement/evaluation, offset to the threshold for cell-reselection, measurement accuracy and so on.

Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN?

<Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC, Samsung): - The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. - Proposal 1a (CATT): RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands. - Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs. - Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs. - Proposal 1b (CMCC): Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario: - For GEO deployment, all DRX and eDRX cycle can be supported. - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle - Proposal 1c (Samsung): - RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario. - Proposal 1d (QC): For eRedCap UE, RAN4 can consider the following aspect for RRM requirement definition if necessary and applicable for NR NTN support. - Enhanced eDRX in RRC INACTIVE - Proposal 2 (LG): - For NTN RRM requirements for RedCap UE, - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

RAN4 needs more discussion on whether new requirement shall be introduced for RRM relaxation and eDRX. For example, in legacy NTN requirement, the 2.56s DRX is not assumed to be well utilized in the scenario where earth-moving LEO is deployed as the target satellite or serving satellite for measurement. The main reason is: if earth-moving LEO is used and the footprint on earth is not that large, e.g., couple kilometers, then the longer measurement delay due to using 2.56s will fail the mobility. If eDRX>10.24sec is considered/configured (PTW window is used) and earth-moving LEO is serving cell or is configured as target satellite for UE’s measurement, we think if DRX in PTW window is not greater than1.28s, UE shall complete the target NTN satellite measurement/evaluation within single PTW window, i.e., no cross PTW window measurement/evaluation; however, if DRX in PTW window is greater than 1.28s, we still think UE shall be allowed to skip or drop the measurement/detection/evaluation, i.e., UE is not required to meet the requirement for 2.56s DRX cycle length for earth moving LEO deployment when eDRX with PTW window is configured.

However, if the eDRX is without PTW(eDRX <=10.24s) and UE follows the eDRX cycle to perform the measurement, the minimum eDRX cycle is 2.56s and it’s equivalent to the note 2 in the legacy NTN requirement. Thus, we think UE is not required to meet the requirement with eDRX without PTW for earth moving LEO deployment.

In order to simplify the requirement, we propose that UE is not required to meet the RRM requirement with eDRX for earth moving LEO deployment.

Proposal 7: The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. However, UE is not required to meet the RRM requirement with eDRX for earth moving LEO deployment.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

Proposal 2: for NTN (e)RedCap UE, if UE supports IE supportOfERedCap-r18 and does not support IE eRedCapNotReducedBB-BW-r18, only need to consider 15kHz SCS and 30kHz SCS in FR1 NTN RRM requirement; otherwise for all the other cases, consider 15/30/60kHz for FR1-NTN (e)RedCap RRM requirement.

Proposal 3: Same RRM requirement shall be applied to the eRedCap NTN UE with and without BB bandwidth reduction, only except that RRM requirements with 60kHz SCS can only be applied for the UE without BB bandwidth reduction.

Proposal 4: RAN4 to define requirements for MDT, RA-SDT and CG-SDT requirement for NTN (e)RedCap UE.

Proposal 5: RAN4 to check with RAN1/2 about the feasibility of supporting NR Rel-17/18 Conditional Handover, and RACH-less Handover for NTN (e)RedCap UE.

Proposal 6: the relaxation that RAN4 applied to 1Rx RedCap UE compared with 2Rx RedCap UE can be also applied to legacy FR1 NTN requirement for the FR1 NTN 1Rx (e)RedCap UE, and relaxation can be in terms of sample number for measurement/evaluation, offset to the threshold for cell-reselection, measurement accuracy and so on.

Proposal 7: The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. However, UE is not required to meet the RRM requirement with eDRX for earth moving LEO deployment.

# References

[1] R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT, RAN4#112

[2] R4-2411821, Topic summary for [112][226] NR_NTN_Ph3, CATT, RAN4#112