# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415545

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.2.2

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#112, and the open issues have been summarized in the topic summary for [112][218] for NR_RRM_Ph5 [2].

The revised WID has been approved in last RANP #105 meeting in [3], and the revision agreed is:

- The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

Issue 2-1: Clarification on the bullets in WID for this CSSF optimization

This issue has been discussed as scope refinement in RAN plenary #105 and the revision of WID has been agreed in [3], i.e., the CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement, and Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities.

Thus, we don’t need to further discuss this issue 2-1 in the RAN4.

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

We also think RAN4 can prioritize the baseline scenarios for CSSF optimization study as starting point, i.e., SSB based Intra-frequency measurement without MG; and the other aspects can be further discussed after concluding on the baseline case.

Proposal 1: the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

- Aspect 1: SSB based Intra-frequency measurement without MG, including:

  - TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

[Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

RAN plenary agreed in the WID that the baseline assumption on number of searchers is 2, and that’s also aligned with the RAN4 fundamental assumption from Rel-15. Some companies raised one issue that: in current spec, if UE supports per-FR MG, and if one FR measurement is using MG and the other FR measurement is not using MG, we don’t have a joint section of CSSF requirement to define a per-UE CSSF factor including both FRs, and that implies UE supports 3 searchers in this case. Unfortunately, we disagree with this assumption and interpretation, our view is: this is just a missing piece of current spec, and we don’t have specific test case to verify UE behavior in this case. In the field, the deployment of FR2 is very rear and the UE measurement speed cannot be easily verified in the current field log (at least no error observed in current industry). Additionally, searcher number change is a quite big challenge to UE implementation and design, and it will even impact the most essential part of the UE architecture. Thus, we don’t think the current spec implies certain UE capability beyond 2 searcher assumption, and 2 searcher assumption shall still be kept for this R19 WI.

The searcher number increasing would be a big challenge to UE implementation and complexity, and it shall be carefully checked only if certain special condition is met. For instance, when UE is capable of per-FR MG and UE with FR1+FR2 CA/DC is configured to perform measurement on FR1 carrier and FR2 carrier, the more than 2 searchers might be considered only if FR1 measurement doesn’t need MG but FR2 measurement needs per-FR2 MG. Moreover, the limitation will be probably needed on that only single searcher can be assumed on FR2 carrier measurement while up to 2 searchers are assumed on FR1 carrier measurement if UE is in FR1+FR2 CA/DC. Another possible condition could be that UE can only consider these 3 searchers in high mobility status or at cell edge status(serving cell quality is below certain threshold) to speed up the measurement. But such discussion shall not be the prioritized one in this WI.

To move forward, we also suggested that if companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. But such searcher discussion can still be contribution driven as business as usual.

Proposal 2: RAN4 only considers the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF.

- If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers.

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

[Way forward]: FFS: - Option 1 (Apple, CTC. Ericsson, Intel, Samsung, ZTE): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 2 (CATT, Apple, CTC, ZTE): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - Option 4 (CATT, HW): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers. (HW, CATT) - We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed. (HW)

For the case that UE is not in multiple-Rx simultaneous reception mode but it’s in CA/DC mode, the CSSF enhancement can be considered if multiple serving carriers are in the same band. In the current spec, the UE measurement capability requirement is defined as following:

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

For intra-frequency layer measurement, RAN4 assumed UE is allowed to only measure one CC in a same band, as highlighted in red above, but for serving CC measurement we require UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band. In R15 discussion, the reason behind such requirement is because RSRQ might be different among different serving carriers (different interference level), however, FR2 is actually noise-limited system rather than interference-limited system, and therefore we don’t think RSRQ would be too much different among serving CCs in the same band. Thus, we can enhance to require UE only measure one serving CC in the same FR2 band like what RAN4 specified for intra-frequency neighbor cell measurement.

In last meeting, companies mentioned that the candidate solution of optimizing CSSF (UE only needs to measure one serving CC per band if multiple serving CCs are in the same band) can be implemented by network configuration, e.g., Only one servingCellMO is configured on one SCC and no MOs are configured on the other SCCs, if multiple serving CCs are in the same band. We think it’s same situation as the discussion in R15 for neighbor cell measurements, i.e., we cannot limit the network implementation and configuration behavior, and therefore such enhancement to measure one serving cell in one band is based on the scenario when network configures multiple serving MOs in the same FR2 band to the UE.

Another option we are thinking for enhancement is to reduce the searcher occupancy ratio of PCC and PSCC for some special conditions, and then the SCC measurement can be speeded up by increasing the searcher occupancy ratio. For example, in current spec, for NR SA (e.g., FR2 only CA), the CSSF table is specified as following,

It means that: PCC measurement utilizes a dedicated searcher, and all the SCCs share the 2nd searcher. If SCC has FR2 neighbor cell configured CCs, the SCCs with FR2 neighbor cell utilize the 50% of the 2nd searcher. And all the other SCCs and inter-freq CCs equally share the other 50% of the 2nd searcher. If SCC has no FR2 neighbor cell configured CCs, all the SCCs and inter-freq CCs without MG equally share the 2nd searcher.

It's obvious that, if the PCC searcher occupancy ratio can be reduced, e.g., to 50% of the 1st searcher, the other 50% of the 1st searcher can be shared by SCCs to speed up the measurement. However, PCC measurement is most important thing to the RRM, and we can discuss the conditions in which PCC measurement can reduce the searcher occupancy ratio. Such enhancements and condition can also be applied to PSCC in DC case. The possible condition to reduce the occupancy of the searcher resource on PCC or PSCC could be similar as the RRM relaxation conditions in IDLE/Inactive, i.e., not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both.

Proposal 3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC:

- Option 1: UE only needs to measure one serving carrier per band if multiple serving carriers are in the same band.

  - If PCC in the band, measure PCC

  - Otherwise if PSCC in the band, measure PSCC

  - Otherwise if SCC is in the band, measure the SCC with neighbor cell MO

  - Otherwise up to UE implementation

- Option 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions

  - The conditions can be not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both.

Issue 2-6: feature capability of L3 measurement delay reduction by optimizing CSSF

[Way forward]: FFS: - Option 1 (Apple, Nokia): RAN4 to introduce a new individual capability for CSSF reduction in R19. - Option 1a(Nokia):The reduced CSSF shall be applied to the UE supporting the capability and starting from R19 - Option 2 (CATT, Apple): delay the capability discussion to the end of the core part.

Even though it’s too early to discuss the capability at this stage, we share the same view with companies that a new capability is needed for this CSSF optimization feature. We are also fine to delay this capability discussion to the end of the core part as legacy procedure.

Proposal 4: RAN4 to introduce a new individual capability for CSSF reduction in R19. But also fine to delay the capability discussion to the end of the core part.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

- Aspect 1: SSB based Intra-frequency measurement without MG, including:

  - TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15

Proposal 2: RAN4 only considers the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF.

- If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers.

Proposal 3: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC:

- Option 1: UE only needs to measure one serving carrier per band if multiple serving carriers are in the same band.

  - If PCC in the band, measure PCC

  - Otherwise if PSCC in the band, measure PSCC

  - Otherwise if SCC is in the band, measure the SCC with neighbor cell MO

  - Otherwise up to UE implementation

- Option 2: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions

  - The conditions can be not-at-cell-edge (PCell/PSCell quality is good enough) or low mobility or both.

Proposal 4: RAN4 to introduce a new individual capability for CSSF reduction in R19. But also fine to delay the capability discussion to the end of the core part.

# References

[1] R4-2414026, WF for NR_RRM_Ph5_Part1, Moderator (Apple), RAN4#112

[2] R4-2411813, Topic summary for [112][218]Moderator (Apple), RAN4#112

[3] RP-242380, Revised WID of NR RRM Phase 5, RANP #105