3GPP TSG-RAN WG4 Meeting #112bis                                  	                        R4-2415547

# Hefei, Anhui, China, October 14 – October 18, 2024

Agenda item:	6.1.1.3.2

Source:	Apple Inc.

Title:	Views on Maximum Number of MIMO Layers for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

In this discussion paper, we share our views on the maximum number of MIMO layers for 6Rx handheld UEs. Enabling 6Rx for large form-factor handheld UEs has the potential of improving DL coverage and throughput. However, it is important to understand if supporting 6 layers vs. 4 layers can produce practical throughput gains. We continue discussing our views below, including simulation results supporting our conclusions.

# Scope

During the previous RAN4#112 meeting in Maastricht, Netherlands, discussion continued on the general considerations for the MIMO layers evaluation for 6Rx UEs, and in addition, specific outcomes on 6 layers support. As captured in the previous WF [2], we highlight the discussion outcome below:

Sub-topic 3-1: General considerations for MIMO layer evaluation for 6Rx UE

Issue 3-1-1: Tightening BS EVM requirement

Agreement:

-	Do not tighten the minimum requirements of BS Tx EVM

-	Further discuss the assumption of Tx EVM for 6-layer performance evaluation.

Issue 3-1-2: 6-Layer Performance Evaluation Assumptions

Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios.

Issue 3-1-4: Performance requirements for 6Rx

Agreement: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices.

Sub-topic 3-2: 6-layer Support

Issue 3-2-1: 6-layer Support

Way Forward: RAN4 to further discuss the following options.

- Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

- Option 2: 6-layer support is feasible for FWA devices.

- Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point.

- Feasibility criteria

- Consider 4-layer vs 6-layer throughput performance.

- Simulation assumptions

- Only consider link-level simulation for performance evaluations.

- Simulation assumptions provided in Annex A as a starting point.

- Companies to clarify the settings used in simulation assumptions for BS correlation matrix and Tx EVM.

- Option 4: 6-layer support is not feasible for handheld UEs.

Issue 3-2-2: 6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

# Discussion

One of the main objectives of this WID is 1) to support 4 MIMO layers as a baseline, and 2) study the gain and feasibility of supporting 6 MIMO layers with a 6Rx UE. During the past meeting we were able to reach some baseline agreements to numerically determine whether it makes sense to support up to 6 MIMO layers. More specifically, we defined 1) that the feasibility criteria should be determined based on 4-layer vs 6-layer throughput performance, 2) we defined that only link-level simulations are relevant to this case, and 3) an initial set of detailed simulation assumptions were captured in Annex A of the past WF[2]. Even though this was good progress, we propose to reduce even further the parameter space to facilitate the decision. In addition, we provide our own simulation results to conclude that 6 MIMO layers is not really feasible for a handheld UE.

Simulation Parameters:

In Annex A, channel bandwidth has been defined to be either 40MHz or 100MHz. We believe that for any practical purposes, only one channel bandwidth should be defined for the feasibility exercise. We lean towards 40 MHz since it’s also a common channel bandwidth used for UE demodulation performance requirements. In addition to lower simulation runtime, any results obtained at this stage (Core Requirements) would be directly comparable and reusable during the next stage (Performance Requirements).

Observation #1: For MIMO layers evaluation, channel bandwidth is not a critical parameter since the performance comparison is a relative comparison.

Proposal #1: Consider only 40MHz as the channel bandwidth for MIMO layer evaluation.

With respect to the transmit and receive antenna configuration, two options have been proposed: Option 1: 16Tx, 6Rx and Option 2: 8Tx, 6Rx.

Observation #2: There is no clear justification to increase the number of Tx beyond 8 when considering only 6 MIMO layers.

Proposal 2: Consider only 8Tx/6Rx as transmit/receive antenna configuration.

With respect to propagation channel, still three options are being considered: Option 1: TDLA30-5, Option 2: TDLA30-10, and Option 3: TDLC300-100. From these three options, we believe that only TDLA30-10 is representative of the typical propagation needed for high throughput condition (that in addition RAN4 uses in its demodulation requirements). TDLC300-100 is simply too pessimistic, and TDLA30-5 is only a minor variation of TDLA30-10 that shouldn’t have a significant   impact.

Observation #3: TDLA30-10 propagation is representative of practical high throughput scenarios, while TDCL300-100 is not expected to sustain 6 layers.

Proposal #3: Consider only TDLA30-10 as the propagation channel.

With respect to rank, Rank 4, Rank 5 and Rank 6 are still under consideration in this exercise. However, the spirit of the WID is to evaluate if 6 MIMO layers can outperform 4 MIMO layers, despite other possible interpretations based on the specific WID wording. For example, if in any specific scenario only 5 MIMO layers outperform 4 MIMO layers instead of 6 layers, we believe the question remains whether the feature would be useful enough to be deployed. Our opinion in this regard is that it wouldn’t.

Observation #4: The spirit of the WID is to compare if 6 MIMO layers can outperform 4 MIMO layers.

Proposal #4: Focus on Rank 4 and Rank 6 only for the purposes of this evaluation.

With respect of how to choose MCS values to perform the analysis, we believe that the most robust and straightforward alternative is to consider link adaptation, as this better represents the dynamic performance that should be observed in the field. Fixed MCS at this point can be simple and provide some insight, but the most robust and comparable approach is adaptive MCS, which we use for our simulation results. As agreed in the previous meeting, we focus on the 64QAM and 256QAM range for this evaluation.

Observation #5: Link adaptation not only reduces the simulation effort but also better represent practical network conditions.

Proposal #5: Focus on 64QAM and 256QAM MCS values for high throughput comparison using link adaptation.

With respect to DMRS configuration, during the past meeting there were still three options being considered: Option 1 [1+1(4L), 2+0(6L)], Option 2 [2+2(4L), 2+2(6L)], Option 3: [1+1 (4L), 2+2 (6L)]. We believe that the fairest comparison is to consider Option 3: [1+1 (4L), 2+2 (6L)] as these two represent the typical configuration. It is expected that most deployments in the field will only use 1+1 for 4 layers since it would be enough. We somewhat understand the purpose of proposing Option 2 to equalize the overhead and truly compare only spectral efficiency differences. However, this does not preclude that operators will choose in practice to still deploy 1+1.

Observation #6: Typical DMRS configuration by operators is 1+1 for 4 layers, and configuring 2+2 for 6 layers is inevitable based on RAN1 design.

Proposal #6: Focus on Option 3: [1+1 (4L), 2+2 (6L)] as the selected DMRS configuration.

Finally, with respect to the assumption on TxEVM impairments, we consider a typical assumption in line with previous experience in demodulation performance requirements. This is, 6% for 64QAM and 3% for 256QAM. Considering that it has been agreed that BS RF minimum requirements would not be tightened, we believe that same requirements should be used for both feasibility studies and demodulation performance requirements. We do not favor custom tightening for the sake of this particular WI, but rather an overall tightening of the requirements if multiples companies support the claim that TxEVM in the field is much better than the minimum requirements. In addition, no tightening was agreed during 8Rx evaluation, and we believe same should apply here.

Observation #7: If multiples companies support the claim that TxEVM in the field is much better than its minimum BS RF requirements, then requirements should be tightened, which in turn is a broader scope beyond this WI.

Proposal #7: Do not relax TxEVM only for the purpose of this feasibility study and consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM.

3GPP MIMO Correlation Models:

ULA Models: We extend the use of the ULA 3GPP model based on \alpha and \beta parameters. In our previous contribution we considered MediumB as the existing 3GPP model with the least correlation. For \alpha = 0.3 and \beta = 0.005154, the resulting correlation matrix at Tx and Rx side are given by the figure below, however, with this correlation matrix we were unable to observe any gain at all.

Despite having relatively reasonable correlation coefficients in the Rx matrix, we can agree that the Tx correlation might be too high for this particular scenario. Hence, moving forward, we consider relaxing \alpha at the level of 0.005154 to portray a new pair of matrices. For our simulation results, we choose to consider two custom models described as

- Custom_1: \alpha = 0.005154 and \beta = 0.005154

- Custom_2: \alpha = 0, \beta = 0.005154

For illustration purposes, the obtained correlation matrices for the Custom_1 model are given below:

XPL Models: For completeness, we also consider the use of the XPL model. For our simulation results, we choose to consider two custom models described as

- Custom_3: \alpha = 0.005154, \beta = 0.005154, \gamma = 0.2

- Custom_4: \alpha = 0, \beta = 0.005154, \gamma = 0.2

In the case of the XPL model, the corresponding matrices are given below:

Overall, we expect the performance to be superior with XPL models.

Observation #8: ULA and XPL 3GPP correlation models with parameters as low as \alpha = 0.005154, \beta = 0.005154, (and \gamma = 0.2) are relatively optimistic but somewhat reasonable options for this study.

Proposal #8: Consider both the ULA and XPL 3GPP correlation model structure for this evaluation.

Simulation Results:

As the starting point of our analysis, it is worth noting that a good calibration exercise is to compare the performance of 4 MIMO layers vs 6 MIMO layers under somewhat ideal conditions. This is, we consider no correlation at both Tx and Rx side ( \alpha = 0, \beta = 0 ), and no effect of TxEVM. Under this approach we can obtain the best possible case, and then move forward with the analysis of how correlation and TxEVM may affect our conclusion.

In Figure 1 below, we show a comparison between Rank 4 and Rank 6 by using MCS adaptation for the entire range of 64QAM and 256QAM (MCS Table 2).

Figure 1: Absolute throughput comparison between Rank 4 and Rank 6 under the assumptions of no antenna correlation and no impact of TxEVM. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2.

At this point it’s already clear that any possible gain between 6 layers and 4 layers would occur at an extremely high SNR, which may be cataloged as non-practical or overly optimistic with respect to the SNR values that can be obtained in the field.

Observation #9: Performance gain between 6 layers and 4 layers is only visible at very high SNR >> 30dB under idealized conditions of no correlation and no TxEVM.

Moving on, the next step is to understand the effect of antenna correlation in this comparison. In Figure 2 below, we show the performance obtained with the ULA Custom models described above.

Figure 2: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom ULA model coefficients but no impact of TxEVM. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2.

It is immediately clear that the correlation at both Tx and Rx sides play an important role in the performance. Unfortunately, Rank 6 this time performs worse than Rank 4.

Observation #10: Rank 4 performs better than Rank 6 under optimistic MIMO antenna correlations and no TxEVM.

In the the next step we now include the effect of TxEVM in the results. See Figure 3.

Figure 3: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom ULA model coefficients and a TxEVM impairment of 6% for 64QAM modulation and 3% for 256QAM modulation. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2.

Once again, including this additional impairment significantly degrades the performance one more time. Here, Rank 6 show basically no throughput.

Observation #11: Under simulation assumptions, effect of TxEVM under the previously defined optimistic MIMO correlation models severely reduces throughput for 4 layers and reduces throughout to virtually zero for 6 layers case.

Finally, the last set of simulation results is about the use of customized XPL model results, for which we show the same analysis used in the case of ULA models. See Figure 4 and Figure 5.

Figure 4: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom XPL model coefficients but no TxEVM impact. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2.

Figure 5: Absolute throughput comparison between Rank 4 and Rank 6 considering antenna correlation based on custom XPL model coefficients and a TxEVM impairment of 6% for 64QAM modulation and 3% for 256QAM modulation. Results obtained using link adaptation over 64QAM and 256QAM MCS values in MCS Table 2.

Observation #12: Under custom XPL models performance improves for both 4 layers and 6 layers, however when TxEVM is considered, 4 MIMO layers still outperform 6 MIMO layers.

At this point, and based on the simulations results above, the conclusion is quite clear.

Proposal #9: Define that 6 MIMO layers under 6Rx is infeasible at least for handheld scenarios.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: For MIMO layers evaluation, channel bandwidth is not a critical parameter since the performance comparison is a relative comparison.

Proposal #1: Consider only 40MHz as the channel bandwidth for MIMO layer evaluation.

Observation #2: There is no clear justification to increase the number of Tx beyond 8 when considering only 6 MIMO layers.

Proposal 2: Consider only 8Tx/6Rx as transmit/receive antenna configuration.

Observation #3: TDLA30-10 propagation is representative of practical high throughput scenarios, while TDCL300-100 is not expected to sustain 6 layers.

Proposal #3: Consider only TDLA30-10 as the propagation channel.

Observation #4: The spirit of the WID is to compare if 6 MIMO layers can outperform 4 MIMO layers.

Proposal #4: Focus on Rank 4 and Rank 6 only for the purposes of this evaluation.

Observation #5: Link adaptation not only reduces the simulation effort but also better represent practical network conditions.

Proposal #5: Focus on 64QAM and 256QAM MCS values for high throughput comparison using link adaptation.

Observation #6: Typical DMRS configuration by operators is 1+1 for 4 layers, and configuring 2+2 for 6 layers is inevitable based on RAN1 design.

Proposal #6: Focus on Option 3: [1+1 (4L), 2+2 (6L)] as the selected DMRS configuration.

Observation #7: If multiples companies support the claim that TxEVM in the field is much better than its minimum BS RF requirements, then requirements should be tightened, which in turn is a broader scope beyond this WI.

Proposal #7: Do not relax TxEVM only for the purpose of this feasibility study and consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM.

Observation #8: ULA and XPL 3GPP correlation models with parameters as low as \alpha = 0.005154, \beta = 0.005154, (and \gamma = 0.2) are relatively optimistic but somewhat reasonable options for this study.

Proposal #8: Consider both the ULA and XPL 3GPP correlation model structure for this evaluation.

Observation #9: Performance gain between 6 layers and 4 layers is only visible at very high SNR >> 30dB under idealized conditions of no correlation and no TxEVM.

Observation #10: Rank 4 performs better than Rank 6 under optimistic MIMO antenna correlations and no TxEVM.

Observation #11: Under simulation assumptions, effect of TxEVM under the previously defined optimistic MIMO correlation models severely reduces throughput for 4 layers and reduces throughout to virtually zero for 6 layers case.

Observation #12: Under custom XPL models performance improves for both 4 layers and 6 layers, however when TxEVM is considered, 4 MIMO layers still outperform 6 MIMO layers.

Proposal #9: Define that 6 MIMO layers under 6Rx is infeasible at least for handheld scenarios.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2414285 “WF on 6Rx UE requirements”, AT&T, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024