# 3GPP TSG-RAN WG4 Meeting #112bis                               	R4-2415548

# Hefei, Anhui, China, October 14 – October 18, 2024

Agenda item:	6.23.4.1

Source:	Apple

Title:	Simulation Assumptions and Results for LP-WUR Core Requirements

Document for:	Discussion

# Introduction

In last RAN4#111 meeting, the WF for RRM requirements for LP-WUS/WUR was agreed in [1], and open issues have been summarized in [2]. In this contribution, we continue the discussion on the simulation assumptions, and we also provide some preliminary results.

# Simulation Assumptions

As mentioned before during RAN4#112, in our view the simulations performed by RAN4 should be as close as possible with respect to simulation assumptions used in RAN1’s evaluation. In general, RAN4 can modify and add necessary improvements tailored to RAN4 requirement design work. Our proposal goes in line to making sure that RAN4 assumptions for requirement design are substantially aligned with the performance expectations of RAN1 WG work. In addition, we propose to closely follow RAN4 RF agenda, which continues its parallel work.

Proposal 1: For LP-WUR based RRM requirements design, RAN1 link level simulation assumptions and RAN4 RF session’s simulation assumptions should be continuously monitored as the baseline for RAN4 RRM simulation work.

In the tables stated below, we reproduce the simulation assumptions discussed during the last RAN4#112 meeting. Fields highlighted in green have been agreed, and others are still up to discussion.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | Agreement: 2.6 GHz initially |
| Prior knowledge of Cell 1 / Cell 2 by the UE | Interfering cell (Cell 2) is not known to UE |
| DRX | No applicable for LP-WUR |
| BS transmit antennas for LP-SS blocks | 1 Tx |
| UE receive antennas | 1 Rx |
| Data and control channel subcarrier spacing | [Data, SSB and LP-SS have the same SCS] |
| Subcarrier spacing | 30KHz initially TBD for 15 KHz |
| Measurement period (in number of measurement samples) | [4, 5, other number could be studied upon a need] |
| LP-SS/SSB measurement interval | LP-SS: 320 ms SSB: 320 ms |
| LP-SS BW | 132 subcarriers for SCS=30kHz for LP-SS initially TBD for 15KHz SCS |
| SSS | 30KHz for SSS, TBD for 15KHz |
| Actual LP-SS transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| NR-PSS, NR-SSS (OFDM based LP-WUR) | - | To be indicated by companies | To be indicated by companies |
| LP-SS |  | OOK-1; or OOK-4 with M = [2,4] Note: M value [2 4] are up to company selection | when Cell 1 uses OOK-1; OOK-1 or NR signal is used for Cell 2 when Cell 1 uses OOK-4, OOK-4 with same M value as cell 1 or NR signal is used for Cell 2 |
| LP-SS pattern |  | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them | [M sequence] [Golden sequence] [Computer search sequence] Note: Company can simulate one or all of them |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation | - | QPSK | QPSK |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency offset relative to UE frequency reference | Hz | OFDM based receiver [5] ppm OOK based receiver [ 10 20] ppm | N/A |
| Timing error |  | Residual timing error + timing drift (frequency offset* 320ms (reference signal periodicity) Residual timing error: company report |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | CP/2 |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | ms | 0 | 3 ms |
| SNR | dB | SNR setting for serving and interference cell are derived based on agreement of Issue 2-1-1-1 |  |
|  |  | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-2.7] - When SNR of cell 2 is 6 dB lower compared with cell 2 SNR = [-2.4] | When Ês/Iot = -3 dB - When SNR of cell 2 is 9 dB lower compared with cell 2; SNR = [-11.7] - When SNR of cell 2 is 6 dB lower compared with cell 2; SNR = [-8.4] |
|  |  | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = -0.5 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
|  |  | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 | When Ês/Iot = 2 dB Note: Determine the SNR based on based on agreement of Issue 2-1-1-1 |
| Ês/Iot | dB | -3; -0.5dB; 2dB | N/A |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | 3 km/h |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [3th/5th Order Butterworth with 3.96MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4 or 8-bitADC] |
| [Receiver Sampling Rate for LP-SS only] | [3.84 or 7.68MHz] |

From Table 1 we can observe that a specific sequence, or a set of LP-SS sequence hasn’t been defined in RAN4. This is also the case in RAN1. However, to

Observation 1: For LP-WUR based RAN1 LP-SS design and RAN4 RRM requirements, no specific LP-SS sequence has been defined yet.

Observation 2: Previous RAN1 agreement states that 4 LP-SS sequences should be defined in terms of facilitating co-channel operation.

Proposal 1: Define a bundle of 4 LP-SS sequences for simulation results alignment. Once RAN1 has defined definitive sequences, switch to the final design.

In addition, another aspect that is not clearly defined yet is related to the residual timing offset. In our RAN1 contribution we have proposed the use of a preamble in LP-WUS to counteract the case where LP-SS may not be sufficient to reach sufficient time synchronization. Whether a preamble is necessary or not will finally depend on the time synchronization requirement. An OOK-based receiver is in general quite robust against frequency offset. However, the frequency offset affects the achievable time offset, which can have an impact. As previously agreed, we consider a model where the timing error is equal to Residual timing error + timing drift (frequency offset * 320ms (reference signal periodicity). Assuming no frequency synchronization is performed, and 20ppm is used for time tracking, the time drift for 320ms (which is assumed as the initial periodicity of LP-SS) would be 6.4us, which is too long. This means that other measures may need to be considered, and would need to be stated explicitly.

Observation 3: Time synchronization may be challenging for OOK-4 receivers and other measures such as using MR to assist LR in frequency synchronization may be needed.

Proposal 2: RAN4 to discuss the implications, if any, of the use of a preamble in LP-SS.

Observation 4: The use of a preamble or MR assistance may allow LP-SS being transmitted with periodicities longer than 320ms, which can help reducing power consumption.

Proposal 3: RAN4 to consider LP-SS periodicities of 320ms and above, such as 640ms, 1280ms for evaluating requirement design.

On timing synchronization error performance, the main assumptions are summarized in Table 4 below. Specifically, we checked both 5ppm and 20ppm for the residual frequency error in the time drifting model. Length-16 sequences are used. For length-7 or length-8 sequences, it is difficult to find 4 sequences with good cross-correlation properties, except if we use different shifted version of a m-sequence. But this can impact the potential robustness of the sequence design, as the UE may accidentally find the LP-SS from a neighbor cell. So we are not providing any evaluation results for the shorter length.

Table 4 Simulation assumptions for LP-SS timing synchronization error evaluation

| LP-SS Periodicity | 320 ms |
| --- | --- |
| Residual frequency error | 5ppm, 20ppm |
| Time drifting model | Linear drifting based on residual frequency error |
| LP-SS sequences | Length: 16 [1 1 1 1 1 0 0 1 1 0 1 0 1 1 0 0] [1 0 1 1 1 0 0 0 1 1 1 0 1 1 0 1] [1 1 0 1 0 1 1 1 1 1 1 0 0 1 0 0] [1 0 1 0 1 1 0 0 1 0 0 1 1 1 1 1] |
| OOK-4 | M = 2 |
| Channel | AWGN |

Figure 1: CDF of time synchronization error for 5ppm residual frequency error

Note that we assume the sliding window for LP-SS detection is sufficiently large to cover the timing error.

Observation 5: With the given simulation assumptions, for LP-SS with M = 2 and length 16, with 5ppm residual frequency error and AWGN channel, SNR = -3 dB can achieve +/- 1.1us time error for 90% of the time, and SNR = -6 dB can achieve +/- 4us time error for 90% of the time.

Proposal 4: RAN4 to consider 0.5us, 1us, and 4us as exploration parameters for RRM requirement design.

For the serving cell RRM measurement, we have some preliminary results based on perfect timing using a single LP-SS occasion.

Observation 6: With the given simulation assumptions, for LP-SS with length 16 and AWGN channel, assuming perfect timing and a single-shot measurement, SNR = -3 dB can achieve +/- 0.8 dB accuracy in RSRP measurement for M = 2, and +/- 1.2 dB accuracy for M = 4, for 90% of the time.

# Implications to Requirement Design

Based on the simulation assumptions stated above, we believe that RRM requirements can be crafted in terms of several degrees of freedom

    - Observation 7: Periodicity of LP-SS, defined in ms, can impact requirements since a larger periodicity can deteriorate time/frequency tracking, hence detection performance.

    - Observation 8: LP-SS Duration and/or Repetition Number in time domain within a slot, or across consecutive slots has an impact since increasing repetition improves detection performance.

    - Observation 9: Number of measurement instances performed by the UE to perform L1 filtering has an impact since improved performance can reduce the number of measurements performed by UE, i.e. improving latency.

    - Proposal 5: Design RRM requirements based on periodicity, LP-SS duration and number of measurement instances.

Finally, one last aspect is the fact that OFDM LR, if approved the overlaid sequences in LP-SS, count with two reference signals at disposal.

    - Proposal 6: RAN4 to study the implications for RRM requirements design if OFDM LR has available both LP-SS and SSS for performing measurements.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Observation 1: For LP-WUR based RAN1 LP-SS design and RAN4 RRM requirements, no specific LP-SS sequence has been defined yet.

Observation 2: Previous RAN1 agreement states that 4 LP-SS sequences should be defined in terms of facilitating co-channel operation.

Proposal 1: Define a bundle of 4 LP-SS sequences for simulation results alignment. Once RAN1 has defined definitive sequences, switch to the final design

Observation 3: Time synchronization may be challenging for OOK-4 receivers and other measures such as using MR to assist LR in frequency synchronization may be needed.

Proposal 2: RAN4 to discuss the implications, if any, of the use of a preamble in LP-SS.

Observation 4: The use of a preamble or MR assistance may allow LP-SS being transmitted with periodicities longer than 320ms, which can help reducing power consumption.

Proposal 3: RAN4 to consider LP-SS periodicities of 320ms and above, such as 640ms, 1280ms for evaluating requirement design.

Observation 5: With the given simulation assumptions, for LP-SS with M = 2 and length 16, with 5ppm residual frequency error and AWGN channel, SNR = -3 dB can achieve +/- 1.1us time error for 90% of the time, and SNR = -6 dB can achieve +/- 4us time error for 90% of the time.

Proposal 4: RAN4 to consider 0.5us, 1us, and 4us as exploration parameters for RRM requirement design.

Observation 6: With the given simulation assumptions, for LP-SS with length 16 and AWGN channel, assuming perfect timing and a single-shot measurement, SNR = -3 dB can achieve +/- 0.8 dB accuracy in RSRP measurement for M = 2, and +/- 1.2 dB accuracy for M = 4, for 90% of the time.

    - Observation 7: Periodicity of LP-SS, defined in ms, can impact requirements since a larger periodicity can deteriorate time/frequency tracking, hence detection performance.

    - Observation 8: LP-SS Duration and/or Repetition Number in time domain within a slot, or across consecutive slots has an impact since increasing repetition improves detection performance.

    - Observation 9: Number of measurement instances performed by the UE to perform L1 filtering has an impact since improved performance can reduce the number of measurements performed by UE, i.e. improving latency.

    - Proposal 5: Design RRM requirements based on periodicity, LP-SS duration and number of measurement instances.

    - Proposal 6: RAN4 to study the implications for RRM requirements design if OFDM LR has available both LP-SS and SSS for performing measurements.

# References

[1] R4-2413899, WF for RRM core requirements for LP-WUS/WUR, Moderator (vivo), RAN4 #112

[2] R4-2411818, Topic summary for [111][229] NR_LPWUS, vivo, RAN4 #112

[3] R4-2413898, Coffee break discussion minutes for [112][223] NR_LPWUS, vivo, RAN4 #112