3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415550

Hefei, Anhui, China, 14th – 18rd October, 2024

Agenda item:		6.8.3.4

Source:		Samsung

Title:	Discussion on RRM requirements of Rel-19 NR-NTN in less than 5MHz

Document for:		Discussion

- Introduction

The objectives of Rel-19 “Enhanced for NR NTN and IoT NTN” are described in latest WID [1], and “less than 5MHz for NTN” are shown as below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In last meeting, we have already discussed the yellow part above and achieved some agreements. In this contribution, we continue to discuss the RRM core requirements for less than 5MHz for NR-NTN.

- Discussion

In last RAN4#112 meeting, RAN4 achieved some agreements in [2]. There are still some open issues in [2] and we will discuss them one by one as below.

In RAN4#112 meeting, the handover delay requirement for NTN less than 5MHz is agreed to use extension of delay for TN less than 5MHz as baseline, it is listed in WF [2]:

#### Issue 1-3-2: Handover Agreement: Define HO delay requirement for NTN less than 5MHz, and take extension of delay for TN less than 5MHz as baseline. For NTN specific HO requirement: - Time/location-based CHO delay requirement will be defined for NTN less than 5MHz. - Satellite switch time requirement will be defined for NTN less than 5MHz. - Further discuss the details of the requirements.

In current 38.133, the extension of Interruption time as part of handover delay is defined in T∆

T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

In RAN4 BDaT session, the system parameters were agreed in [3] as:

Issue 1-1-1: Channel bandwidth - Agreement - Only introduce 3MHz channel bandwidth with 15kHz SCS Issue 1-1-3: Maximum transmission bandwidth configuration(NRB) - Agreement - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

In TN less than 5MHz system, the root is the additional time is needed for SIB and MIB reading because of the SSB is punctured to 3MHz. In NTN less than 5MHz scenario, according to the agreement above in other session, the system parameter is also 3MHz with 15 PRBs. Therefore, the same issue as TN system of less than 5MHz occurs. Compare current NTN handover delay procedure and TN handover delay procedure, the process and timeline are the same. So, the only difference shall be the time of SIB and MIB reading due to punctured BW. Due to same frame structure and physical layer parameter, we think the simulation in TN of 3MHz can be reused (only difference is the channel condition). Therefore, the delay requirements in NTN less than 5MHz system can be defined as the same as TN in 3MHz.

Proposal 1: In 6.1C.1.2, NR SAN FR1-NR SAN FR1 handover delay requirements:

For both RACH-based handover and RACH-less handover, Dhandover equals to the RRC procedure delay plus the interruption time, the interruption time is defined as:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

For the T∆, use two more samples for extended time.

T∆ = 3*Trs for both known and unknown target cells for 3MHz NTN FR1.

Compared to TN system, the other difference is: in NTN system, the conditional handover is supported including:

- Time/location-based CHO with L3 measurement

- Time/location-based CHO without L3 measurement.

In time/location-based CHO with L3 measurement, the legacy CHO handover delay is as:

DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution

In those components, TRRC, TEvent_DU, TCHO_execution should be the same as legacy. Only Tmeasure or Tinterrupt including time for fine time tracking depends on PBCH detection such as L3 SSB index reading. However, in the requirements, they are referred to L3 measurements. If RAN4 define new L3 requirements, it can be used directly for Tmeasure. RAN4 to discuss the impacts on Tmeasure and T∆ (time for fine time tracking and acquiring full timing information) in Tinterrupt.

For time/location-based CHO without L3 measurements, the legacy CHO handover delay is as:

DCHO = TRRC + TEvent_DU + Tinterrupt + TCHO_execution

In those components, TRRC, TEvent_DU, TCHO_execution should be the same as legacy. Only Tinterrupt depends on T∆ which has impacts.

Proposal 2: For time/location-based CHO with L3 measurement and time/location-based CHO without L3 measurement, due to the same root, the T∆ in Tinterrupt should be extended as well:

T∆ = 3*Trs for both known and unknown target cells for 3MHz NTN FR1. Others are as legacy.

In 6.1C.3.2, for the satellite switching with re-synchronization, for both hard/soft switching, in the satellite switching delay: the switching delay equals the interruption time, the interruption time is defined as:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

only T∆ should be extended as well.

Proposal 3: For satellite switching with re-synchronization, for both hard switch and soft switch, only the T∆ in Tinterrupt should be extended as well:

T∆ = 3*Trs for both known and unknown target cells for 3MHz NTN FR1.

In RAN4#112 meeting, the timing requirements for NTN less than 5MHz is still FFS, listed in WF [2] as below:

#### Issue 1-3-3: whether timing requirements for NTN in current TS38.133 will be impacted becasue of less than 5MHz bandwidth WF: FFS on - Option 1: - No impacts

For timing requirements, no impacts due to less bandwidth.

Proposal 4: In 3MHz for FR1-NTN, for timing requirements, no RRM impacts.

In RAN4#112 meeting, the RLM/BFD/CBD requirements for NTN less than 5MHz is still FFS listed in WF [2] as below:

#### Issue 1-3-4: RLF/BFD/CBD WF: FFS on - Proposal 1: whether RAN4 need to define hypothetical PDCCH transmission parameter for RLF/BFD/CBD - Yes - FFS - Proposal 2: - The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario

Due to the reduced number of PRBs in 3MHz (15PRBs), the current hypothetical PDCCH transmission parameter cannot be used anymore. They should be defined for 3MHz in FR1-NTN. RAN4 can reuse the parameters in TN 3MHz for SSB-based RLM/BFD requirements.

Proposal 5: For SSB based RLM requirements, the hypothetical PDCCH transmission parameter should be re-defined for 3MHz as:

PDCCH transmission parameters for out-of-sync evaluation for NTN 3MHz in FR1-NTN:

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| Channel BW | 3MHz |
| DL Transmission BW | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-DistributedNote 1 |
| NOTE 1: For 15 PRB Transmission BW, distributed mapping may lead to unexpected out-of-sync indication. |  |

PDCCH transmission parameters for in-sync evaluation for NTN 3MHz in FR1-NTN:

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| Channel BW | 3MHz |
| DL Transmission BW | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-Distributed |

Proposal 6: For SSB based BFD requirements, the hypothetical PDCCH transmission parameter should be re-defined for 3MHz as:

PDCCH transmission parameters for beam failure instance for NTN 3MHz in FR1-NTN

| Attribute | Value for BLER |
| --- | --- |
| Channel BW | 3MHz |
| DL Transmission BW | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 8 Note 1 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-DistributedNote2 |
| NOTE 1: PDCCH puncturing as defined in 38.214 [26] applies. NOTE 2: For 15PRB Transmission BW, distributed mapping may lead to unexpected beam failure detection. |  |

In RAN4#112 meeting, the L3 measurement requirements for NTN less than 5MHz is still FFS listed in WF [2] as below:

#### Issue 1-3-5: Measurement(SSB index detection requirements) WF: FFS on: - Proposal 1: Whether the time period for time index detection within NTN intra/inter-frequency measurements with/without MG shall be redefined. - Option 1: Yes - Option 2: FFS - Proposal 2a: - The new SSB index detection requirements can be defined by add number of SSB samples as Rel18 TN with less than 5MHz. - Proposal 2b: - RAN4 shall define the related requirements for less than 5MHz in NTN scenario with considering Klayer1_measurement , Kmulti_SMTC and Kgap

In current TN less than 5MHz, for SSB based L3 measurement, for time period for time index detection of both intra-frequency measurement without MG or with MG, 4 additional samples are needed. For inter-frequency measurement without MG or with MG, 3 additional samples are added. For NTN 3MHz, there are no more difference for the simulation parameters except the channel condition. So, we think 4/3 samples are needed as well for intra-frequency measurement and inter-frequency measurement, respectively.

Proposal 7: For Intra-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(7 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

For Intra-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

For Inter-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

For Inter-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, Ceil(6 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(6 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(6 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

In RAN4#112 meeting, the SSB based L1-RSRP measurement requirements for NTN less than 5MHz is still FFS listed in WF [2] as below:

#### Issue 1-3-6: L1-RSRP Measurement WF: FFS on - Proposal 1: - For L1-RSRP measurement based on SSB there is no RRM impacts,

For SSB based L1-RSRP measurement, it has no impacts of punctured PBCH. The SSS is the same as that in larger than 5MHz. The legacy SSB based L1-RSRP measurement period requirements in FR1-NTN can be reused for that in 3MHz FR1-NTN system.

Proposal 8: For SSB based L1-RSRP measurement, the legacy SSB based L1-RSRP measurement period requirements in FR1-NTN can be reused for that in 3MHz FR1-NTN system.

- Conclusion

In this contribution, we provide our proposals for RRM core requirements for less than 5MHz for NR-NTN:

Proposal 1: In 6.1C.1.2, NR SAN FR1-NR SAN FR1 handover delay requirements:

For both RACH-based handover and RACH-less handover, Dhandover equals to the RRC procedure delay plus the interruption time, the interruption time is defined as:

Tinterrupt = Tsearch + TIU + Tprocessing  + T∆ + Tmargin ms

For the T∆, use two more samples for extended time.

T∆ = 3*Trs for both known and unknown target cells for 3MHz NTN FR1.

Proposal 2: For time/location-based CHO with L3 measurement and time/location-based CHO without L3 measurement, due to the same root, the T∆ in Tinterrupt should be extended as well:

T∆ = 3*Trs for both known and unknown target cells for 3MHz NTN FR1. Others are as legacy.

Proposal 3: For satellite switching with re-synchronization, for both hard switch and soft switch, only the T∆ in Tinterrupt should be extended as well:

T∆ = 3*Trs for both known and unknown target cells for 3MHz NTN FR1.

Proposal 4: In 3MHz for FR1-NTN, for timing requirements, no RRM impacts.

Proposal 5: For SSB based RLM requirements, the hypothetical PDCCH transmission parameter should be re-defined for 3MHz as:

PDCCH transmission parameters for out-of-sync evaluation for NTN 3MHz in FR1-NTN:

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| Channel BW | 3MHz |
| DL Transmission BW | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 8 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-DistributedNote 1 |
| NOTE 1: For 15 PRB Transmission BW, distributed mapping may lead to unexpected out-of-sync indication. |  |

PDCCH transmission parameters for in-sync evaluation for NTN 3MHz in FR1-NTN:

| Attribute | Value for BLER Configuration #0 |
| --- | --- |
| Channel BW | 3MHz |
| DL Transmission BW | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 4 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-Distributed |

Proposal 6: For SSB based BFD requirements, the hypothetical PDCCH transmission parameter should be re-defined for 3MHz as:

PDCCH transmission parameters for beam failure instance for NTN 3MHz in FR1-NTN

| Attribute | Value for BLER |
| --- | --- |
| Channel BW | 3MHz |
| DL Transmission BW | 15 PRBs |
| DCI format | 1-0 |
| Number of control OFDM symbols | 3 |
| Aggregation level (CCE) | 8 Note 1 |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |
| Bandwidth (PRBs) | 15 |
| Sub-carrier spacing (kHz) | Same as the SCS of RMSI CORESET |
| DMRS precoder granularity | REG bundle size |
| REG bundle size | 6 |
| CP length | Normal |
| Mapping from REG to CCE | Non-DistributedNote2 |
| NOTE 1: PDCCH puncturing as defined in 38.214 [26] applies. NOTE 2: For 15PRB Transmission BW, distributed mapping may lead to unexpected beam failure detection. |  |

Proposal 7: For Intra-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, ceil( 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 7 x Kp x Klayer1_measurement) x Kmulti_SMTC x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | Ceil(7 x Kp x Klayer1_measurement) x Kmulti_SMTC x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

For Intra-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(120ms, 7 x Kgap x Kmulti_SMTC x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x Kgap x Kmulti_SMTC x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 7 x Kgap x Kmulti_SMTC x max(MGRP, DRX cycle) x CSSFintra |

For Inter-frequency measurements without measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| DRX cycle | TSSB_time_index_inter |
| --- | --- |
| No DRX | max(120ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter Ksatellite |
| DRX cycle≤ 320ms | max(120ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter Ksatellite |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFinter Ksatellite |
| NOTE 1: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

For Inter-frequency measurements with measurement gaps, the time index detection requirements for 3MHz FR1-NTN should be defined:

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(120ms, Ceil(6 x Kgap) Max(MGRP, SMTC period NOTE2)) CSSFinter K_satellite |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(6 1.5 x Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter K_satellite |
| DRX cycle > 320ms | Ceil(6 x Kgap) DRX cycle CSSFinter K_satellite |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: SMTC period is the SMTC period in SMTC configuration which is associated with the target cell to be measured configured in SSB-MTC4List-r17. |  |

Proposal 8: For SSB based L1-RSRP measurement, the legacy SSB based L1-RSRP measurement period requirements in FR1-NTN can be reused for that in 3MHz FR1-NTN system.

- Reference

[1] RP-241985, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] R4-2413974, WF for RRM of NTN with channel BW less than 5MHz, Xiaomi

[3] R4-2413528, WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF , Xiaomi