3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415551

Hefei, Anhui, China, 14th – 18rd October, 2024

Agenda item:		6.19.3.1

Source:		Samsung

Title:	Discussion on RRM requirements of UE-initiated beam management in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of Rel-19 MIMO Phase 5 are described in [1] including:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last RAN4 meeting, we analyzed the overall impacts of RRM requirements related to the above objectives and achieved some agreements.

In this contribution, we continue to discuss the RRM requirements for objective#1: UE-initiated/event-driven beam management.

- Discussion

In RAN4#112 meeting, RAN4 agreed to define requirement for event triggered measurement reporting delay, which is listed in WF [2] as below:

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period? Agreement: - RAN4 will define Event triggered measurement reporting delay requirement - Further discuss: - Whether to support both mode A and mode B - Whether to differentiate mode A and mode B - Other related aspects

For this new reporting delay requirements, it is still FFS on whether to support both mode A and mode B, and whether to differentiate mode A and mode B.

In RAN1 design of UE-initiated/event-driven beam management, such two modes are agreed in RAN1#116-bis meeting as below:

Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, following modes are supported: - Mode A (dynamically scheduling UCI by gNB): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report - FFS: Request format, e.g., SR or a new UCI type. - Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. - Step 3: Beam report is transmitted in second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - This option is basic UE capability (i.e. all UE supporting UE-initiated/event-driven beam reporting should support this feature). - No new DCI format is introduced. - Mode B (UCI in pre-configured resource(s) for second UL channel): - Step 1: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report - FFS: Notification format, e.g., SR or a new UCI type. - Step 2: UE transmits the beam report in the second UL channel. - FFS: Details on the second UL channel, e.g., whether the second UL channel is PUCCH, PUSCH or both - The notification in Step1 is in a separate reporting instance from the beam report in Step 2. FFS: Whether UE receives acknowledge information with response to each step for all options For above procedures, cross-CC beam reporting is supported for both options. - FFS: Details.

For the difference between mode A and mode B in RAN1, mode A is for UE request an uplink resource for beam report while mode B uses the pre-configured resource for beam report. The differences are marked:

P1 A: UE transmits a first PUCCH (one-bit/multi-bit) to request a resource for a second UL channel to carry beam report B: UE transmits a first PUCCH (one-bit/multi-bit) notifying a second UL channel to carry beam report

P2 A: Step 2: UE detects the DCI format to indicate a resource for a second UL channel to carry beam report. B: No

P3 A: Beam report is transmitted in second UL channel. B: UE transmits the beam report in the second UL channel

The start point of the measurement reporting delay can be the event triggered a measurement report.

However, for the end point of the measurement reporting delay, there are different options. If the end point is chosen from the UE transmits the beam report in the second UL channel. For Mode A and Mode B, there is difference due to the steps above. If the end point is chosen from P1 which UE transmits a first PUCCH, for Mode A and Mode B, they are the same. For Mode A, the additional time is for the gNB scheduled UL channel. However, this time duration is no relationship with measurement and depends on the gNB scheduling. If RAN4 defined such measurement reporting delay requirements, there is no need to including the time gap for scheduling UL. Then, the end point of the measurement reporting delay can be the point when UE transmits a first PUCCH. Therefore, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits a first PUCCH.

Proposal 1: RAN4 to specify the Event triggered measurement reporting delay as time between an event that will trigger a measurement report and the point when the UE transmits a first PUCCH.

Proposal 2: If P1 is agreed, this requirement can be applied for both mode A and mode B. There is no difference of the delay for mode A and mode B.

For the Event, it is agreed in RAN1#116-bis meeting for Event-2 should be supported:

Agreements: Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

In addition, in RAN1#118 meeting, there is a working assumption to include Event-1 and Event-7 as below:

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

All the event are based on the measurement of the quality of the current/new beam(s) such as L1-RSRP. In RAN4#112 meeting, the measurement and CSI-RS measurement are general agreed as:

Issue 2-1-5: measurement metrics for beam report? Agreement: - Based on L1-RSRP - Note: others can be revisited if RAN1 have further agreement Issue 2-1-6: L1-RSRP for CSI-RS measurement? Enhancement for UE-initiated/event-driven beam management Agreement: Define CSI-RS based L1-RSRP measurement requirements. CSI-RS is periodic CSI-RS.

In RAN1#116 meeting,

RAN1: Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management

Therefore, RAN4 can specify the RRM requirements for event triggered measurement reporting delay shall be within T_L1_measurement_period. According to the measurement RS, both SSB and CSI-RS are supported for current beam.

If it is based on SSB, T_L1_measurement_period can use the legacy SSB based L1 measurement period. If it is based on CSI-RS, T_L1_measurement_period can use the legacy CSI-RS based L1 measurement period.

Proposal 3: The event triggered measurement reporting delay shall be within T_L1_measurement_period. T_L1_measurement_period can be based on TL1-RSRP_Measurement_Period_SSB or TL1-RSRP_Measurement_Period_CSI-RS. If it is based on SSB for inter-cell, T_L1_measurement_period can be based on TL1-RSRP_Measurement_Period_SSB_CDP.

For the beam report, RAN1 agreed to comprise N beams. N includes at least one new beam that satisfies the event-2 declaration/triggering condition.

The definition of N is further concluded in RAN1#118 meeting as:

Agreement On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4} - FFS: additional candidate value(s) of {5, 6, 7, 8} - FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

Proposal 4: For FR2, the T_L1_measurement_period is scaled by N.

Issue 2-1-7: Whether to specify new unified TCI state switching delay? <Way forward> - FFS on whether to define new TCI state switching delay or existing requirements are sufficient

In RAN4#112 meeting, companies are suggested to discuss whether to define new TCI state switching delay or existing requirements are sufficient. The motivation is for the new beams. If the TCI states of RS for new beams hasn’t been activated, there might be TCI state switching.

In RAN1#118 meeting, it is agreed that the RS(s) for new beams are explicitly configured in one RS resource set associated with an CSI reporting configuration:

Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

By our understanding for associated with an CSI reporting config, the TCI state has been activated, no need to define new TCI state switching delay requirements. This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion.

Proposal 5: No need to define new TCI state switching delay requirements. This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion.

Issue 2-1-8: Whether to filter L1-RSRP? <Way forward> FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

In RAN4#112 meeting, one company suggest to RAN4 specify RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

In RAN1#117 meeting, the agreement is shown as:

Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

The time window is for the triggering event determination for such event, the measurement delay is after the event trigger. The L1-RSRP measurement period in event triggered measurement reporting delay is no need to be filtered.

Proposal 6: The L1-RSRP measurement period in event triggered measurement reporting delay is no need to be filtered.

Issue 2-1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward> FFS: The assumption of N = 8 fine beam sweeping can be enhanced to realize more efficient BM. For example, the combination of coarse Rx beam and fine Rx beam would facilitate faster Tx and Rx beam refinement.

Compare to legacy L1-RSRP measurement, the only difference is the new L1 beam reporting is UE-initiated while the legacy is NW triggered. The principle of measurement of RSRP are the same.

Proposal 7: No enhancement for beam sweeping in event triggered L1 beam reporting.

Issue 2-1-11: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward> - Option 1: No new accuracy requirements for existing L1 measurements are required. - Option 2: RAN4 need to discuss whether to update legacy accuracy requirements.

For the accuracy requitements, if the delay requitements are based on legacy L1 RSRP measurement, the legacy accuracy requitements can be reuse.

Proposal 8: The existing L1-RSRP accuracy requitements can be reused for event-driven beam management.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for UE-initiated/event-driven beam management in Rel-19 MIMO phase 5:

Proposal 1: RAN4 to specify the Event triggered measurement reporting delay as time between an event that will trigger a measurement report and the point when the UE transmits a first PUCCH.

Proposal 2: If P1 is agreed, this requirement can be applied for both mode A and mode B. There is no difference of the delay for mode A and mode B.

Proposal 3: The event triggered measurement reporting delay shall be within T_L1_measurement_period. T_L1_measurement_period can be based on TL1-RSRP_Measurement_Period_SSB or TL1-RSRP_Measurement_Period_CSI-RS. If it is based on SSB for inter-cell, T_L1_measurement_period can be based on TL1-RSRP_Measurement_Period_SSB_CDP.

Proposal 4: For FR2, the T_L1_measurement_period is scaled by N.

Proposal 5: No need to define new TCI state switching delay requirements. This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion.

Proposal 6: The L1-RSRP measurement period in event triggered measurement reporting delay is no need to be filtered.

Proposal 7: No enhancement for beam sweeping in event triggered L1 beam reporting.

Proposal 8: The existing L1-RSRP accuracy requitements can be reused for event-driven beam management.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung