3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415552

Hefei, Anhui, China, 14th – 18rd October, 2024

Agenda item:		6.19.3.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of Rel-19 MIMO Phase 5 are described in [1] including:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In last RAN4 meeting, we analyzed the overall impacts of RRM requirements related to the above objectives and achieved some agreements.

In this contribution, we continue to discuss the RRM requirements for objective#2~5 in Rel-19 MIMO phase 5.

- Discussion

2.1 CSI enhancement

In RAN4#112 meeting, the agreements have been already achieved in [2], and listed as below:

Issue 2-2-1: Whether RRM impacts exist by CSI enhancement to support up to 128 ports? Agreement: - No RRM impact by CSI enhancement to support up to 128 ports

It means for the first three sub-bullets in the second objective, there are no RRM impacts:

a. Type-I codebook refinement to support 128 CSI-RS ports

b. Type-II codebook refinement to support 128 CSI-RS ports

c. Extension of CRI(s)-based CSI reporting to supports 128 CSI-RS ports

In last RAN plenary meeting RAN#105, another sub-bullet was approved as:

- SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI

For this sub-bullet, during the RAN discussion, it will introduce the SRS group definition. The UE receive antennas are divided into two antenna groups. And each group corresponds to each CSI acquisition. In addition, for uplink, the UE antennas for SRS ports are also grouped such as two groups. It might have RAN4 impact for Demod performance. However, for RRM requirements, we only focus on the SRS usage of antenna switching. We defined the interruption time for it. By this sub-bullet, SRS port grouping is some processing part of TDD CSI procedure.

Proposal 1: No RRM impact by added SRS port grouping to support more than 4 layers.

2.2 UE reporting enhancement for CJT calibration

In RAN4#112 meeting, companies have already discussed the RRM impacts for UE reporting enhancement for CJT calibration but no agreement, the options are listed as below:

Issue 2-3-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? <Way forward> - Option 1: No RRM core requirements impact. - Option 2: RAN4 to define the measurement delay or measurement behaviour for the aperiodic standalone CJT reporting. Issue 2-3-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration? <Way forward> - Option 1: No RRM performance requirements impact. - Option 2: No accuracy requirements. Potential mapping tables are needed if any. - Option 3: Define accuracy requirements. - Option 4: Define accuracy requirements and report mapping tables.

For Issue 2-3-1, there are different views on whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration.

As we pointed out in last meeting, this new feature introduced in RAN1 is based on aperiodic reporting. RAN 1 specified all three types of reporting for:

    - Delay offset reporting

    - Frequency offset reporting

    - Phase offset report

In RAN1#117 meeting, RAN1 agreed:

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for delay offset reporting Dn,offset, i.e. AD, at least support the following values: {0.5CP, CP} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.75CP, 1.5CP, 14∆f, 112∆f,124∆f} Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the dynamic range for frequency offset reporting FOn, i.e. AFO, at least support the following values: {0.1ppm, 0.2ppm} - Decide, by RAN1#117, whether any of the following candidate values are supported: {0.025ppm, 0.05ppm, 1/(8t), 1/(16t), 1/(32t)} Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-P’ (DL/UL phase offset), =1 only (agreed in RAN1#116bis) implies that the measured/reported phase offsets {n,, n=0, 1, …, NTRP – 1, n≠nref} are associated with the entire configured CSI reporting band (i.e. ‘wideband’).

In RAN1#118 meeting, additional resolution is also supported:

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for frequency offset reporting FOn, additionally support MFO = 256. Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding the resolution for delay offset reporting Dn,offset, additionally support MD = {128, 256}.

However, all three CSI reporting are still based on the legacy CSI framework. The reporting type is still aperiodic measurement report. The timeline/offset need to follow the reporting delay for aperiodic measurement report. In the past, RAN4 didn’t define the measurement period for aperiodic reporting. When NW configures the aperiodic reporting, the UE just reports the measurement based on the measurement of the resources and according to when NW indicates UE should report. We suggest to follow the same principle as legacy.

Proposal 2: For CJT calibration reporting of delay offset/frequency offset/phase offset, no RRM core requirements.

For Issue 2-3-2, there are different views on whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration.

At least, the mapping tables should be specified in section 10. FFS on the accuracy requirements.

Proposal 3: For CJT calibration reporting of delay offset/frequency offset/phase offset, the mapping tables should be specified in section 10. FFS on the accuracy requirements.

2.3 3Tx: 3-antenna-port codebook-based UL transmission

In RAN4#112 meeting, the RRM core impact of 3TX is FFS in [2] as below:

Issue 1: RRM core impact analysis

Agreement: RRM core impact analysis:

| Topic | Description | RRM impact |
| --- | --- | --- |
| UE-initiated/event-driven beam management | UE-initiated/event-driven beam management | YES |
| CSI enhancement | Type-I codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | Type-II codebook refinement supporting up to a total of 128 CSI-RS ports | NO |
|  | CRI-based CSI refinement for up to 128 CSI-RS ports | NO |
|  | Aperiodic standalone CJT calibration reporting | FFS |
| 3TX | 3-antenna-port codebook-based UL transmission | FFS |
| Asymmetric DL sTRP/UL mTRP |  | FFS |

As we pointed out in last meeting, the only potential RRM impacts are for the requirements of Interruptions at NR SRS antenna port switching. In legacy RAN4 requirements, the interruption is related to the duration of SRS symbol. It is not special for MIMO no matter whether to use codebook-based UL transmission or non-codebook-based UL transmission.

If 3TX is supported, the 3 ports SRS transmission will be specified in general way. Although in last RAN4 meeting, in the WF on WI NR_ENDC_RF_Ph4 [3], it is still FFS as below:

<Way Forward> Issue 1-4: Interruption requirements at SRS antenna switching for 6Rx capable UE - Proposals - Option 1(Nokia): To wait for RAN1 conclusion on the 6Rx SRS antenna switching pattern to make sure that the legacy requirements can be reused. - Option 2(Huawei, Ericsson): The existing interruption requirements at SRS antenna switching are applicable to 6RX capable UE, and no specification impact is observed.

By our understanding, as mentioned above, if 3t6r is supported, no additional enhancement for MIMO 3-antenna-port codebook-based transmissions, we can reuse the same requirement in 3t6r SRS antenna switching in NR_ENDC_RF_Ph4 in a general way.

Proposal 4: If 3t6r is supported in NR_ENDC_RF_Ph4, it is supposed no additional enhancement for MIMO 3-antenna-port codebook-based transmissions, reuse the same requirement in 3t6r SRS antenna switching.

2.4 Asymmetric DL sTRP/UL mTRP

In last RAN plenary meeting RAN#105, another sub-bullet in asymmetric DL sTRP/UL mTRP was approved as:

- Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

In RAN4#112 meeting, the open issue related to two TAs are listed in [2] as below:

Issue 2-5-3: Whether RRM impacts exist by 2TA enhancement? <Way forward> - FFS on whether there are RRM impacts by 2TA enhancement.

In Rel-18, two TAs for m-DCI and m-TRP are introduced. RAN4 define such RRM requirements including: MRTD/MTTD, DL reference timing, other RRM requitements which have impacts such as L1-RSRP and TCI state switching delay requitements.

In RAN1 discussion of this bullet and as the description in WID, the only impact is removing the restriction of the configured coresetPoolIndex.

In Rel-18, the two DL reference timings for two TAGs are defined in m-DCI m-TRP scenarios.

For multi-DCI based multi-TRP operation with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point. . For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal. For intra-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1. For inter-cell multi-DCI based multi-TRP operation with two TAs, the reference point for PRACH transmission triggered by PDCCH order, - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG. Where NTA and NTA offset are commanded by the network independently for each TAG [TS 38.331]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in Table 7.1.2-2.

In Rel-19 asymmetric DL sTRP/UL mTRP, the scenarios are: UE can receive DL transmission and may transmit UL transmission from the TRP#1 while transmit UL transmission from multiple other TRPs.

In Rel-18 two TA mechanism, two uplink transmission timing are related to the reception of the downlink reference point for each TAG, which means for each TAG, there can be dedicated downlink reference point.

In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the same serving cell (intra-cell), for both two TAGs, they are corresponding to the same downlink reference point.

In addition, RAN1 support ICBM in asymmetric DL sTRP/UL mTRP scenarios as agreed in RAN1#117:

RAN1#117: Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the serving cell and additional PCI cell (inter-cell), for both two TAGs, they can be corresponding to the different downlink reference points.

Proposal 5: RAN4 to specify RRM requirements for two TAs in asymmetric DL sTRP/UL mTRP. Rel-18 uplink timing requirements can be used as start point. In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the same serving cell (intra-cell), for both two TAGs, they are corresponding to the same downlink reference point. If all TRPs are from the serving cell and additional PCI cell (inter-cell), for both two TAGs, they can be corresponding to the different downlink reference points. For each TAG, uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point.

For the TCI state switching delay requitements of Rel-19 asymmetric DL sTRP/UL mTRP scenarios, it is still open and the options are listed in [2] as below:

Issue 2-5-4: RRM core impacts of Active uplink TCI state switching delay for unified TCI? <Way forward> - Option 1: The legacy requirements can be reused. - Option 2: The legacy requirements cannot be reused. - Option 3: RAN4 to discussion how to update R17/R18 requirement for asymmetric DL sTRP/UL mTRP scenarios

As we pointed out in last meeting, for both FR1 and FR2, the source RS cannot be DL-RS.

Observation 1: For FR2, joint TCI state cannot be supported in asymmetric DL sTRP/UL mTRP scenario. For both FR1 and FR2, the UL TCI states can only be associated to SRS.

Observation 2: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement.

Proposal 6: In asymmetric DL sTRP/UL mTRP scenario:

      - The legacy requirements cannot be applicable.

      - No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

      - Do not define such requirements if the UL TCI state is associated to SRS.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for objective#2~5 in Rel-19 MIMO phase 5:

Proposal 1: No RRM impact by added SRS port grouping to support more than 4 layers.

Proposal 2: For CJT calibration reporting of delay offset/frequency offset/phase offset, no RRM core requirements.

Proposal 3: For CJT calibration reporting of delay offset/frequency offset/phase offset, the mapping tables should be specified in section 10. FFS on the accuracy requirements.

Proposal 4: If 3t6r is supported in NR_ENDC_RF_Ph4, it is supposed no additional enhancement for MIMO 3-antenna-port codebook-based transmissions, reuse the same requirement in 3t6r SRS antenna switching.

Proposal 5: RAN4 to specify RRM requirements for two TAs in asymmetric DL sTRP/UL mTRP. Rel-18 uplink timing requirements can be used as start point. In Rel-19 asymmetric DL sTRP/UL mTRP, if all TRPs are from the same serving cell (intra-cell), for both two TAGs, they are corresponding to the same downlink reference point. If all TRPs are from the serving cell and additional PCI cell (inter-cell), for both two TAGs, they can be corresponding to the different downlink reference points. For each TAG, uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point.

Observation 1: For FR2, joint TCI state cannot be supported in asymmetric DL sTRP/UL mTRP scenario. For both FR1 and FR2, the UL TCI states can only be associated to SRS.

Observation 2: For UL TCI states switching delay if the UL TCI state is associated to SRS, no RAN4 requirement.

Proposal 6: In asymmetric DL sTRP/UL mTRP scenario:

      - The legacy requirements cannot be applicable.

      - No RRM impacts on UL TCI state switching delay requirement for unified TCI states.

      - Do not define such requirements if the UL TCI state is associated to SRS.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2414035, WF on RRM requirements for NR_MIMO_Ph5, Samsung

[3] R4-2413972, WF on RRM requirements for R19 UE RF enhancements for NR FR1/FR2 and EN-DC, Huawei, HiSilicon