3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415553

Hefei, Anhui, China, 14th – 18rd October, 2024

Agenda item:		6.24.2.1

Source:		Samsung

Title:	Discussion on RRM requirements of enhanced LTM

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of enhanced LTM are updated in [1] as below:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify RRM requirements related to the above objectives as necessary [RAN4]

In last RAN4#112 meeting, we analyzed the overall impacts of RRM requirements related to the above objectives.

For first bullet, it is agreed in WF [2] on NO RRM impact to support inter-CU LTM. The existing R18 LTM RRM requirements can be applied to same procedures in inter-CU scenario.

For second bullet, there are still many open issues. In this contribution, we continue to discuss the RRM requirements for enhanced LTM.

- Discussion

Event triggered L1 measurement reporting

In last RAN4#112 meeting, all proposed items of the scope in listed in WF [2] as below:

Issue 2-2-1: RRM scope of Event triggered L1 measurement reporting - Candidate solutions: - Option 1: RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value. RAN4 can wait for more RAN2 input. (CATT) - Option 2: (Apple) - Requirements of Capabilities for Support of Event Triggering and Reporting Criteria specified in TS38.133 section 9.1.4 need to be updated to cover event triggered L1 measurement reporting. Details are FFS. - Existing framework in L3 event triggered reporting RRM requirements can be reused to cover L1 event triggered reporting. - Option 3: define measurement reporting requirements for event triggered L1 measurement reporting, e.g. the L1 measurement reporting delay. (CMCC, Apple, Samsung, HW, ZTE, Nokia, E///, vivo,CTC) - Option 3a: “Event Triggered Reporting” for LTM L1 measurement is to be introduced in section 9.14.3 and section 9.15.3. - Option 3b: RAN4 can start the discussion based on SSB firstly. Further RAN1/RAN2 progress are needed. (Samsung) - Option 3c: RAN4 to define event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements. (Nokia, E///, vivo, CMCC) - Option 4: RAN4 should discuss the accuracy requirement for event triggered L1 measurement reporting. (ZTE) - Option 5: RAN4 to discuss potential impact of event-triggered L1 reporting to other LTM requirements, for example (Nokia) - Known TCI state condition for early TCI state activation and cell switch - Applicability of cell switch delay requirements - early DL sync (Tfirst-RS = 0 conditions) - early ASN.1 decoding and validity check - LTM L1 measurement requirements - Option 6: For the topic of ‘Measurements related enhancements for the purpose of supporting LTM,’ RAN4 to wait for further progress to be made in other working groups under this WI . (QC, vivo) - Recommended WF - Continue discussion.

There is no agreement for Event triggered L1 measurement reporting. All options above are not contradictory.

Firstly, as mentioned in last meeting, RAN2 designed the measurement enhancement for LTM as:

Agreements on measurement enhancements for LTM:

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

-	Event LTM2: Beam of serving cell becomes worse than absolute threshold;

-	Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

-	Event LTM4: Beam of candidate cell becomes better than absolute threshold;

-	Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Further agreements are achieved in RAN2#127 meeting as:

Agreements on L1 MR event evaluation

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. MAC layer handles the event evaluation and measurement report triggering.

In RAN1#118 meeting, RAN1 agreed:

Agreement Support L1-RSRP measurement based on CSI-RS. FFS: Support L1-SINR measurement based on CSI-RS. Agreement Explicit configuration of CSI-RS resource(s) for candidate cell(s) for L1-measurement is supported. Agreement CSI-RS based L1-RSRP report is supported for gNB scheduled measurement reporting. FFS: CSI-RS based L1-SINR report is supported for gNB scheduled measurement reporting. Rel-18 LTM CSI reporting framework is the baseline for CSI-RS based L1-measurement report by gNB scheduled measurement reporting. Agreement SSB based L1-RSRP measurements is supported for event triggered reporting. CSI-RS based L1-RSRP measurements is supported for event triggered reporting. FFS: CSI-RS based L1-SINR measurements is supported for event triggered reporting.

According to RAN2 agreements, RAN2 defined the event triggered L1 measurement for LTM. Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config.

Proposal 1: RAN4 should discuss and specify event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements.

Proposal 2: For the event-triggered L1 reporting delay requirement, the existing L3 event triggered reporting can be used as reference.

For how to define the delay requirements, it is open in WF [2] as:

Issue 2-2-4: L1 event triggered reporting delay - Candidate solutions: - Option 1: The event triggered L1 measurement reporting delay, measured without L3 filtering, is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. (CMCC) - Option 2: For the delay of event triggered L1 measurement reporting requirements based on SSB, current LTM L1-RSRP measurement delay can be reused. (Samsung) - Option 3: Starting point for the SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap. CSI-RS based delay is pending on Rel-19 work. (Nokia) - Option 4: RAN4 to discuss the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (E///) - Recommended WF - Continue discussion.

In legacy L3 event triggered reporting, the measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface.

Proposal 3: For the event-triggered L1 reporting delay requirement, the measurement reporting delay can also be defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report. The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS.

In Issue 2-2-1, option 4 is proposed to discuss the accuracy requirement for event triggered L1 measurement reporting. When event-triggered L1 measurement is specified, the accuracy requirement is also needed. In Rel-18 LTM, new accuracy requirements are defined for LTM Intra/Inter-frequency L1-RSRP.

Proposal 4: For the event-triggered L1 reporting requirement, RAN4 should discuss and specify accuracy requirements. Rel-18 LTM accuracy requirements can be reused.

Issue 2-2-2: measurement result for event evaluation - Candidate solutions: - Option 1: Use beam level measurement result for event evaluation as baseline and FFS for cell level measurement. (CTC) - Recommended WF - Continue discussion.

According to the agreement in RAN2#127 meeting listed above, “Beam level measurement result, not cell level measurement result, is used LTM event evaluation.”, for Issue 2-2-2, use beam level measurement result for event evaluation, not cell level

Proposal 5: use beam level measurement result for event evaluation, not cell level.

CSI-RS based L1 measurement

Issue 2-3-1: RRM scope of CSI-RS based L1 RSRP measurement on candidate cell(s) - Candidate solutions: - Option 1: (CATT) - All requirements defined based on SSB will be defined for CSI-RS, which includes the following requirements: - PDCCH ordered Random Access for LTM - LTM PCell/ PSCell Cell Switch - Link Recovery Procedures - TRP specific Link Recovery Procedures - TCI state activation for LTM candidate cell - L1-SINR measurements for Reporting - Intra-frequency L1-RSRP measurements for neighbor cell - NR inter-frequency L1 measurement - Option 2: define measurement period requirements for CSI-RS based L1 measurement, and the SSB based L1 measurement period requirements can be used as baseline. (CMCC) - Option 3: For CSI-RS measurement and based beam management for LTM, RAN4 should define new RRM requirements. Need further RAN1 progress. (Samsung) - Option 4: Define early DL sync requirements for using CSI-RS once RAN1 approval, and SSB based DL sync in LTM can be used as baseline. (ZTE) - Option 5: RAN4 can start the requirements discussion of CSI-RS on periodic measurements while RAN1 is discussing about aperiodic and semi-persistent measurement support. (Nokia, vivo) - Option 6: RAN4 to discuss at least the following set of requirements for CSI-RS based LTM measurements: (Nokia) - Measurement delay - Measurement reporting requirements - Measurement restrictions - Scheduling availability - Option 7: For the topic of ‘Measurements related enhancements for the purpose of supporting LTM,’ RAN4 to wait for further progress to be made in other working groups under this WI . (QC) - Agreement: - The following requirements need to be discussed for CSI-RS based L1 RSRP measurement on LTM candidate cell(s) - Measurement delay - Measurement reporting requirements - Measurement restrictions - Scheduling availability - FFS on other potential impact.

In last RAN4#112 meeting, RAN4 agreed to define the CSI-RS based L1 RSRP measurement on LTM candidate cell(s) for such delay requirements etc. The legacy CSI-RS based L1-RSRP and SSB based L1-RSRP measurement for LTM candidate cells can be used as reference.

Issue 2-3-2: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement - Candidate solutions: - Option 1: following CSI-RS based L3 measurement, (Apple) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: Categorize CSI-RS based L1-RSRP measurement into CSI-RS based L1-RSRP measurement within active BWP and outside active BWP for further discussion. (MTK) - Option 3: For CSI-RS based L1 measurement, RAN4 not to introduce definition of intra-frequency/inter-frequency candidate cell measurement. (vivo) - Recommended WF - Continue discussion.

In Rel-18 LTM, intra-frequency L1-RSRP measurements for neighbor cell and inter-frequency L1-RSRP measurement based on SSB are specified. for CSI-RS, the similar measurement should be discussed and specified. Firstly, it should be defined the definition of intra-frequency L1-RSRP and inter-frequency L1-RSRP. We support to reuse the same definition as L3 CSI-RS based measurement.

Proposal 6: For definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, reuse the same definition as L3 CSI-RS based measurement.

Issue 2-3-3: supported measurement types - Candidate solutions: - Option 1: (CATT) - Intra-frequency - Inter-frequency without gap - Inter-frequency with gap - Option 2: (Apple) - Intra-frequency without gap - Inter-frequency with gap - Option 3: deprioritize CSI-RS based L1-RSRP measurement outside active BWP in R19. (MTK) - Option 4: (vivo) - For CSI-RS based L1 measurement on candidate cell with periodic reporting, RRM requirements are specified for the following cases: - CSI-RS based L1 measurement without gap - Recommended WF - Continue discussion.

In current CSI-RS based L3 measurement, intra-frequency measurement is only for without gap while inter-frequency measurement is only for with gap.

Proposal 7: If the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement can be agreed to use L3 CSI-RS based measurement, the measurement type can be:

- Intra-frequency without gap

- Inter-frequency with gap

- Conclusion

In this contribution, we provide our proposals for RRM requirements for enhanced LTM:

Proposal 1: RAN4 should discuss and specify event-triggered L1 reporting delay requirement for SSB based and CSI-RS based measurements.

Proposal 2: For the event-triggered L1 reporting delay requirement, the existing L3 event triggered reporting can be used as reference.

Proposal 3: For the event-triggered L1 reporting delay requirement, the measurement reporting delay can also be defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report. The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS.

Proposal 4: For the event-triggered L1 reporting requirement, RAN4 should discuss and specify accuracy requirements. Rel-18 LTM accuracy requirements can be reused.

Proposal 5: use beam level measurement result for event evaluation, not cell level.

Proposal 6: For definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement, reuse the same definition as L3 CSI-RS based measurement.

Proposal 7: If the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement can be agreed to use L3 CSI-RS based measurement, the measurement type can be:

- Intra-frequency without gap

- Inter-frequency with gap

- Reference

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom

[2] R4-2414060, WF on NR mobility enhancements Phase 4, Apple, China Telecom