3GPP TSG-RAN WG4 Meeting # 112bis		R4-2415554

Hefei, Anhui, China, 14th – 18rd October, 2024

Agenda item:		6.24.2.2

Source:		Samsung

Title:	Discussion on RRM requirements of conditional Intra-CU LTM

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of conditional Intra-CU LTM are updated in [1] including:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This is the first RAN4 meeting after this objective has been approved. In this contribution, we analyzed the potential RRM impacts of conditional Intra-CU LTM.

- Discussion

In RAN#105 meeting, this objective of conditional Intra-CU LTM is confirmed in [1]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

According to the WID, RAN1/RAN2 will specify a new procedure of conditional Intra-CU LTM.

In Rel-18, it cannot support the UE based LTM. In Rel-18 LTM, the cell switch for PCell and/or PSCell are still based on network configured and measurement report. By this objective, it can further enhance the delay by UE determination. Then, it can further improve the reduction in mobility latency.

It is the first meeting for RAN WGs to start the discussion. Since RAN1/RAN2 haven’t started the discussion on this objective, RAN4 can start the discussion from high level.

Proposal 1: RAN4 should specify new RRM requirements including conditional cell switch for conditional Intra-CU LTM.

In the procedure, although RAN2 hasn’t defined the details, the start point might be similar as Rel-18 LTM and similar as the conditional handover.

In Rel-18 LTM, the start point of LTM cell switch delay is from the end of the last TTI containing the MAC-CE command for cell switch. In conditional LTM, the cell switch is UE determination depends on the condition. There is no such MAC-CE command for cell switch.

For NR conditional handover, the start point is when UE receives a RRC message implying conditional handover.

If conditional LTM use the similar conditional handover procedure, UE may receive a command to implying the conditional LTM, UE measured the events of condition, then UE determined the execution of cell switch.

Proposal 2: RAN4 can use Rel-18 LTM requirements and conditional handover requirements as the references to define the new requirements of conditional cell switching for conditional Intra-CU LTM. The start point may be when UE receives the command to config the conditional LTM. For further discussion on each component of the delay, need more RAN2 progress.

- Conclusion

In this contribution, we provide our proposals for RRM impacts of conditional Intra-CU LTM:

Proposal 1: RAN4 should specify new RRM requirements including conditional cell switch for conditional Intra-CU LTM.

Proposal 2: RAN4 can use Rel-18 LTM requirements and conditional handover requirements as the references to define the new requirements of conditional cell switching for conditional Intra-CU LTM. The start point may be when UE receives the command to config the conditional LTM. For further discussion on each component of the delay, need more RAN2 progress.

- Reference

[1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom