3GPP TSG-RAN WG4 Meeting #112bis	R4-2415557

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on MPR reduction for FR2 CA

Source:	Samsung

Agenda item:	6.1.1.2.3

Document for:	Discussion

1.	Introduction

About the MPR enhancement for FR2 CA, the MPR applicability has been discussed and consensus has been achieved over previous meetings. A tiny issue left is about the spec change on the 200MHz granularity. Then the main remaining job is about the UE capability details including release independence aspect.

In this contribution, we further discuss the remaining issues as mentioned above.

2. 	Discussion

2.1 MPR applicability

For the case of single carrier UL with DL intra band CA, in the WF [1, R4-2414441] of RAN4#112 meeting, FFS is expected.

Issue 3-2-1: Applicable MPR for FR2 single carrier UL with DL intra band CA - Proposals - Proposal 1: For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier. (Huawei) - WF - FFS in next meeting.

However, in RAN4#111 meeting, agreement [2, R4-2410749] was already achieved and it is not necessary to FFS.

2.3. Applicable MPR for FR2 single carrier UL with DL intra band CA Way forward: - For the case of single carrier UL with DL intra band CA, the MPR requirements of single carrier case in clause 6.2.2 of TS 38.101-2 applies with UE indication of new capability for MPR improvement.

Proposal 1:	For the case of single carrier UL with DL intra band CA, stick to the agreement achieved in R4-2410749.

Another remaining issue about FR2 MPR applicability on how to deal with the 200MHz BW granularity [1]:

Issue 3-2-2: Applicable MPR for FR2 UL CA with DL intra band CA - WF - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2

The existing MPR requirement for 200MHz BW granularity is as following:

In case of a contiguous RB, DFT-s-BPSK or DFT-s-QPSK UL allocation in a single CC of a CA configuration with contiguous CCs, and whose cumulative aggregated BW 400 MHz, MPRC_CA shall be derived instead as MAX(MPR1, MPR2), where: MPR1 shall be determined from Table 6.2.2.3-1 if CABW 200 MHz, from Table 6.2.2.3-2 if CABW > 200 MHz. MPR2 shall be determined from Table 6.2.2.3-1 if UL BWchannel_CA 200 MHz, from Table 6.2.2.3-2 if UL BWchannel_CA > 200 MHz.

As proposed in our contribution to last meeting [3, 2411693], only a small change is needed, i.e. change the MPR calculation from MPRC_CA = MAX(MPR1, MPR2) to MPRC_CA = MPR2, for UE with UE indication of new capability for MPR improvement.

Proposal 2:	For 200MHz BW granularity, RAN4 to change the MPR calculation for DFT-s-BPSK or DFT-s-QPSK from “MPRC_CA = MAX(MPR1, MPR2)” to “MPRC_CA = MPR2”, for UE with UE indication of new capability for MPR improvement.

2.2 UE capability and other issues

In last meeting, details of UE capability were extensively discussed. In our understanding, an optional UE capability is needed to distinguish normal Rel-19 UE and FR2 MPR enhanced Rel-19 UE. Given the assumption is that the MPR enhanced UE is equipped with separate LO for RX and TX respectively, this capability can be a per UE capability.

On the other hand, though the enhanced MPR is derived based on separate LO assumption, it does not mean UE has to adopt such specific implementation. As RAN4 usual practice, the new UE capability should be implementation agnostic.

Proposal 3:	An optional new UE capability is needed, which can be per UE for FR2 and should be implementation agnostic.

Given hardware enhancement is expected to support this enhancement, it would be reasonable to specify this new capability without release independence.

Proposal 4:	The new UE capability of supporting FR2 MPR enhancement is not release independent.

3. 	Conclusion

Proposal 1:	For the case of single carrier UL with DL intra band CA, stick to the agreement achieved in R4-2410749.

Proposal 2:	For 200MHz BW granularity, RAN4 to change the MPR calculation for DFT-s-BPSK or DFT-s-QPSK from “MPRC_CA = MAX(MPR1, MPR2)” to “MPRC_CA = MPR2”, for UE with UE indication of new capability for MPR improvement.

Proposal 3:	An optional new UE capability is needed, which can be per UE for FR2 and should be implementation agnostic.

Proposal 4:	The new UE capability of supporting FR2 MPR enhancement is not release independent.

4. 	References

- R4-2414441     “WF on power domain enhancements”, Huawei, HiSilicon

- R4-2410749     “WF on power domain enhancements”, Huawei, HiSilicon

- R4-2411693     “Discussion on MPR reduction for FR2 CA”, Samsung