3GPP TSG-RAN WG4 Meeting #112bis	R4-2415558

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	Discussion on UE RF requirements for LP-WUR

Source:	Samsung

Agenda item:	6.23.2.2

Document for:	Discussion

1.	Introduction

In RAN4#111 meeting, there was much progress on LP-WUS UE RF performance metric and baseline architecture. The main remaining issue for REFSENS derivation is the target SNR. As discussed in last meeting, it is urgent to conclude the target SNR first [1, R4-2414309]:

Issue 2-1-3: Whether RAN4 should conclude a target SNR first Agreement: - Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first. Issue 2-1-4: Target SNR value Agreements: - RAN4 further discuss target SNR value for LP-WUS - FFS single SNR for OOK-based and OFDM-based receiver

Meanwhile, there is incoming LS [2, R4-2414909_R1-2407559] from RAN1 about different bands supporting between LR and MR. It seems that more hardware architecture related discussions are needed.

In this contribution, we will focus on REFSENS discussion especially about target SNR, and some architecture related discussion is also involved.

2. 	Discussion

SNR is a very general term and the value can be different at different reference point in the receiver chain. In the legacy equation for REFSENS derivation

REFSENS=-174 dBm+10*logBW+NF+SNR+IM,

The reference point of SNR is at UE baseband. To avoid ambiguity, it is necessary to make clear that the reference point for target SNR in LP-WUR REFSENS discussion in at the UE baseband.

Proposal 1:	In the context of LP-WUR REFSENS derivation, the reference point of target SNR is at UE baseband.

In RAN1#117, the relationship between SNR and NF was agreed based on link budget analysis assuming the msg3 coverage target, reproduced as following:

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

It can be observed that target SNR would be low if assuming LR with big NF and target SNR would be high if assuming LR with small NF. Now there are both OOK-based and OFDM-based receivers in scope. If different target SNR applies for OOK-based and OFDM-based receivers respectively, then the REFSENS requirements for OOK-based and OFDM-based receivers are almost the same which are not aligned with its actual performance. OFDM based receiver consumes more power but no stringent sensitivity requirement. In this sense, the same target SNR value should be applied to both OOK-based and OFDM-based receivers, and the target SNR value should be based on OOK-based receiver as minimum requirement.

Proposal 2:	the target SNR value should be determined based on OOK-based receiver and the same target SNR value applies to both OOK-based and OFDM-based receivers.

There is incoming LS [2] from RAN1 about different bands supporting between LR and MR:

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

It is true that UE may not support LP-WUS reception on all the bands supported by the UE. The follow up question would be if LP-WUR in band A can be used as LP-WUS function for MR in band B. Now all RAN4 assumption is that the LP-WUR band and the MR band is the same, even in the same component carrier, within the required BW. In theory LP-WUR in band A can wake up MR in band B, but the problem is that it is quite challenging for LR in band A can do RRM for MR in band B, because the channel status of band A and band B is probably quite different.

Observation 1:	LR in band A could wake up MR in band B, but can not do RRM job well for MR in band B due to irrelevant channel condition for different bands.

Proposal 3:	In Rel-19 RAN4 not to consider the LP-WUS scenario when LR and MR co-work in different bands.

Following the same principle, the LP-WUS scenario when LR and MR have separated antennas is also quite challenging. Because in practical UE separate antennas are usually designed irrelevant which is the principle of MIMO theory. With irrelevant antennas for LR and MR, the LR can not do RRM job well for MR, because the LR antenna’s measurement results can not be calibrated with a constant offset from MR antenna’s measurement results due to the irrelevancy between them. So it is indeed necessary to assume antenna sharing between LR and MR.

Observation 2:	LR with antenna A could wake up MR with antenna B, but can not do RRM job well for MR with antenna B due to irrelevant channel condition for different antennas.

Proposal 4:	In Rel-19 antenna sharing architecture is assumed between LR and MR.

Another impact of antenna sharing between LR and MR is the IM value for REFSENS. Due to antenna sharing between LR and MR, the antenna switch component and long path from antenna to receiver will lead to larger insertion loss compared with MR. Given the IM for MR in NR legacy REFSENS is 2.5dB, it is reasonable that the IM for LR REFSENS should be no smaller than 3dB.

Observation 3:	antenna sharing between LR and MR lead to larger insertion loss than legacy NR.

Proposal 5:	IM value for LR REFSENS is proposed to be no smaller than 3dB.

3. 	Conclusion

Observation 1:	LR in band A could wake up MR in band B, but can not do RRM job well for MR in band B due to irrelevant channel condition for different bands.

Observation 2:	LR with antenna A could wake up MR with antenna B, but can not do RRM job well for MR with antenna B due to irrelevant channel condition for different antennas.

Observation 3:	antenna sharing between LR and MR lead to larger insertion loss than legacy NR.

Proposal 1:	In the context of LP-WUR REFSENS derivation, the reference point of target SNR is at UE baseband.

Proposal 2:	the target SNR value should be determined based on OOK-based receiver and the same target SNR value applies to both OOK-based and OFDM-based receivers.

Proposal 3:	In Rel-19 RAN4 not to consider the LP-WUS scenario when LR and MR co-work in different bands.

Proposal 4:	In Rel-19 antenna sharing architecture is assumed between LR and MR.

Proposal 5:	IM value for LR REFSENS is proposed to be no smaller than 3dB.

4. 	References

- R4-2414309    “WF on LP-WUS UE RF requirements”, vivo

- R4-2414909_R1-2407559    “LS on LP-WUS operation in IDLE/INACTIVE mode”, RAN1