3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415567

Hefei, China, 14th – 18th Oct. 2024

Source: 	Nokia

Title: 					n28 NS_17 and NS_18 A-MPR proposals

Agenda Item: 			5.20.2

Document for:	Approval

# 1	Introduction

We make proposals for NS_17 and NS_18 A-MPR based on simulations on band n28. The simulation results are published in a separate contribution [1].

# 2	A-MPR proposals

The simulation results are presented in a separate contribution [1]. In this section we provide the A-MPR proposals based on these results. In appendix we show the A-MPR triangles from the proposals for PC2 NS_17, PC2 NS_18 40 MHz CBW and PC3 NS_18 40 MHz CBW.

## 2.1	PC2 NS_17

For PC2 NS_17 we employ an additional constraint that the proposed A-MPR values should not lead to lower output power than at PC3 without A-MPR.

Proposal 1: RAN4 shall consider defining A-MPR for n28 PC2 due to NS_17 as shown in tables 1 and 2.

Table 1: The proposed A-MPR regions for n28 PC2 NS_17.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 728.18 | ≤ 3.1 | ≥ 0.6*12*SCS*RBstart + 5 | A1 |
|  |  | ≤ 0.4 | ≤ -2*12*SCS*RBstart + 1.2 | A2 |

Table 2: The proposed A-MPR values for n28 PC2 NS_17.

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 3 | 2 |
|  | QPSK | 4 | 3 |
|  | 16 QAM | 4.5 | 3 |
|  | 64 QAM | 4.5 | 3 |
|  | 256 QAM |  |  |
| CP-OFDM | QPSK | 5.5 | 4.5 |
|  | 16 QAM | 5.5 | 5 |
|  | 64 QAM | 5.5 | 5 |
|  | 256 QAM |  |  |

## 2.2	PC2 NS_18

For PC2 NS_18 we use the A-MPR-regions from [2] for CBW ≤ 30 MHz, but the A-MPR values are modified according to our simulations results. Additionally, we provide A-MPR proposal for 40 MHz CBW.

Proposal 2: RAN4 shall consider defining A-MPR for n28 PC2 due to NS_18 as shown in tables 3 and 4.

Table 3: A-MPR regions for NS_18 and PC2

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥ 5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥ 5.4 | A6 |
| 40 | 703~743 | ≥ 0 | > 6.0, ≥ 0.9*12*SCS*RBstart – 3 | A4 |
|  |  | ≥ 0 | ≤ 6.0, ≤ -4*12*SCS*RBstart + 45 | A5 |

Table 4: A-MPR values for NS_18 and PC2

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | N/A | 8 | 3.5 | 8 | 2.5 | 2 |
|  | QPSK | 2.5 |  | 8.5 | 3.5 | 8.5 | 3.5 | 2 |
|  | 16 QAM | 3 |  | 8.5 | 3.5 | 9 | 4 | 2 |
|  | 64 QAM | 3.5 |  | 9 | 3.5 | 9 | 4 | 2.5 |
|  | 256 QAM |  |  | 9 | 3.5 | 9 |  |  |
| CP-OFDM | QPSK | 4 |  | 9 | 5 | 9.5 | 5 | 4 |
|  | 16 QAM | 4 |  | 9 | 5 | 9.5 | 5 | 4 |
|  | 64 QAM | 4 |  | 9 | 5 | 9.5 | 5 | 4 |
|  | 256 QAM |  |  | 9 | 5 | 9.5 |  |  |

## 2.3	PC3 NS_18

The A-MPR has already been specified for PC3 NS_18 for CBW ≤ 30 MHz. Here we provide A-MPR tables modified to include our proposal for 40 MHz CBW.

Proposal 3: RAN4 shall consider defining A-MPR for n28 PC3 due to NS_18 as shown in tables 5 and 6.

Table 5: A-MPR regions for PC3 NS_18. The new entries are marked with yellow color.

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
| 40 | 703~743 | ≥ 0 | > 6.0, ≥ 0.9*12*SCS*RBstart – 3 | A6 |
|  |  | ≥ 0 | ≤ 6.0, ≤ -4*12*SCS*RBstart + 45 | A7 |

Table 6: A-MPR values for PC3 NS_18. The new entries are marked with yellow color.

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 | 4.5 | 2 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 | 5.5 | 2.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 | 6 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 | 6 | 3 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 | 6 |  |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 | 7 | 4 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 | 7 | 4 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 | 7 | 4 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 | 7 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |

# 3	Conclusions

We have performed simulations to determine the A-MPR for n28 for NS_17 and NS_18. Based on our simulations we have made the following proposals.

Proposal 1: RAN4 shall consider defining A-MPR for n28 PC2 due to NS_17 as shown in tables 1 and 2.

Proposal 2: RAN4 shall consider defining A-MPR for n28 PC2 due to NS_18 as shown in tables 3 and 4.

Proposal 3: RAN4 shall consider defining A-MPR for n28 PC3 due to NS_18 as shown in tables 5 and 6.

Table 1: The proposed A-MPR regions for n28 PC2 NS_17.

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | Fc < 728.18 | ≤ 3.1 | ≥ 0.6*12*SCS*RBstart + 5 | A1 |
|  |  | ≤ 0.4 | ≤ -2*12*SCS*RBstart + 1.2 | A2 |

Table 2: The proposed A-MPR values for n28 PC2 NS_17.

| Modulation/Waveform |  | A1 | A2 |
| --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 3 | 2 |
|  | QPSK | 4 | 3 |
|  | 16 QAM | 4.5 | 3 |
|  | 64 QAM | 4.5 | 3 |
|  | 256 QAM |  |  |
| CP-OFDM | QPSK | 5.5 | 4.5 |
|  | 16 QAM | 5.5 | 5 |
|  | 64 QAM | 5.5 | 5 |
|  | 256 QAM |  |  |

Table 3: A-MPR regions for NS_18 and PC2

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+3.6 ≤(LCRB*12*SCS)/2+5.76 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥ 5.4 | A6 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
|  |  | >(LCRB*12*SCS)/2+5.22 ≤(LCRB*12*SCS)/2+7.38 | <Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) ≥ 5.4 | A6 |
| 40 | 703~743 | ≥ 0 | > 6.0, ≥ 0.9*12*SCS*RBstart – 3 | A4 |
|  |  | ≥ 0 | ≤ 6.0, ≤ -4*12*SCS*RBstart + 45 | A5 |

Table 4: A-MPR values for NS_18 and PC2

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | 1.5 | N/A | 8 | 3.5 | 8 | 2.5 | 2 |
|  | QPSK | 2.5 |  | 8.5 | 3.5 | 8.5 | 3.5 | 2 |
|  | 16 QAM | 3 |  | 8.5 | 3.5 | 9 | 4 | 2 |
|  | 64 QAM | 3.5 |  | 9 | 3.5 | 9 | 4 | 2.5 |
|  | 256 QAM |  |  | 9 | 3.5 | 9 |  |  |
| CP-OFDM | QPSK | 4 |  | 9 | 5 | 9.5 | 5 | 4 |
|  | 16 QAM | 4 |  | 9 | 5 | 9.5 | 5 | 4 |
|  | 64 QAM | 4 |  | 9 | 5 | 9.5 | 5 | 4 |
|  | 256 QAM |  |  | 9 | 5 | 9.5 |  |  |

Table 5: A-MPR regions for PC3 NS_18. The new entries are marked with yellow color.

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 25 | 703~733 | >(LCRB*12*SCS)/2+3.6 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+3.6 | ≥5.4 | A4 |
|  |  | ≤6.3 | <5.4 | A5 |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |
|  |  | ≤7.92 | <5.4 | A5 |
| 40 | 703~743 | ≥ 0 | > 6.0, ≥ 0.9*12*SCS*RBstart – 3 | A6 |
|  |  | ≥ 0 | ≤ 6.0, ≤ -4*12*SCS*RBstart + 45 | A7 |

Table 6: A-MPR values for PC3 NS_18. The new entries are marked with yellow color.

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) | A3 (dB) | A4 (dB) | A5 (dB) | A6 (dB) | A7 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Inner/Outer | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 | N/A | ≤ 5 | 3 | 8 | 3 | 4.5 | 2 |
|  | QPSK | ≤ 2 |  | ≤ 5 | 3 | 8 | 3 | 5.5 | 2.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 6 | 3 | 8 | 3 | 6 | 3 |
|  | 64 QAM | ≤ 4 |  | ≤ 7 | 3 | 8 | 4.5 | 6 | 3 |
|  | 256 QAM | ≤ 6 |  | ≤ 9 | 3 | 8 | 5.5 | 6 |  |
| CP-OFDM | QPSK | ≤ 5 |  | ≤ 6.5 | 4.5 | 9.5 | 5 | 7 | 4 |
|  | 16 QAM | ≤ 5 |  | ≤ 7 | 4.5 | 9.5 | 5 | 7 | 4 |
|  | 64 QAM | ≤ 5.5 |  | ≤ 8.5 | 4.5 | 9.5 | 5.5 | 7 | 4 |
|  | 256 QAM | ≤ 8.5 |  | ≤ 11.5 | 4.5 | 9.5 | 7.5 | 7 |  |
| NOTE 1: Void NOTE 2: Void |  |  |  |  |  |  |  |  |  |

# 4	References

[1] R4-2415568, n28 NS_17 and NS_18 A-MPR simulations, Nokia

[2] R4-2310245, WF on PC2 A-MPR for band n8 and n28, Huawei, HiSilicon, Apple, China Unicom

# 5	Appendix

Here we provide the A-MPR triangles from the proposals for PC2 NS_17, PC2 NS_18 40 MHz CBW and PC3 NS_18 40 MHz CBW.

## 5.1	PC2 NS_17

## 5.2	PC2 NS_18

## 5.3	PC3 NS_18