3GPP TSG RAN WG4 #112bis	R4-2415573

Hefei, China, October 14 – 18, 2024

Agenda item:		6.20.3

Source:	Nokia

Title:	RRM requirements for evolution of NR duplex operation

Document for:		Discussion

# Introduction

At RAN4#112 meeting, the general scope of the RRM requirements for duplex evolution was identified in the WF [1]. In this contribution, we are discussing the open issues and providing our initial views on the RRM impacts due to SBFD operation and co-channel CLI handling mechanisms based on the progress in other WGs.

# RRM requirements for co-channel CLI handling

The following were agreed in last RAN4 meeting on the RRM requirements for CLI handling. We will discuss the details on the UE-to-UE CLI measurement requirements in this section.

Issue 2-1-1: Scope of requirements for UE-to-UE CLI handling Agreements (from online session) - RAN4 to define RRM requirements for L1 based UE-to-UE CLI measurement and reporting. Issue 2-1-7: Side condition Agreements (from online session) - RAN4 to define side conditions for L1 based UE-to-UE CLI measurement requirements. - RAN4 to discuss at least time offset between DL timing and SRS arrival timing, SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP. - The RAN1 progress will be taken into account when discussing the values for the side conditions. Issue 2-1-8: Measurement period Agreements (from online session) - RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements, - RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design. - Number of shots for simulation: 1, 2, 3. - Companies to bring proposals on other simulation parameters in the next meeting.

CLI measurement scenarios

In R19, the CLI measurements are mostly discussed in SBFD, i.e., the victim UE is aware of the SBFD configuration. During RAN1#116, four methods were identified for how to measure the co-channel UE-to-UE CLI in SBFD as depicted in Figure 1 and the following was agreed in RAN1#118 meeting:

Agreement - For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported​ - Method#1: UE measures RSSI within DL subband​ - Method#2: UE measures RSRP of aggressor UE within UL subband​ - FFS: Method#3: UE measures RSSI within UL subband

Figure 1. Methods for UE-to-UE CLI measurements in SBFD

If SRS-RSRP is configured for CLI measurement, UE needs to measure the RSRP of aggressor UE within UL subband, i.e., Method #2. This SRS-RSRP CLI measurement indicates the interference on the UE’s “UL transmission” due to the UL transmission from aggressor UE which is different from the R16 legacy CLI measurement where the interference is on the DL reception of the victim UE.

Observation #1: The SRS-RSRP CLI measurement in R19 indicates the interference on the UE’s “UL transmission” due to the UL transmission from aggressor UE.

For CLI-RSSI measurement, the main difference between Method#1 and Method#3 is whether the UE measures the CLI over the DL subband(s) or over the UL subband of the victim UE where SBFD is configured. In other words, the UE measures the leakage interference in Method#1 and the fundamental UL signal in Method#3. Our main concern with Method#1 is the fact that the CLI-RSSI measurement likely include legacy inter-cell interference making it difficult for the network to conclude on whether the reported CLI-RSSI value corresponds to high UE-to-UE CLI or high legacy interference. On the other hand, Method#3 ensures that the measurements capture only the UL signals being transmitted over the UL subband.

Observation #2: Method#1 is not able to distinguish whether the CLI-RSSI measurement corresponds to UE-to-UE CLI or legacy inter-cell interference.

Proposal 1: To define the CLI measurement requirements in SBFD as the following:

- SRS-RSRP measurement requirements for Method#2.

- CLI-RSSI measurement requirements for Method#1 and Method#3.

Measurement quantities

Based on the latest RAN1 discussion on R19 duplex evolution, it was agreed to define L1-SRS-RSRP and L1-CLI-RSSI for CLI measurements. These reporting quantities are different from legacy L3 based CLI measurement hence existing CLI measurement requirements do not apply. RAN4 should specify the measurement requirements at least for the agreed L1-SRS-RSRP and L1-CLI-RSSI measurements. If some additional measurement quantities will be agreed in RAN1 in the future, those can be further discussed in RAN4 but these should not prevent the discussion on the agreed quantities.

Observation #3: In R19 duplex discussion, new reporting quantities L1-SRS-RSRP and L1-CLI-RSSI are agreed for CLI measurements hence legacy L3 based CLI measurement requirements do not apply.

Proposal 2: RAN4 should specify the measurement requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements in R19.

Rx beam assumption

Current UE-to-UE CLI measurements for FR2 UEs assume that the Rx beam used for the measurements is the same as the latest beam used for the reception of the PDCCH/PDSCH (as cited below). There is an on-going discussion on RAN1 about allowing the gNB to provide an indication of the QCL-type D assumptions as part of the L1 UE-to-UE CLI measurement configuration. Therefore, the Rel-16 assumptions for Rx beam used for CLI measurement may not be applicable. It is expected that the CSI measurement resource configuration is used to indicate the TCI state of the CLI measurement resources. In principle, it is applicable to CLI-RSSI and SRS-RSRP measurement resources. Given the differences with existing Rel-16 assumptions on Rx beam in FR2, RAN4 should discuss the CLI measurement requirements considering the potential different Rx beams based on RAN1 conclusion.

TS 38.133 clause 9.7.1: For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

Observation #4: The Rel-16 assumptions for Rx beam used for CLI measurement in FR2 may not be applicable provided the latest RAN1 discussion.

Proposal 3: RAN4 to discuss the CLI measurement requirement considering the potential different Rx beams based on RAN1 conclusion.

CLI measurement periods

In R16, the CLI measurement periods for SRS-RSRP and CLI-RSSI are defined in the way of L3 measurements for dynamic TDD. For instance, 3 samples are assumed for deriving the SRS-RSRP measurement period and 1 sample RSSI is assumed for the CLI-RSSI measurement period.

Observation #5: In R16, 3 samples are assumed for deriving the SRS-RSRP measurement period and 1 sample RSSI is assumed for the CLI-RSSI measurement period.

In last RAN4 meeting, it was agreed to define the measurement periods for L1 based UE-to-UE CLI measurement but no conclusion on the number of samples. As the WID has indicated L1 based UE-to-UE CLI measurement and reporting is based on existing CSI framework, the SRS-RSRP CLI measurement requirement can be specified following L1-RSRP measurement. According to TS 38.133 clause 9.5, L1-RSRP measurement period is defined based on the high layer parameter timeRestrictionForChannelMeasurement and both 1 and 3 samples are considered. We think the same assumption can be applied to L1-SRS-RSRP measurement period in R19 up to network configuration. And for L1-CLI-RSSI measurement, the CLI measurement period is specified assuming 1 sample the same as the legacy CLI-RSSI measurement requirement.

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

Proposal 4: For L1-RSRP-RSRP measurement, the CLI measurement period can be specified following L1-RSRP measurement and both 1 and 3 samples are considered based on network configuration.

Proposal 5: For L1-CLI-RSSI measurement, the CLI measurement period is defined assuming 1 sample.

Side conditions

Another aspect on the measurement requirement is the side conditions. Similar as for R16 CLI measurement, since the UE performs L1-SRS-RSRP measurement based on the DL timing in victim cell, there is time difference between the DL timing and the SRS arrival timing from the aggressor UE. This will impact the accuracy performance of CLI measurement.

#### Issue 2-1-7: Side condition - Proposals - Proposal 1 (CATT, CTC, E///, vivo, SS, HW, ZTE, QC): - RAN4 to discuss side conditions for L1 based UE-to-UE CLI measurement requirements. - Proposal 2 (SS): - For Rel-19 L1 CLI SRS-RSRP measurement, the time difference between UE’s DL reference timing in the serving cell and SRS arrival time shall be further discussed, e.g. the residual timing error can be - For intra-cell L1 CLI SRS-RSRP measurement: Tother = 1.67usec for FR1 and 0.67usec for FR2 can be considered (i.e. without 3us cell phase error). - For inter-cell L1 CLI SRS-RSRP measurement: Reduced values compared R16 CLI assumption can be considered. - Proposal 3 (SS): - Reuse R16 L3 CLI SRS-RSRP measurement assumption on SRS configuration and SINR condition as starting point - Proposal 4 (ZTE): - The side condition for Method#2 should be considered from the following aspects: - Side condition of time offset between UE’s DL reference timing in the serving cell and SRS arrival time - Side condition of SINR - Side condition of maximum/minimum RSRP - Recommended WF - RAN4 to define side conditions for L1 based UE-to-UE CLI measurement requirements. - RAN4 to discuss at least time offset between DL timing and SRS arrival timing, SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP.

In R16 CLI measurement discussion, the time offset between DL timing and SRS arrival timing was derived as below. It was assumed UE would apply a constant offset to the DL reference timing in the serving cell to compensate the UL timing difference. And the residual timing error Tother is defined as a side condition for the CLI measurement accuracy requirement. We believe the same methodology can be adopted in R19 to define the time offset side condition for L1 CLI measure accuracy. Hence, we propose for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

Observation #6: In R16 CLI measurement, UE would apply a constant offset to the DL reference timing in the serving cell, and the residual timing error Tother is defined as a side condition for the CLI measurement accuracy requirement.

Proposal 6: For L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

R16 CLI measuerment discussion: Definition for DL timing error (TCLI) in R16 CLI: - Consider cell phase synchronization error for TCLI derivation and for residual timing error assumptions for deriving measurement accuracy - TCLI = Tc*NTA_offset + Tother - Tother = Tpropagation_delay + Tcell_phase_synchronization - Tother = 4.67usec for FR1 and 3.67usec for FR2 - under the assumption that ISD=500m for FR1 and ISD=200m for FR2 - Residual timing error for measurement accuracy - 4.67usec for FR1 and 3.67usec for FR2 TS 38.133 clause 9.7: When the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

Regarding to the residual timing error Tother, it has been determined by the propagation delay difference and the cell phase error between the cells for victim and aggressor UEs. While the same rule can be applied to R19 discussion, the SRS-RSRP measurement is being discussed on the UL subband with SBFD operation where the aggressor UE mainly comes from the same cell of victim UE. We believe this is the most likely scenario to configure the L1-SRS-RSRP measurement hence the side condition can be defined assuming intra-cell i.e. aggressor and victim UEs are in the same cell. The main reason to focus the SRS-RSRP measurements applicability on the intra-cell scenario is the lack of exchange of SRS information across gNBs. As per agreement on RAN1#117, “The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs.”, and thus, it is not possible for a cell to configure the SRS-RSRP resources that match the SRS configuration of an aggressor cell in the neighbour cell.

Proposal 7: To define the residual timing error Tother in intra-cell scenario as one side condition for the L1-SRSR-RSRP measurement accuracy requirement.

If the intra-cell scenario is agreeable as the starting point, the cell phase error can be zero as victim and aggressor UEs are assumed in the same cell. As the propagation delay difference is derived assuming the victim and aggressor UEs are located in the cell center and cell edge respectively, it remains the same irrespective of intra-cell or inter-cell. Hence the residual timing error can be derived assuming cell phase error is zero and same cell size as in R16.

Proposal 8: For L1-SRS-RSRP measurement, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

As for SRS configuration, we don’t see the need to change SRS configuration by introducing L1 CLI measurements with SBFD operation hence the same SRS configuration as in R16 CLI measurement can be assumed. But the SRS Es/Iot value may be different considering the R19 SRS-RSRP intends to measure the interference on UL subband where there is no other DL transmission hence less interference in the victim UE. RAN4 should discuss the value of SRS Es/Iot on the UL subband in SBFD operation.

Proposal 9: For L1-SRS-RSRP measurement, RAN4 to adopt the same SRS configuration, and to discuss the value of SRS Es/Iot on the UL subband in SBFD operation.

CLI measurement reporting

With the updated WID, it informs the L1 based CLI measurement and reporting are based on existing CSI framework and at least aperiodic reporting is supported. This means the CLI measurement reporting will be handled similar as L1-RSRP reporting, hence at least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

Proposal 10: At least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

CLI subband for CLI-RSSI measurements

Due to the frequency dependency of the CLI, RAN1 is currently discussing whether finer reporting granularity rather than wideband reporting should be supported. This approach allows that multiple CLI-RSSI values are reported for a single CLI-RSSI measurement resource. This is similar to the already existing subband reporting supported in CSI report for CQI and PMI. However, now the interest is on L1-CLI-RSSI measurements. Moreover, the subband sizes for CQI and PMI derivation might be too larger for the CLI-RSSI measurements and, therefore, new subband sizes might need to be introduced. If this feature is supported by RAN1, RAN4 should define new requirements for the sub-band CLI-RSSI reporting accordingly.

Proposal 11: RAN4 should define new requirements for the sub-band CLI-RSSI reporting if this feature is supported by RAN1.

Measurement/scheduling restriction

As discussed in CLI measurement scenarios, different CLI measurement methods are being discussed in RAN1 based on if the CLI measurements are configured on DL subbands or UL subband. In particular, if the UE is configured with CLI measurements on UL subband i.e Method#2 and Method#3, no further DL reference signals are expected to be measured other than the CLI measurement resources hence there will not be measurement restriction. Otherwise, the measurement restriction shall be defined.

Similarly, when the UE is performing CLI measurements on serving cell, the scheduling availability also depends on if the CLI measurements are configured on DL subbands or UL subband i.e. the respective CLI measurement methods. RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

Proposal 12: RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

# RRM requirements for SBFD operation

SBFD is being specified in Rel-19 as a new mode of operation in unpaired spectrum where separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. RRM requirements for SBFD operation were discussed for the first time in RAN4#112, where the following agreements were reached:

#### Issue 2-3-1: Requirements for legacy UE Agreements (from online session) - The legacy UEs shall perform and operate according to the legacy RRM requirements and no new RAN4 RRM requirement for legacy UEs, despite being served by a gNB operating with SBFD. - Clarification on the applicability of the existing RRM requirements in RAN4 spec is not precluded. - The legacy UEs refer to non-SBFD aware UE. #### Issue 2-3-2: Requirements for SSB based measurement Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell.

There were multiple open issues captured in the Way Forward (WF) which we address in the subsections below.

## Requirements for CSI-RS based measurement

The following way forward was agreed in RAN4#112 regarding this topic:

#### Issue 2-3-3: Requirements for CSI-RS based measurement Recommended WF - RAN4 to discuss the impact of SBFD operation on the CSI-RS measurement requirements.

RAN1 has reached quite a few agreements regarding CSI-RS transmission on SBFD symbols which are listed in the following:

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Essentially, this means that CSI-RS signals are configured according to the legacy way (start and end RB, density, etc.), but only the CSI-RS resources in the DL subband are measured by the UE and used to derive the report. The CSI-RS resources in the UL subband and guardbands are assumed not to be present, i.e. “punctured”. This principle is illustrated in Figure 1, where CSI-RS signals are transmitted in both non-SBFD and SBFD symbols, but the latter one is non-contiguous in frequency due to the puncturing operation.

As a first step in RAN4, it should be agreed that the measurement of CSI-RS signals according to RAN4 specifications should be aligned with the RAN1 agreements: i.e. SBFD-aware UE measures only CSI-RS frequency resources within DL usable PRBs.

Observation #7: According to RAN1 agreement, only CSI-RS resources within DL subbands are measured by the UE and used to derive the report.

Proposal 13: For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

Figure 1: Puncturing of CSI-RS due to SBFD DUD operation

RAN4 TS 38.133 specify the measurement delay requirements related to CSI-RS based measurements and reporting. In particular, the measurement delay defineshow long time it should take for the UE to derive a qualified CSI-RS based measurement result and is counted by a number of measurement samples. This includes CSI-RS based L3 measurements for mobility, CSI-RS based L1-RSRP measurement, and CSI-RS based RLM/BFD measurement etc.

When SBFD operation is considered, it may happen a periodic CSI-RS signal takes place in both SBFD and non-SBFD symbol types. For the CSI-RS resource in SBFD symbols, if it is overlapping with UL subband, UE is expected to measure a less number of PRBs due to puncturing behaviour hence the number of PRBs to be measured for the CSI-RS resource may be different on SBFD and non-SBFD symbols. RAN4 shall discuss if the measurement samples over SBFD and non-SBFD symbols can be combined when performing CSI-RS based measurements.

Observation #8: The number of PRBs measured for the CSI-RS resource may be different on SBFD and non-SBFD symbols.

Proposal 14: RAN4 shall discuss if the measurement samples over SBFD and non-SBFD symbols can be combined when performing CSI-RS based measurements.

On the other hand, there are measurement accuracy requirements specifying the absolute and/or relative accuracy that CSI-RS based measurement reporting shall meet. In TS38.133 clause 10, “the bandwidth of CSI-RS is 48 PRBs and the density is 3” has been assumed.With respect to these RRM requirements, the following issues are foreseen due to SBFD operation:

- For a periodic CSI-RS signal that takes place in both SBFD and non-SBFD symbol types, the number of CSI-RS measurement samples collected per CSI-RS instance may be different for each measurement instance .

- The “residual” amount of CSI-RS PRBs on SBFD symbols after puncturing operation may be smaller than 48 PRBs, which is a prerequisite for fulfilling many of the RAN4 RRM requirements.

- Even if the “residual” amount of CSI-RS PRBs is larger or equal than 48 PRBs, the CSI-RS resources may be split across two disjoint DL subbands (i.e. the CSI-RS is not contiguous in frequency).

In our view, if the CSI-RS resource is within one of the DL subbands or there are at least 48 consecutive CSI-RS PRBs in one of the DL subbands, RAN4 should agree that the existing requirements shall apply. Note that this is a reasonable scenario since SBFD is primarily suitable for large carrier bandwidths (e.g. 50-100 MHz in FR1).

Observation #9: The “residual” amount of CSI-RS PRBs on SBFD symbols after puncturing operation may be smaller than 48 PRBs, which is a prerequisite for fulfilling many of the RAN4 RRM requirements.

Proposal 15: For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (i.e. within one of the DL subbands) is no less than 48 PRBs.

Now, if the CSI-RS PRBs are split across two DL subbands, RAN4 should discuss whether or not existing measurement period and accuracy requirements can still be applied. If the residual CSI-RS RB bandwidth is below 48 PRBs, we think existing requirements should be either modified (relaxed) or not be applicable at all. If the residual CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, we think the existing requirements can be applied but we are also open to hear if additional side conditions need to be defined.

Proposal 16: For CSI-RS on SBFD symbols, existing accuracy requirements do not apply if the residual CSI-RS bandwidth (number of PRBs) after applying the puncturing operation is below 48 PRBs.

Proposal 17: For CSI-RS on SBFD symbols, if the residual CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, RAN4 to discuss if existing accuracy requirements for CSI-RS based measurements can apply and whether additional side conditions need to be specified.

## UL/DL Collision handling and scheduling restrictions

The fact that the gNB has full-duplex capabilities (i.e. can transmit and receive at the same time), while the UE remains half-duplex, introduces new challenges in terms of handling UL-DL transmissions at the UE-side, as compared to existing UE operation in legacy TDD bands. This topic has been discussed extensively in RAN1 under the term Collision handling. For unpaired TDD spectrum, several rules were specified since Rel-15 primarily targeting dynamic or flexible TDD operation where some OFDM symbols are assumed as flexible by the higher layers and can be more dynamically switched between UL and DL at the lower layers. Those rules are primarily defined in TS 38.213 and generally differentiate between a dynamically scheduled signal (e.g. UL or DL transmission triggered by a downlink control information (DCI) reception) and semi-static or higher-layer configured signal (UL or DL transmission not triggered by DCI, e.g. PRACH, SRS or periodic configured-grant PUSCH). More specifically, the following cases have been studied in RAN1:

Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Some agreements have been reached by RAN1 for these different collision cases which are noted in Table 2 below. Overall, RAN1 has agreed to keep same behaviour as legacy, that is, a dynamically scheduled signal is prioritized over a semi-static (RRC-configured signal), and overlapping of semi-static UL and DL signals, or dynamic UL and DL signals is not expected to happen at the UE side (thus is to be avoided at the gNB side).

Table 2: RAN1 agreed operation for UL-DL collision cases in SBFD operation

| Collision case | RAN1 agreed operation |
| --- | --- |
| Case 1: Dynamic DL vs semi-static UL | Same as legacy (if link direction is not supported): UL transmission is cancelled if cancellation timeline is met. FFS on dynamically scheduled DL reception with repetition |
| Case 2: Dynamic UL vs semi-static DL | Same as legacy (if link direction is not supported): UE does not receive DL channel/signal. FFS on dynamically scheduled UL transmission with repetition |
| Case 3: Semi-static DL vs Semi-static UL | Same as legacy (if link direction is not supported): An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol |
| Case 4: Dynamic UL vs dynamic DL | Same as legacy (if link direction is not supported): it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. FFS DL reception with repetitions and/or UL transmission with repetitions |
| Case 5: Dynamic/semi-static UL vs. SSB reception | Same as legacy: Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. |

For RRM point of view, Case 2 is a relevant one as it implies that the UE would prioritize a dynamically-scheduled UL transmission (i.e. triggered by DCI) over a semi-statically configured DL signal, e.g. periodic CSI-RS (see Figure 2). This does not happen in static TDD operation since symbols are exclusively used for one link direction (either UL or DL) and this link direction is fixed well in advance. Similar as how the measurement delay is impacted by the overlapping of CSI-RS with measurement gaps, we think similar principles shall be applied for this collision case.

Observation #10: According to RAN1 agreements, UE should prioritize a dynamically-scheduled UL transmission over a semi-statically configured DL signal, e.g. periodic CSI-RS.

Proposal 18: CSI-RS measurement delay requirements may need to be updated in case a CSI-RS resource is not received by the UE due to overlapping dynamic-UL transmission.

Figure 2: Prioritization of dynamic UL transmission over a semi-static DL reception

## UL/DL Collision handling and scheduling restrictions

As noted in Section 3.1, RAN1 has agreed that CSI-RS are configured in a contiguous manner (i.e. as legacy) but only the CSI-RS resources in the DL usable RBs are assumed to be valid, while other resources are ‘punctured’. For CSI-RS transmitted from the serving cell, the UE is aware of the SBFD configuration so it can determine the validity of the CSI-RS resources accordingly. However, how to perform CSI-RS measurements from neighbour cells (e.g. whether the puncturing is to be applied or not) is unclear considering the neighbour cells may or may not operate in SBFD mode. In order to measure CSI-RS from neighbor cell, UE needs to be aware of the SBFD configuration from neighbour which we understood not yet agreed in RAN1/2. RAN4 can wait for RAN1/2 discussion on the feasibility before discussing the RRM requirements for CSI-RS based neighbour cell measurement.

Proposal 19: RAN4 to wait for RAN1/2 discussion on the feasibility before discussing the RRM requirements for CSI-RS based neighbour cell measurement.

## DL-UL switching in SBFD slots

The following was captured in the WF from RAN4#112:

#### Issue 2-3-10: Limits on the maximum number of DL/UL switching Recommended WF - RAN4 to discuss whether to put limits on the maximum number of DL/UL switching during SBFD slots within the SBFD periodicity

During SBFD symbols, the UE is expected to switch between Tx and Rx mode according to the configured/scheduled transmissions/receptions for the UE (and/or potential link direction information, if supported). TS 38.211, Clause 4.3.2 specify the requirement for UE Tx-Rx transition time in the form of an error case as follows: “A UE not capable of full-duplex communication is not expected to transmit {receive} in the uplink {downlink} earlier than 𝑁Rx-Tx𝑇c {𝑁Tx-Rx𝑇c } after the end of the last received downlink {uplink} symbol”.

As noted in the WF, it has been discussed whether to have a limit on the number of transitions (from UE point of view) within e.g. a slot or the TDD UL-DL pattern period. From our point of view, we do not see any need to restrict the number of transitions in any manner, i.e. the UE can switch between Tx and Rx as many times as needed (based on scheduling or configuration) and it is up to the gNB to ensure the Tx-Rx switching time is respected, and gNB can guarantee the overhead due to such Tx-Rx switching time is kept to a reasonable level. Note that current specifications do not have any restriction in terms of number of Tx-Rx transitions for operation in flexible symbols; on top of that, some slot formats in TS 38.213, Table 11.1.1-1 allow up to 4 transitions within ONE slot, e.g. entry 51 with D->U->D->U. Based on these, we do not see motivation to introduce a different and more restrictive behavior for SBFD symbols.

Observation #9: Current specifications do not have any restriction in terms of number of number Tx-Rx transitions for operation in flexible symbols, e.g. for dynamic TDD scenarios. Some slot formats in TS 38.213, Table 11.1.1-1 allow D->U->D->U within ONE slot.

Proposal 20: RAN4 does not specify any limit regarding the number of Tx-Rx transitions the UE can perform within a time period (e.g. slot or TDD UL-DL pattern period).

# Simulation scenario

The following was captured in the WF from RAN4#112 wherein companies were asked to bring proposals on the simulation scenarios and assumptions for determining the CLI measurement requirement:

#### Issue 2-1-14: Link level simulations Recommended WF - RAN4 to discuss link level simulation assumptions for evaluating the performance of L1 UE-to-UE CLI measurement e.g. based on the possible new side condition and/or sample number Issue 2-1-8: Measurement period Agreements (from online session) - RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements, - RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design. - Number of shots for simulation: 1, 2, 3. - Companies to bring proposals on other simulation parameters in the next meeting.

As starting point, we can reuse similar assumptions as for the Rel-16 CLI study, see e.g. R4-1907392, consisting of one aggressor UE UE0 and one victim UE UE1, where both UE0 and UE1 are in the same serving cell. UE0 measures SRS-RSRP based on SRS transmitted by UE1. Simulations shall be conducted by varying the timing offset between SRS-RSRP transmission in the aggressor UE and corresponding measurement in the victim UE, and measuring the delta between the ideal RSRP measurement and the actual measurement. Other assumptions are listed below:

| Parameters |  | Value |  |
| --- | --- | --- | --- |
|  |  | UE0 (cell 0) [victim UE] | UE1 (cell 0) [aggressor UE] |
| Carrier frequency |  | 4GHz for FR1, 30GHz for FR2 |  |
| Subcarrier spacing |  | 30kHz for FR1, 120kHz for FR2 |  |
| SRS configuration | Number of port | N/A | 1 |
|  | TransmissionComb |  | 2 |
|  | combOffset |  | 0 |
|  | Cyclic shift |  | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 |
|  | Number of symbol |  | 1 |
|  | Number of RB Note1 |  | 24 or 48 |
|  | Frequency domain position and shift |  | 0 |
|  | Periodicity [slot] |  | 5 |
| Timing errorNote2 for UE 1 and UE2 |  | [13.5]usec for FR1, [7.2]usec for FR2 | - |
| CP Length |  | Normal |  |
| Number of measurement samples |  | 1, 2, or 3 | N/A |
| Measurement bandwidth [RB] |  | Same as SRS transmission RB |  |
| Measurement period [slot] |  | 5 | N/A |
| Number of UE receive/transmit antennas |  | 2 Rx | 2 Tx |
| Propagation conditions |  | AWGN, TDL-A10, TDL-C100 for FR1, AWGN, TDL-A75, TDL-C300 for FR2 |  |
| SINR |  | -3dB to 5 dB | N/A |
| Note 1: Allocated RBs are within active BWP for measurement UE. Note 2: Timing error without constant offset is (NTA_offset + NTA)A + Dpropagation_V – Dpropagtion_V2A . (NTA_offset + NTA)A is timing advance for aggressor(A) UE. Dpropagation_V is downlink propagation delay for victim(V) UE from the gNB of the victim UE. Dpropagtion_V2A is propagation delay from aggressor(A) UE to victim(V) UE. Note 3: No frequency hopping is assumed. |  |  |  |

Proposal 21: To reuse the R16 simulation scenario and assumptions except the simulation is run in intra-cell scenario.

# Conclusion

In this contribution, our views on the RRM requirements are presented. The following proposals were listed.

Observation #1: The SRS-RSRP CLI measurement in R19 indicates the interference on the UE’s “UL transmission” due to the UL transmission from aggressor UE.

Observation #2: Method#1 is not able to distinguish whether the CLI-RSSI measurement corresponds to UE-to-UE CLI or legacy inter-cell interference.

Proposal 1: To define the CLI measurement requirements in SBFD as the following:

- SRS-RSRP measurement requirements for Method#2.

- CLI-RSSI measurement requirements for Method#1 and Method#3.

Observation #3: In R19 duplex discussion, new reporting quantities L1-SRS-RSRP and L1-CLI-RSSI are agreed for CLI measurements hence legacy L3 based CLI measurement requirements do not apply.

Proposal 2: RAN4 should specify the measurement requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements in R19.

Observation #4: The Rel-16 assumptions for Rx beam used for CLI measurement in FR2 may not be applicable provided the latest RAN1 discussion.

Proposal 3: RAN4 to discuss the CLI measurement requirement considering the potential different Rx beams based on RAN1 conclusion.

Observation #5: In R16, 3 samples are assumed for deriving the SRS-RSRP measurement period and 1 sample RSSI is assumed for the CLI-RSSI measurement period.

Proposal 4: For L1-RSRP-RSRP measurement, the CLI measurement period can be specified following L1-RSRP measurement and both 1 and 3 samples are considered based on network configuration.

Proposal 5: For L1-CLI-RSSI measurement, the CLI measurement period is defined assuming 1 sample.

Observation #6: In R16 CLI measurement, UE would apply a constant offset to the DL reference timing in the serving cell, and the residual timing error Tother is defined as a side condition for the CLI measurement accuracy requirement.

Proposal 6: For L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

Proposal 7: To define the residual timing error Tother in intra-cell scenario as one side condition for the L1-SRSR-RSRP measurement accuracy requirement.

Proposal 8: For L1-SRS-RSRP measurement, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

Proposal 9: For L1-SRS-RSRP measurement, RAN4 to adopt the same SRS configuration, and to discuss the value of SRS Es/Iot on the UL subband in SBFD operation.

Proposal 10: At least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

Proposal 11: RAN4 should define new requirements for the sub-band CLI-RSSI reporting if this feature is supported by RAN1.

Proposal 12: RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

Observation #7: According to RAN1 agreement, only CSI-RS resources within DL subbands are measured by the UE and used to derive the report.

Proposal 13: For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, SBFD-aware UE is required to measure only CSI-RS frequency resources within DL usable PRBs.

Observation #8: The number of PRBs measured for the CSI-RS resource may be different on SBFD and non-SBFD symbols.

Proposal 14: RAN4 shall discuss if the measurement samples over SBFD and non-SBFD symbols can be combined when performing CSI-RS based measurements.

Observation #9: The “residual” amount of CSI-RS PRBs on SBFD symbols after puncturing operation may be smaller than 48 PRBs, which is a prerequisite for fulfilling many of the RAN4 RRM requirements.

Proposal 15: For CSI-RS on SBFD symbols, existing accuracy requirements for CSI-RS based measurements apply if the number of consecutive CSI-RS PRBs (i.e. within one of the DL subbands) is no less than 48 PRBs.

Proposal 16: For CSI-RS on SBFD symbols, existing accuracy requirements do not apply if the residual CSI-RS bandwidth (number of PRBs) after applying the puncturing operation is below 48 PRBs.

Proposal 17: For CSI-RS on SBFD symbols, if the residual CSI-RS RB bandwidth is at least 48 PRBs but it is split across the two subbands, RAN4 to discuss if existing accuracy requirements for CSI-RS based measurements can apply and whether additional side conditions need to be specified.

Observation #10: According to RAN1 agreements, UE should prioritize a dynamically-scheduled UL transmission over a semi-statically configured DL signal, e.g. periodic CSI-RS.

Proposal 18: CSI-RS measurement delay requirements may need to be updated in case a CSI-RS resource is not received by the UE due to overlapping dynamic-UL transmission.

Proposal 19: RAN4 to wait for RAN1/2 discussion on the feasibility before discussing the RRM requirements for CSI-RS based neighbour cell measurement.

Proposal 20: RAN4 does not specify any limit regarding the number of Tx-Rx transitions the UE can perform within a time period (e.g. slot or TDD UL-DL pattern period).

Proposal 21: To reuse the R16 simulation scenario and assumptions except the simulation is run in intra-cell scenario.

# References

- R4-2413900, WF on SBFD RRM, Huawei