3GPP TSG-RAN WG4 Meeting #112bis                                     R4-2415578

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:            Discussion on L3 measurement enhancement by optimizing Rx BSF

Agenda item:			6.15.2.1

Document for:	Discussion

1. Introduction

In last meeting, RAN4 had some discussions on FR2-1 SSB based L3 measurement delay reduction for connected mode. In this meeting, we will give some further discussions for some open issues captured in the agreed WF [1].

2 Discussion

2.1 FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier

From our understanding, the searcher can be deemed as a processing capability for multi-carrier, if we have multiple searchers, it may therefore be able to measure multiple carriers simultaneously, and the scaling would not be necessary accordingly, while in this case, it should be made sure that the UE utilizes its full capability and can get full benefits from the increased hardware complexity.

However, based on the objective of the WID, which is “For UE supporting multiple-Rx simultaneous reception on single carrier”, under such assumption, the only possibility given for UE is that to share the searcher(s) between only one configured CC. In this sense, no matter one searcher or two searchers, the RRM impact on measurement requirement (measurement period) scaled with the number of configured CCs is the same, since the minimum scaling factor is always one (Ncc=1)

Observation 1: No matter one searcher or two searchers, the minimum scaling factor for measurement period requirement is one (i.e., Ncc=1), the RRM impact on measurement period requirement can be the same

Nevertheless, if multi-carrier case will be discussed in the future, UE with one searcher would handle in parallel with several CCs. In such case, longer measurement period may occur when the number of CCs becomes large. Instead, two searchers are more flexible and reasonable, since it can provide a possibility that UE can perform measurement simultaneously when SMTC occasions on different CCs are overlapping. But CA case is not in the scope of Rel-19 enhanced L3 measurement

Proposal 1: In Rel-19, one searcher is considered for single carrier. Two searchers can be considered for the multi-carrier case discussed in the future release

2.2 FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement)

From our understanding, there are mainly three approaches to trigger UE using FBS for L3 measurement.

- Event triggers

- Event Ax

  - Define a new FBS L3 measurement event Ax or re-consider the existing triggering events and/or procedures, e.g., mobility-related events including measurement events.

- Triggering conditions

  - Depends on UE measurement and the entering condition. The triggered event would be UE to evaluate the measured quantity against a given network threshold within configured TTT duration. The rationale of low mobility and not-at-cell edge criteria for UE in idle mode can be used.

- Accuracy/Performance

  - The threshold for triggering the event should be properly set.

- NW control

- NW signalling to configure UE to follow different RRM measurement requirements for UE supporting L3 multi-Rx FBS capability. The different sets of enhanced RRM requirements in terms of number of Rx beams per UE

  - Whether to/How to introduce the new NW signalling

  - Which existing NW configuration/indication signalling (s) can be considered and how to reuse/modify the existing ones

Explicit and clear, with the NW signalling, NW have different expectations on the UE L3 measurement’s Rx beam scaling factor. The mechanism might be feasible. Since in FR2 HST, a NW signalling is introduced to configure UE to follow either Set 1 or Set 2 RRM requirements. The two sets of enhanced RRM requirement are related to Rx beam sweeping factor per UE. Set 1: 2 RX beams; Set 2: 6 RX beams. Maybe the similar principle can be inherited in this WI. But case by case analysis is needed. The triggering condition and the necessity of the (new) signalling should be further discussed in our PC3 FBS L3 measurement.

- UE measurement along with NW assistance of indication/configuration

Not prefer due to the implementation complexity

Observation 2: Three main approaches can trigger UE using FBS for L3 measurement

- Event triggers, e.g., measurement events

- NW signalling to configure UE to follow different RRM measurement requirements for UE supporting L3 multi-Rx FBS capability

- UE measurement along with NW assistance of indication/configuration

However, the question is, seems that we don’t have RAN2 TU, if we to define a new FBS L3 measurement event Ax or introduce a new NW signalling to configure/indicate UE to follow different RRM measurement requirements for UE supporting L3 multi-Rx FBS capability, additional RAN2 impact will be considered, since if the necessity of introducing (new) NW signalling is confirmed, detailed signalling definition is in RAN2 scope. While, minimal involvement from RAN2 should be expected in our opinion.

Proposal 2: The NW based solution to trigger UE using FBS for L3 measurement is simpler, but it may introduce additional RAN2 spec impact in terms of the detailed signalling design

From this, based on the comprehensive consideration of RAN2 Spec impact minimization and feasibility, we think:

Proposal 3: Suggest to prioritize event trigger mechanism to trigger UE using FBS for L3 measurement

- Consider the existing triggering events as the baseline

- Triggering conditions depend on UE measurement result and the entering condition including given network threshold. FFS: The low mobility and not-at-cell edge criteria for UE in idle mode can be taken as the baseline

Furthermore, as an optional function, it is not necessary and expected for UE to perform RRM measurement using FBS continuously. For example, in some cases, there is no need to receive signals from all panels simultaneously for saving power. In this sense, mechanism to quit BFS based L3 measurement is also needed. From our understanding, the approaches could be:

- Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off

- Dynamic indication

- Semi-static configuration

- Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event

- UE autonomously fallback based on measurement

Proposal 4: Suggest to discuss the mechanism to quit BFS based L3 measurement, the Alt.s can be

- Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off

- Dynamic indication; Semi-static configuration

- Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event

- UE autonomously fallback based on measurement

2.3 FFS: DRX is configured or not

From our understanding, in LTE, the inter-/intra-f measurement requirements with DRX operation had been defined for idle state and connected state mobility, respectively. Besides, for legacy PC3, the power consumption issue is still critical. So

Proposal 5: For FBS based L3 measurement, no need to deprioritize DRX case

2.4 Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

From our understanding, the legacy Rx beam sweeping procedure assumes that UE does not move or rotate significantly/quickly from the start of the beam sweep until it is complete, otherwise, the worst case is that the UE reports false beams, which were the best ones when the measurement was performed, but become rather weaker when the measurement period is complete, and the result is reported. In this sense, one of the benefits on faster beam sweeping in our case is that short delay in cell detection and measurement will shorten the difference between valid RSRP measurement report and actual condition when the report is provided, which can improve the system performance.

However, there is a tradeoff between mobility performance and number of samples to be measured

- The L3 delay requirements are based on total number of required samples for measurement M. To be specific, the M is proportional to the number of samples and RX beam sweeping factor N (i.e., 24= 3 × 8 ).

- In the previous release, there were two candidate values for the N, that is N=4 and N=8. The N=8 was finally compromised by taking mobility performance verification based on SLS and UE antenna implementation (spherical coverage) into consideration. Actually, from our understanding, from system performance point of view only, the N=4 for both PSS/SSS detection and RSRP measurement delay requirements can be accepted by companies.

- If the UE is able to simultaneously sweep K beam directions at one time instance, the multi-Rx capable and available UE need Nbeam/K measurement samples for scanning once over all the Rx beam directions. The BSF due to Rx beam sweeping operations equals to Nbeam/K. For K, here we can assume, e.g., K=2, for 2 Rx chains

If N/measured sample is reduced, the performance degradation issue may unavoidably arise. While, since no SLS results were obtained based on the new sweeping behaviour, we cant observe how much performance loss the UE will experience with different number of Rx beam sweeping, and cant conclude how to reduce the N, i.e, the total number of required samples for measurement accordingly under the new scenario. But we prefer to use N=4, the corresponding total samples is 20= 5 × 4 as baseline, at least one of the candidate values to define the delay requirements of inter-/intra-frequency measurement for Rel-19 L3 multi-Rx measurement, since as we mentioned before, the performance based on N=4 can be accepted by the companies in the previous release.

Proposal 6: To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline

- Whether to/How to define the other values can be based on SLS results (if necessary)

2.5 feature capability of L3 measurement delay reduction by optimizing Rx BSF

From our understanding: 1) The cell/beam identification delay and the measurement period might be variant for devices in the field with different N-factor, and such devices will likely not have the same system performance under similar conditions in mobility scenario, which may bring different impact on UE ability to track cells and NW decision; 2) The minimum RRM requirements defined in the current Spec., are based on the assumption that UE could scan/sweep one beam direction at one time instance; 3) The new multi-Rx L3 measurement is a new feature and most likely be optional, the NW needs to be functional with UEs that do/do not support the optional feature, and without capability signalling, the NW will be unaware of which UEs can support lower N (N1,N2,N3), and cant configure/handle them differently. From this, we prefer to define a new UE capability of supporting L3 multi-Rx FBS, and which is optional.

Proposal 7: Prefer to define a new UE capability of supporting L3 multi-Rx FBS, and which is optional

3 Conclusion

In this contribution, we provided our viewpoints to requirement impact analyses on L3 multi-Rx measurement. The following observations and proposals are obtained:

Observation 1: No matter one searcher or two searchers, the minimum scaling factor for measurement period requirement is one (i.e., Ncc=1), the RRM impact on measurement period requirement can be the same

Observation 2: Three main approaches can trigger UE using FBS for L3 measurement

- Event triggers, e.g., measurement events

- NW signalling to configure UE to follow different RRM measurement requirements for UE supporting L3 multi-Rx FBS capability

- UE measurement along with NW assistance of indication/configuration

Proposal 1: In Rel-19, one searcher is considered for single carrier. Two searchers can be considered for the multi-carrier case discussed in the future release

Proposal 2: The NW based solution to trigger UE using FBS for L3 measurement is simpler, but it may introduce additional RAN2 spec impact in terms of the detailed signalling design

Proposal 3: Suggest to prioritize event trigger mechanism to trigger UE using FBS for L3 measurement

- Consider the existing triggering events as the baseline

- Triggering conditions depend on UE measurement result and the entering condition including given network threshold. FFS: The low mobility and not-at-cell edge criteria for UE in idle mode can be taken as the baseline

Proposal 4: Suggest to discuss the mechanism to quit BFS based L3 measurement, the Alt.s can be

- Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off

- Dynamic indication; Semi-static configuration

- Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event

- UE autonomously fallback based on measurement

Proposal 5: For FBS based L3 measurement, no need to deprioritize DRX case

Proposal 6: To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline

- Whether to/How to define the other values can be based on SLS results (if necessary)

Proposal 7: Prefer to define a new UE capability of supporting L3 multi-Rx FBS, and which is optional

4 References

[1] R4-2414026, “WF for NR_RRM_Ph5_Part1”, Apple