3GPP TSG-RAN WG4 Meeting #112bis		                R4-2415580

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	Coexistence evaluation on D1T1 deployment scenario

Agenda item:	6.21.2.2

Document for:	Approval

# Background

In RAN4#112, the coexistence simulation assumption was further discussed [1] and agreed as captured in [2]. In this contribution, we analyze selected simulation parameters and results for the D1T1 deployment scenario.

# Discussion on the simulation assumption

The simulation assumptions for formal coexistence evaluation were agreed upon in RAN4#112 [1] but with a few parameters open. In this section, we analyze some parameters' impact on coexistence performance.

## Low pass filter assumption in R2D receiver for RF-ED

In RAN4#112, whether and how to consider the low-pass filter (LPF) in the R2D receiver for the RF envelope detector (ED) in device 1 was discussed. LPF is an essential component of the RF-ED receiver to remove the high frequency components that remain within the signal after the diode in the detector.

Proposal 1: the effect of LPF in the R2D receiver shall be taken into account for the coexistence simulation.

To model the effect of the LPF, we need to look into the operation mechanism of the RF-ED receiver. A simplistic RF envelope detector usually comprises a rectifier (a diode) and an LPF. The diode can be modeled as a component that squares the signal, which can be seen as a self-mixer that down-converts signals at any frequency point to DC, while some other inter-modulation components will also appear. Then, the LPF can (ideally) remove all the high-frequency components. Such a procedure is illustrated in Fig. 1.

Figure. 1 The illustration of RF-ED on the signals.

Based on this principle, it can be seen that an interference signal on a neighboring channel and the noise will be converted to DC after the diode; thus, the LPF will not be able to remove such interference. On the other hand, the noise bandwidth will be reduced to the same bandwidth as the LPF.

Observation 1: the LPF cannot remove the interference from neighboring channels as that interference and noise will self-mix to the DC after the diode in the RF-ED.

In RAN4#112, three approaches on how to model the LPF have been considered:

- Alt-1: Assume no frequency selectivity

- Alt-2:

| SINR | R2D with LPF |
| --- | --- |
| Interference from NR | Frequency selectivity: [7.5 dB] |
| Noise bandwidth | [180kHz] |
| SINRR2D_with_LPF=Wanted_SR2D_180kHzNoiseFloor 180kHz+(InterferenceNR10MHz/LPF_Selectivitylinear) |  |

- Alt-3:

| For R2D receiver with LPF |  |
| --- | --- |
| RF BW | 10MHz |
| BB BW | [180kHz] |
| SINRi (linear, not dB) | Input SINR at the antenna connector with noise/interferencde BW=RF BW |
| SINRo (linear, not dB) | Output SINR after BB LPF assuming downconverted noise/interference BW=BB BW |
| SINRo=SINRi28*SINRi+4*BWRFBWBB |  |

Based on the analysis of the RF-ED mechanism shared above, we provide our analysis of the three approaches below:

- Alt.1, as discussed above, does not reflect the actual RF-ED implementation, and it overestimates the interference level in the BB.

- For Alt. 2, it does not consider that both interference signal and noise will be converted to DC via the self-mixing of the RF-ED. In addition, the suppression level of 7.5 dB may also need to be further justified.

- Alt 3 originates from [4], which follows the principle of the RF-ED explained above (passing signal, noise, and interference via the self-mixing procedure) and derives the SINR equation accordingly. However, one critical assumption here is that a perfect square filter is assumed, and there is no consideration of the roll-off factor, which will underestimate the SINR degradation caused by the interference in neighboring channels.

Observation 2: coexistence simulation without LPF does not reflect the actual implementation of RF-ED and overestimates the interference. However, assuming a perfect square shape LPF as Alt 3 from RAN4#12 may underestimate the SINR degradation.

The SINR performance with different LPF assumptions is simulated and presented in Fig. 2, where we present the SINR of the AIoT R2D link when NR UL is the aggressor (100% indoor). It can be observed that almost 10 dB SINR difference can be observed between different approaches, and the actual performance may lie between the results of Alt 1 and Alt 3.

Figure. 2. The impact of different LPF modeling on the SINR performance.

In our view, modeling the LPF in RF-ED is not a simple task. Considering that the purpose of coexistence simulation shall ensure the protection between different systems and take into account the worst-case scenario, it might be okay to focus on the case with no LPF as the baseline. Then, If RAN4 identifies any R2D scenario that would be challenging for coexistence, RAN4 can further consider the effect of LPF. For example, consider adopting Alt.3, as shown above, but maybe with a slightly wider LPF BW to mimic the effect of the roll-off factor in actual LPF implementations.

Proposal 2: Considering the complication of modeling the LPF and the necessity of simulating the case scenario in a coexistence study, no LPF results can be taken as baseline results. If any coexistence cases are problematic, RAN4 can further discuss if those cases can be resolved by taking into account the LPF.

## Active rate of reader

Another critical assumption for coexistence simulations discussed in RAN4#112 is the active rate of the reader. Currently, RAN4 assumes that in total 18 readers would be uniformly deployed in a factory with 120m *60 m area, but only 2 readers with a minimum separation of 60m as baseline are activated simultaneously at each simulation drop. The reason for limiting the active rate of readers is to ensure an acceptable intra-system interference (baseline SINR) for the AIoT system. To show the effect, we randomly choose x of 18 readers, where x = 1 to 18 in the simulation results plotted in Fig. 3. It can be observed that when multiple readers (more than 1) are activated, the baseline SINR will be significantly reduced. Considering RAN4 has not concluded on the target SINR of the AIoT system (e.g., what SINR level is acceptable), the baseline assumption on the number of active readers may need to be further adjusted.

Figure. 3 the impact of AIoT reader activation rate on the intra-system SINR of AIoT R2D link.

Observation 3: Activating only a single reader can significantly improve the baseline SINR.

Proposal 3: the number of simultaneously active readers shall ensure the baseline SINR meets the target value.

On the other hand, deploying a denser number of readers and activating multiple of them simultaneously can improve the response speed in use cases like inventory management. Therefore, it may still be a preferred deployment case in real life. From this perspective, RAN4 may suggest further considering the mechanism to coordinate multiple readers as the outcome of the coexistence study via, e.g., multiplexing spatial domain when operating simultaneously.

Proposal 4: Some mechanisms to coordinate multiple readers to be active simultaneously may need to be considered.

## Indoor NR UE percentage for scenario option 1-2

Two options on the indoor UE percentage for scenario options 1-2 were discussed, which are 10% and 100%, and the effect of different percentages of indoor UE are studied in this section. In particular, we looked into the AIoT reader as the aggressor and NR DL as the victim, e.g., scenario 1-2 c. In this scenario, by increasing the percentage of the NR UE indoor, more NR DL will be affected by the transmission from AIoT R2D link

The simulation results are shown in Fig. 4. From the simulation results, it can be observed that SINR degraded linearly with a increased indoor UE percentage. Therefore, to ensure the coexistence, a reasonable indoor UE percentage shall be assumed for the simulations. In addition, further studies can be conducted on the mechanism that can support a higher percentage of indoor UE.

Figure. 4 The impact of indoor UE percentage on the SINR degradation of NR DL

Proposal 5: mechanisms to support a higher percentage of indoor UE may need to be studied.

# Discussion on the simulation results

The simulation results for D1T1 will be submitted together in the simulation collection file. In this section, we focus on the analysis of those results.

## Deployment scenario 1-1

Under this scenario, NR gNB is outdoor macro gNB, with NR UE only allowed outdoors while AIoT reader/CW/devices are indoors. As a result, indoor and outdoor penetration loss ensures a good isolation between the two systems. Based on our simulation results, no SINR degradation can be observed due to the inter-system interference in this case, where the worst case scenario is shown in Fig. 5 (NR UL as victim and reader as aggressor). Therefore, RAN4 can conclude that such a deployment scenario is feasible for D1T1 AIoT system from the coexistence performance.

Figure. 5. The worst case (NR UL as victim and reader as aggressor) SINR degradation  in deployment scenario 1-1

Observation 4: From the coexistence with the NR system perspective, the D1T1 scenario with NR UE is fully indoor and the AIoT system is fully outdoor, is feasible.

## Deployment scenario 1-2

Under this scenario, legacy NR gNB is an outdoor macro gNB with NR UE partially indoors, while AIoT reader/CW/devices are indoors. According to the simulation assumption, Two indoor NR UE percentage values are considered: 10% and 100%. As per the analysis already presented in section 1.3, the impact with a low percentage of indoor NR UE (10%) is small, and the performance is very similar to the case above in that NR UE is fully outdoor. On the other hand, when the NR UE is 100% indoor, the SINR degradation on the AIoT system due to NR UL, and the SINR degradation on NR DL due to the AIoT system are significant, as shown in Fig. 6. RAN4 may need to study the mechanism of handling such a scenario, as discussed in section 1.3.

Observation 5: the SINR degradation on AIoT system due to NR UL, and the SINR degradation on NR DL due to the AIoT system are significant when the NR UE percentage is 100% indoor.

Figure. 6. The performance with 100% indoor NR UE of (a) NR DL with AIoT reader as the aggressor, (b) NR DL with AIoT device as the aggressor, (c) AIoT D2R with NR UL as the aggressor, and (d) AIoT R2D with NR UL as the aggressor.

# Summary:

In this contribution, we provide our analysis of the coexistence performance of AIoT, where the following observations and proposals are given:

Observation 1: the LPF cannot remove the interference from neighboring channels as that interference and noise will self-mix to the DC after the diode in the RF-ED.

Observation 2: coexistence simulation without LPF does not reflect the actual implementation of RF-ED and overestimates the interference. However, assuming a perfect square shape LPF as Alt 3 from RAN4#12 may underestimate the SINR degradation.

Observation 3: activating only a single reader can significantly improve the baseline SINR.

Observation 4: From the coexistence with the NR system perspective, the D1T1 scenario with NR UE is fully indoor and the AIoT system is fully outdoor, is feasible.

Observation 5: the SINR degradation on AIoT system due to NR UL, and the SINR degradation on NR DL due to the AIoT system are significant when the NR UE percentage is 100% indoor.

Proposal 1: the effect of LPF in the R2D receiver shall be taken into account for the coexistence simulation.

Proposal 2: considering the complication of modeling the LPF and the necessity of simulating the case scenario in a coexistence study, no LPF results can be taken as baseline results. If any coexistence cases are problematic, RAN4 can further discuss if those cases can be resolved by taking into account the LPF.

Proposal 3: the number of simultaneously active readers shall ensure the baseline SINR meets the target value.

Proposal 4: some mechanisms to coordinate multiple readers to be active simultaneously may need to be considered.

Proposal 5: mechanisms to support a higher percentage of indoor UE may need to be studied.

# References

- R4-2412833   Topic summary for [112][131] FS_Ambient_IoT_solutions_part1       Moderator(CMCC)

- R4-2414304   WF on Coexistence study for ambient IoT	CMCC

- R4-2411537   Further considerations on the ambient IoT device implementation and RF aspect      Sony

- X. Huang, G. Dolmans, H. de Groot and J. R. Long, "Noise and Sensitivity in RF Envelope Detection Receivers," in IEEE Transactions on Circuits and Systems II: Express Briefs, vol. 60, no. 10, pp. 637-641, Oct. 2013