3GPP TSG-RAN WG4 Meeting #112bis		               R4-2415581

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	Further considerations on the ambient IoT device implementation and RF aspect

Agenda item:	6.21.3.2

Document for:	Approval

# Background

The RF aspect of ambient IoT (AIoT) devices has been further discussed in RAN4#112[1], where the following RF requirements have been concluded as potentially needed or not needed for various AIoT device types [2]:

- D2R

  - Requirements potentially needed:

    - Transmit OFF power for device 2b

    - Power control requirement for device 2b

    - ON/OFF time mask for device 2b

    - Frequency error for device 2b

    - In band emissions (IBE) for device 2b

    - Carrier leakage for device 2b

    - [SEM]

    - [ACLR]

    - [Spurious emissions]

  - Requirements not needed:

    - Power control requirement for device 1/2a

    - ON/OFF time mask for device 1/2a

    - Frequency error for device 1

    - In band emissions (IBE) for device 1

    - Carrier leakage for device 1

- R2D

  - Requirements potentially needed:

    - Reference sensitivity

  - Requirements not needed:

    - Spurious response

In this contribution, we further analyze selected RF implementation issues of AIoT devices and their impact on the remaining RF requirements.

# Ambient IoT device implementation assumptions

Three types of Ambient IoT devices, device 1, device 2a, and device 2b, have been discussed so far (see below) where device type 1 and device type 2a operate with the backscattering mechanism while device type 2b generates the transmission actively by itself.

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device's UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device's UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device's UL transmission is generated internally by the device.

In addition, the following architectures for the AIoT devices, including one architecture for device 1, one architecture for device 2a, and three architectures for device 2b, have also been agreed to be studied:

- (b)

(c)

Figure 1: The architecture of device 1, device 2a and device 2b that are to be studied in RAN1 .

Generally speaking, the RF architecture of AIoT devices can be separated into three parts:

1) Energy harvesting and storage part: collect and store the energy used to power the reader-to-device (R2D) and device-to-reader (D2R) operations at the AIoT device.

2) reception part: To receive the interrogation signal from the reader (R2D).

3) transmission part: To transmit the signal to the reader (D2R) either via backscattering (devices 1 and 2a) or generated by the AIoT device itself (device 2b).

In the following sections, we further analyse selected implementation issues of AIoT devices and their impact on RF performance.

## Reception part (R2D)

For the reception part on the R2D link, the RF envelope detector (RF-ED) receiver will likely be used for devices type 1 and 2a due to the constrained power budget. In addition, the RF-ED, IF, and zero-IF architecture have been proposed for device 2b. The AIoT receiver types are similar to those being discussed for low-power wake-up receivers (LP-WUR). Therefore, it can be expected that the reception part on the R2D link for ED-based AIoT devices can be leveraged from the ED-based LP-WUS receiver.

Observation 1: From the RF requirement perspective, the reception part of the R2D link for ED-based AIoT devices can be leveraged from the ED-based LP-WUS receiver.

### RF BPF

One particular issue observed during the LP-WUS SI in Rel-18 is that the RF-ED receiver lacks frequency selectivity [3]. Therefore, to suppress the interference from signals on adjacent spectrums, a high-Q RF BPF is usually required, where the bandwidth of BPF needs to be equal to the AIoT BW.

Observation 2: The RF-ED envelope detector lacks frequency selectivity.

However, if a multi-band operation needs to be supported in the R2D link, either no RF BPF at all or multiple RF BPFs that can cover all the frequency bands with all the possible RB allocations shall be used. Considering the complexity and power consumption limits for AIoT devices, the former choice (no RF BPF) may be more feasible, at least for device type 1.

In addition, even if there is only a single FDD band that the AIoT device needs to support, if the R2D link may happen both in the DL and UL spectrum due to the reader being either BS or intermediate UE, it will also be preferred not to implement any RF BPFs.

Observation 3: If multiple bands or different RB allocations need to be supported in the R2D link, using RF BPF in AIoT devices may not be feasible due to the complexity and power consumption limits.

Observation 4: Even under a single band operation, if the R2D link may happen both in the DL and UL spectrum due to the reader being either BS or intermediate UE, it will also be hard to implement any RF BPFs.

On the other hand, although the interference level would inevitably be increased after removing the RF BPF, some guard RBs between R2D signal and the neighboring signals may be configured to reduce further the interference level. Eventually, considering the target coverage of AIoT (~ 50m) is significantly smaller than the normal NR system, this may allow the implementation of RF-ED without any RF BPF.

Observation 5: The performance of the R2D link may still be acceptable without any RF BPF, considering potential guard RB, and also the limited coverage target.

Proposal 1: Assuming no RF BPF filter for at least device type 1 when discussing the RF performance of AIoT devices type 1, FFS on device type 2b and 2a.

Therefore, from the aspect of RF requirements and tests, verifying the R2D performance under various interference signal types is more critical due to the lack of frequency selectivity. In other words, receiver tests, including blocking, ACS, and spurious and intermodulation interference tests, are all relevant and maybe necessary for all AIoT devices. However, the exact level of interference signal and their offset to the centre frequency of R2D signal needs to be further discussed based on the applicable deployment scenario.

Observation 6: From the aspect of RF requirements and tests, verifying the R2D performance under various interference signal types is more critical due to the lack of frequency selectivity.

### BB LPF

For the BB LPF, a similar issue has also been studied for LP-WUS [3], where 3rd-order and 5th-order Butterworth filters were taken as baseline assumptions. The same assumption was adopted in the RAN1 LLS assumption for AIoT in [4]. Considering the extremely constrained power consumption and complexity,  a possible starting point of BB LPF filters could be the 3rd Butterworth filter.

Proposal 2: Considering the 3rd order Butterworth filter as the baseline assumption for the BB LPF to discuss the RF performance of AIoT devices further.

### ADC

Another critical aspect that affects R2D performance is the ADC. As we know, many ADC bits provide a higher dynamic range and SNR but also increase power consumption. Therefore, considering the minimal power budget for device type 1, a 1-bit ADC seems the only viable option. Meanwhile, more power consumption may be allocated to ADC for devices 2a and 2b, which allows 4-bit ADC to be considered. To our knowledge, this also aligns with RAN1 LLS assumption that was agreed in RAN1#117.

Proposal 3: Considering 1-bit ADC for device type 1 and 4-bit ADC for devices 2a and 2b.

### Requirements for R2D

Based on the discussion from the last meeting, the following requirements are needed for further discussion:

- Maximum input power

- Receiver intermodulation

- Rx spurious emission

- ACS

- ACSC

- In-band blocking

- Out-of-band blocking

In Tab. I below, we summarize our views on those RF requirements based on the assumptions and observations we made above:

Table I RF Requirement for AIoT device- RX part

| RX requirement | Maximum input power | The maximum input power is affected mainly by non-linear components, including LNA, the ADC, and the mixer. Therefore, this requirement should be specified by all types of AIoT devices. However, different maximum input power levels can be specified for different types of devices. |
| --- | --- | --- |
|  | Receiver intermodulation | Intermodulation response rejection measures the receiver's capability to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals that have a specific frequency relationship to the wanted signal. As discussed, the ED-based R2D receiver is likely to have a wide frequency response due to the lack of frequency selectivity, and the intermodulation production is likely to fall in the receiver bands. Therefore, it is necessary to test the intermodulation. |
|  | Rx spurious emission | The spurious emissions power is the power of emissions generated or amplified in a receiver that appears at the antenna connector. Considering no amplification in device 1, it may be okay for device 1 to omit such a requirement. However, for devices 2a and 2b, due to the usage of LNA in the receiver of R2D link, the Rx spurious emission requirements are relevant and should be defined. |
|  | ACS | The ACS and ASCS requirements shall be derived based on co-existence simulation, and guard band/guard RB may also be defined. - It should also be taken into account that no RF BPF would be used, at least for device type 1, so different sizes of guard band/RB may be considered for different device types. |
|  | ASCS | The ACS and ASCS requirements need to be specified for all AIoT devices, and they shall be derived based on co-existence simulation. The guard band/guard RB may also be defined. - It should also be taken into account that no RF BPF would be used, at least for device type 1, so different sizes of guard band/RB may be considered for different device types. |
|  | Out-of-band blocking | The in-band and out-of-band blocking requirements need to be specified for all AIoT devices, and it shall be based on the co-existence simulation while also considering the actual deployment scenario. |
|  | In-band blocking |  |

## Transmission part (D2R)

### Output power

The maximum output power requirements have been defined for all 3GPP UE types, which not only indicates a UE power capability but also ensures balanced uplink coverage v.s. interference level. Therefore, the maximum output power level is also needed for ambient IoT devices regardless of whether it is a backscattering or active type.

Observation 7: The maximum output power requirements have been defined for all 3GPP UE types, which not only indicates a UE power capability but also ensures balanced uplink coverage v.s. interference level.

Proposal 4: the maximum output power needs to be defined for devices 1, 2a, and 2b.

In the transmission direction (D2R), for device 2b, as it can generate the signal by itself, the device function can be more similar to legacy IoT devices than backscattering-type devices. Therefore, its maximum output power and the corresponding power class can be specified similarly to legacy UEs. However, the value is expected to be significantly lower than legacy IoT devices due to the power consumption limits.

Observation 8: For the D2R link, the AIoT device 2b might be similar to the UL of legacy IoT devices but with lower transmission power and reduced availability due to the energy harvesting and power consumption constraint.

Proposal 5: the maximum output power of device type 2b can be specified similarly to legacy UEs, and a new power class with a significantly lower power level than existing ones may need to be specified accordingly.

On the other hand, the backscattering type of devices, e.g., devices 1 and 2a, is transmitted to the reader via the reflection of a carrier wave (CW) from external nodes. Therefore, its transmission performance depends on both the design of AIoT devices and the property of the external CW and the emitter node.

Observation 9: For device types 1 and 2a, its transmission performance depends not only on the design of AIoT devices but is also highly impacted by the design of the CW and the corresponding transmission node.

Therefore, the maximum output power of device types 1 and 2a will be up to the device's reflection gain and the maximum input power level it can tolerate from the CW. Please note that this maximum input power level is different from the one in R2D, as this power level would be determined by the backscattering modulator and the reflection amplifier if applicable.

Proposal 6: The maximum output power and power class of device types 1 and 2a can be defined based on the reflection gain of the device and the maximum input power level it can tolerate from the CW.

For passive RFID tags, the type of maximum input power level that does not saturate the tag ranges from about -20 to -10 dBm. When it comes to the reflection amplifier, based on the literature review, a 10-20 dB amplification gain with a couple of hundred of uW power consumption can be considered as a starting point for the reflection performance. Therefore, defining the maximum output power and power class of device type 1 in the range of -20 to -10 dBm while about -10 dBm to 0 dBm for device 2a might be feasible. RAN4 shall further study what would be a reasonable assumption here.

Observation 10: The maximum out power of device type 1 could be in the range of -20 to -10 dBm, while about -10 dBm to 0 dBm for device 2a.

Proposal 7: RAN4 shall further study the maximum input power level of CW of device types 1 and 2a and the maximum gain if the reflection amplifier applies.

### Output power dynamic

Based on the discussion in RAN4#112, it is a common understanding that no power control requirements would be needed for the backscattering type of devices, and the power control can be applied on the CW node instead.

However, considering that active components might still be used, it is critical to ensure the backscattering device does not emit any high-power level when there is no incoming CW signal, and thus, an off-power requirement would be needed. The same value as NR and LTE of -50 dBm can be re-used.

Observation 11: it is critical to ensure the backscattering device does not emit any high-power level when there is no incoming CW signal.

A minimum output power requirement would also be necessary to ensure the AIoT devices can operate in a controlled manner under a minimum incoming CW power level. the corresponding CW power level for this minimum output power level can be further studied.

Observation 12: A minimum output power requirement would also be necessary to ensure the AIoT devices can operate in a controlled manner under a minimum incoming CW power level.

Proposal 8: Re-use the -50 dBm transmit off power and -40 dBm Minimum output power for all device types

Proposal 9: The transmit-off power of the backscattering devices should be defined as the emission level when no CW is illuminated on the devices. FFS on what the CW power level should be for minimum output power for devices type 1 and type 2a.

### Transmit signal quality and Output RF spectrum emissions

Transmit signal quality requirements and output RF spectrum emission requirements shall be applicable for all AIoT device types to ensure the nonlinearity components in the device can perform at an acceptable level and the spectrum of CW signal will not grow beyond the limit after modulation and reflection. In particular, the ACLR requirements will be based on the outcome of the co-existence study.

Proposal 10: Transmit signal quality requirements and output RF spectrum emission requirements shall be specified for all AIoT device types.

In addition, when it comes to the frequency accuracy requirement for device type 2a, it is critical if a frequency shift function would be needed for the backscattering type of devices. A common approach for such a frequency shift can be carried out by using an oscillator to toggle the switch speed on the load impedance, as shown in Fig. 3(a). Generally speaking, there are three levels of frequency shift: 1) no explicit frequency shift where the backscattering signal is on the same frequency as the CW signal (note that any frequency shift caused by the coding scheme may still be included). 2) a small but explicit frequency shift where the backscattering signal is slightly shifted away from the CW signal (e.g., one AIoT channel away) so that the interference from the CW on the backscattering signal can be reduced. 3) a large explicit frequency shift where the backscattering signal can be shifted to another band than the CW signal, e.g., the CW can be transmitted in the DL bands where the backscattering signal is transmitted in the UL frequency bands. The three different frequency shift schemes are illustrated in Fig. 4, which are represented by backscattering signals 1, 2, and 3, respectively, in the plot.

Observation 13: A small frequency shift (a few MHz) can be used to separate the backscattering signal from the CW signal to improve the SINR on the reader, while a larger frequency shift (tens or hundreds MHz) can be used to separate the CW and the backscattering signals on different bands or the same FDD band but different UL/DL spectrum.

Generally speaking, an oscillator consumes more power at a higher frequency. For example, assuming a ring oscillator has a few uW when it oscillates at a few MHz but can go up to hundreds of uW or even mW if it needs to oscillate at hundred(s) of MHz (depending on the oscillator design, for example, the number of stages in a ring oscillator). Therefore, with the given power budget (≤ a few hundred µW peak power consumption), the feasibility of a larger frequency shift needs to be further studied. Based on the state-of-the-art works, at least generating a frequency shift for backscattering communication in the range of tens of MHz has been realized.

Observation 14: The frequency translation range of the backscattering signal needs to be studied under the power consumption limitation.

(a)                                                                              (b)

Figure. 3. A possible method to perform (a) the frequency shift on backscattering signal and (b) reflection amplification.

Figure. 4. The possible frequency shift scheme for backscattering AIoT devices.

Another aspect related to the feasibility of frequency shift is the accuracy. Based on the latest discussion in RAN1#118 [4], it has been observed that large frequency shift may result in [e.g., 5kHz~50kHz] of frequency uncertainty in target frequency for clock accuracy of [e.g., 0.01%~0.1%] assuming the large frequency shift range is 50MHz. Therefore, maintaining good frequency accuracy when the frequency shift is larger will be more challenging, and the feasibility of using frequency shift and the corresponding requirements may also differ depending on the type of frequency shift that needs to be applied.

Proposal 11: RAN4 shall study the feasibility and necessity of frequency shift function for device type 2a and investigate its impact on frequency accuracy.

Regarding the necessity of frequency shift, it has also been observed in RAN1 that a large frequency shift may allow the reader to avoid implementing in-band full duplex capability on the reader in the case that CW is within the topology. In our understanding, a large frequency shift can enable a conventional FDD implementation of the reader by shifting the CW signal from DL spectrum to the UL spectrum. Meanwhile, a small frequency shift may enable the sub-band full duplex (SBFD) implementation on the reader side. Eventually, a full duplex mode may need to be supported by the reader if no frequency shift can be supported at all on the device side, assuming the CW node is integrated with the reader. Therefore, it is also necessary to consider from the reader implementation aspect on whether the frequency shift function would be necessary for AIoT devices to be implemented.

Proposal 12: The reader implementation aspect (e.g., which duplex mode reader can support) shall also be discussed regarding whether the frequency shift function would be necessary for implementing AIoT devices.

### Summary of transmission requirements for D2R

Requirement-wise, the following requirements are left for  further discussion from RAN4#112:

- Maximum output power

- EVM

- Transmit OFF power for device 1/2a

- Transmit time mask

- Minimum output power

- Frequency error for device 2a

- In-band emissions (IBE) for device 2a

- Carrier leakage for device 2a

- Occupied bandwidth:

- Transmit intermodulation:

With the above-mentioned analysis, our view regarding those requirements for the D2R link is summarized in the tab. II below:

Table II RF Requirement for AIoT device- TX part

| RF Requirement for AIoT device- TX part |  |  |  |
| --- | --- | --- | --- |
| Tx requirement | Transmit output power | Maximum output power | The maximum output power for the backscattering device can be specified based on the device's reflection gain and the maximum input power level it can tolerate from the CW. - We can consider -20 to -10 dBm dBm for device 1 while around -10 dbm to 0 dBm for device 2a as a starting point. For device type 2b, a similar approach as legacy UEs can be used to define its maximum output power. - A bit lower power level as device 2a (around -10 dBm) might be a starting point, consider that the device 2b needs to generate the CW wave internally by itself, thus more limited output power if we assume the same power consumption budget would be applied on device 2a and 2b. |
|  | Output power dynamic | Transmit OFF power | For all device types, consider the same level as NR and LTE, e.g., -50 dBm. - For the backscattering type of the device, this power level can be defined as the emission level from the device when there is no incoming CW signal. |
|  |  | Transmit time mask | It may not be necessary for backscattering types of AIoT device (1 and 2a), but specified for the CW node. |
|  |  | Minimum output power | Shall be specified for all device types. Consider the same level as NR and LTE for all AIoT device types, e.g., -40 dBm. - Further study should be conducted on the corresponding CW power level in this case. |
|  | Transmit signal quality | Frequency error | Transmit signal quality requirements shall be specified for all AIoT device types to ensure the device's nonlinearity component can operate adequately. - The frequency shift function of device 2a needs to be studied, as it may affect the frequency accuracy of the performance. - EVM is necessary for all device types. EVM is the most fundamental measure of the device that can modulate the signal properly. Therefore, regardless of whether it is a backscattering or active type device, the EVM needs to be specified. - For device type 1, even if there is no amplification component, signal quality requirements, at least EVM are still necessary. Otherwise, the OOK/PSK modulation quality cannot be guaranteed. - The IBE and carrier leakage requirements are necessary for device 2a to ensure the reflection amplifier can perform adequately. |
|  |  | EVM |  |
|  |  | In-band emissions |  |
|  |  | Carrier leakage |  |
|  | Output RF spectrum emissions | Occupied bandwidth | Output RF and spectrum emissions requirements shall be specified for all AIoT device types. - The ACLR level will depend on the co-existence. - The Occupied bandwidth is needed for all device types to ensure the transmission of devices is concentrated within the specific bandwidth, and no further spectrum grows after the backscattering due to the nonlinearity response of the AIoT devices. - The transmission intermodulation is needed for devices 2a and 2b at least, considering the usage of reflection amplifier and PA. |
|  |  | SEM |  |
|  |  | ACLR |  |
|  |  | Spurious emissions |  |

# Testability

In RAN4##112, it has been agreed that the OTA test is taken as the baseline for backscattering type of AIoT devices (devices 1 and 2a) [2]. Considering the AIoT device needs to harvest an external energy source to function, and backscattering devices also require a CW wave, how the test should be done can be further discussed.

Proposal 13: RAN4 shall study how CW signal and energy source should be provided during the conformance test.

## Impact of device availability and energy harvesting on Testability

To be fully functional, AIoT devices also require a dedicated energy source for harvesting to be provided during the test. Different types of resources can be used for different devices, and it is still a question of how the energy source can be provided during the OTA test chamber.

In addition, In [5], it is assumed that the duration of one device's unavailability due to charging by energy harvesting can be assumed to be up to several tens of seconds, which was endorsed in RAN#103. Meanwhile, no clear observation or conclusion has been made regarding the device's availability. Based on the experience of legacy UEs, the device transmission availability could have a huge impact on the feasibility of certain testing.

Observation 15: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds based on the endorsed observation in RAN#103. Meanwhile, no clear observation or conclusion has been made regarding the device's availability.

Observation 16: Based on the experience of legacy UEs, the device transmission availability could have a huge impact on the feasibility of some RF tests.

It is worth highlighting that this is an entirely different situation than for normal UEs in a test. Regular IoT devices or normal smartphones can operate over hours in conformance tests when intensive transmission and reception are performed. In addition, it is known that the conformance test can be very time-consuming, especially for OTA tests, as each test may last tens of minutes or even over an hour since the probe needs to scan a full space over the device. If an AIoT device were tested similarly, the test time may explode, considering that the device would be unavailable from time to time.

Observation 17: the transmission and reception behavior of AIoT devices are different from legacy UEs in 3GPP.

Proposal 14: RAN4 needs to study the device behavior (availability and unavailability) due to energy harvesting and investigate how it would affect the conformance test design.

## Impact from CW wave on Testability

As mentioned above, CW wave also needs to be provided in the conformance test. Under the OTA environment, while the CW node is transmitted, it is unclear how CW would affect the reception of the test system. One possibility is to calibrate the test system to treat CW as background noise, but it is still unclear if TE can separate the backscattered signal from the CW signal. Such an issue may also related to the CW cancellation discussion since the test equipment may need to mimic the behaviour of the actual AIoT reader. Considering that CW is necessary to verify the AIoT device behavior, investigating how to provide CW and how it would affect the test performance is necessary.

Proposal 15: RAN4 shall study CW, which would affect the D2R reception test under the OTA environment.

# Conclusion

In this paper, we provide our further analysis of the AIoT device RF implementation and requirement concept, where the following observations and proposals have been given:

Observation 1: From the RF requirement perspective, the reception part of the R2D link for ED-based AIoT devices can be leveraged from the ED-based LP-WUS receiver.

Observation 2: The RF-ED envelope detector lacks frequency selectivity.

Observation 3: If multiple bands or different RB allocations need to be supported in the R2D link, using RF BPF in AIoT devices may not be feasible due to the complexity and power consumption limits.

Observation 4: Even under a single band operation, if the R2D link may happen both in the DL and UL spectrum due to the reader being either BS or intermediate UE, it will also be hard to implement any RF BPFs.

Observation 5: The performance of the R2D link may still be acceptable without any RF BPF, considering potential guard RB, and also the limited coverage target.

Observation 6: From the aspect of RF requirements and tests, verifying the R2D performance under various interference signal types is more critical due to the lack of frequency selectivity.

Observation 7: The maximum output power requirements have been defined for all 3GPP UE types, which not only indicates a UE power capability but also ensures balanced uplink coverage v.s. interference level.

Observation 8: For the D2R link, the AIoT device 2b might be similar to the UL of legacy IoT devices but with lower transmission power and reduced availability due to the energy harvesting and power consumption constraint.

Observation 9: For device types 1 and 2a, its transmission performance depends not only on the design of AIoT devices but is also highly impacted by the design of the CW and the corresponding transmission node.

Observation 10: The maximum out power of device type 1 could be in the range of -20 to -10 dBm, while about -10 dBm to 0 dBm for device 2a.

Observation 11: it is critical to ensure the backscattering device does not emit any high-power level when there is no incoming CW signal.

Observation 12: A minimum output power requirement would also be necessary to ensure the AIoT devices can operate in a controlled manner under a minimum incoming CW power level.

Observation 13: A small frequency shift (a few MHz) can be used to separate the backscattering signal from the CW signal to improve the SINR on the reader, while a larger frequency shift (tens or hundreds MHz) can be used to separate the CW and the backscattering signals on different bands or the same FDD band but different UL/DL spectrum.

Observation 14: The frequency translation range of the backscattering signal needs to be studied under the power consumption limitation.

Observation 15: The duration of one device's unavailability due to charging by energy harvesting can be assumed up to several tens of seconds based on the endorsed observation in RAN#103. Meanwhile, no clear observation or conclusion has been made regarding the device's availability.

Observation 16: Based on the experience of legacy UEs, the device transmission availability could have a huge impact on the feasibility of some RF tests.

Observation 17: the transmission and reception behavior of AIoT devices are different from legacy UEs in 3GPP.

Proposal 1: Assuming no RF BPF filter for at least device type 1 when discussing the RF performance of AIoT devices type 1, FFS on device type 2b and 2a.

Proposal 2: Considering the 3rd order Butterworth filter as the baseline assumption for the BB LPF to discuss the RF performance of AIoT devices further.

Proposal 3: Considering 1-bit ADC for device type 1 and 4-bit ADC for devices 2a and 2b.

Proposal 4: the maximum output power needs to be defined for devices 1, 2a, and 2b.

Proposal 5: the maximum output power of device type 2b can be specified similarly to legacy UEs, and a new power class with a significantly lower power level than existing ones may need to be specified accordingly.

Proposal 6: The maximum output power and power class of device types 1 and 2a can be defined based on the reflection gain of the device and the maximum input power level it can tolerate from the CW.

Proposal 7: RAN4 shall further study the maximum input power level of CW of device types 1 and 2a and the maximum gain if the reflection amplifier applies.

Proposal 8: Re-use the -50 dBm transmit off power and -40 dBm Minimum output power for all device types

Proposal 9: The transmit-off power of the backscattering devices should be defined as the emission level when no CW is illuminated on the devices. FFS on what the CW power level should be for minimum output power for devices type 1 and type 2a.

Proposal 10: Transmit signal quality requirements and output RF spectrum emission requirements shall be specified for all AIoT device types.

Proposal 11: RAN4 shall study the feasibility and necessity of frequency shift function for device type 2a and investigate its impact on frequency accuracy.

Proposal 12: The reader implementation aspect (e.g., which duplex mode reader can support) shall also be discussed regarding whether the frequency shift function would be necessary for implementing AIoT devices.

Proposal 13: RAN4 shall study how CW signal and energy source should be provided during the conformance test.

Proposal 14: RAN4 needs to study the device behavior (availability and unavailability) due to energy harvesting and investigate how it would affect the conformance test design.

Proposal 15: RAN4 shall study CW, which would affect the D2R reception test under the OTA environment.

In addition, our view on the RF requirements are summarized in Table below:

Table I RF Requirement for AIoT device- RX part

| RX requirement | Maximum input power | The maximum input power is affected mainly by non-linear components, including LNA, the ADC, and the mixer. Therefore, this requirement should be specified by all types of AIoT devices. However, different maximum input power levels can be specified for different types of devices. |
| --- | --- | --- |
|  | Receiver intermodulation | Intermodulation response rejection measures the receiver's capability to receive a wanted signal on its assigned channel frequency in the presence of two or more interfering signals that have a specific frequency relationship to the wanted signal. As discussed, the ED-based R2D receiver is likely to have a wide frequency response due to the lack of frequency selectivity, and the intermodulation production is likely to fall in the receiver bands. Therefore, it is necessary to test the intermodulation. |
|  | Rx spurious emission | The spurious emissions power is the power of emissions generated or amplified in a receiver that appears at the antenna connector. Considering no amplification in device 1, it may be okay for device 1 to omit such a requirement. However, for devices 2a and 2b, due to the usage of LNA in the receiver of R2D link, the Rx spurious emission requirements are relevant and should be defined. |
|  | ACS | The ACS and ASCS requirements shall be derived based on co-existence simulation, and guard band/guard RB may also be defined. - It should also be taken into account that no RF BPF would be used, at least for device type 1, so different sizes of guard band/RB may be considered for different device types. |
|  | ASCS | The ACS and ASCS requirements need to be specified for all AIoT devices, and they shall be derived based on co-existence simulation. The guard band/guard RB may also be defined. - It should also be taken into account that no RF BPF would be used, at least for device type 1, so different sizes of guard band/RB may be considered for different device types. |
|  | Out-of-band blocking | The in-band and out-of-band blocking requirements need to be specified for all AIoT devices, and it shall be based on the co-existence simulation while also considering the actual deployment scenario. |
|  | In-band blocking |  |

Table II RF Requirement for AIoT device- TX part

| RF Requirement for AIoT device- TX part |  |  |  |
| --- | --- | --- | --- |
| Tx requirement | Transmit output power | Maximum output power | The maximum output power for the backscattering device can be specified based on the device's reflection gain and the maximum input power level it can tolerate from the CW. - We can consider -20 to -10 dBm dBm for device 1 while around -10 dbm to 0 dBm for device 2a as a starting point. For device type 2b, a similar approach as legacy UEs can be used to define its maximum output power. - A bit lower power level as device 2a (around -10 dBm) might be a starting point, consider that the device 2b needs to generate the CW wave internally by itself, thus more limited output power if we assume the same power consumption budget would be applied on device 2a and 2b. |
|  | Output power dynamic | Transmit OFF power | For all device types, consider the same level as NR and LTE, e.g., -50 dBm. - For the backscattering type of the device, this power level can be defined as the emission level from the device when there is no incoming CW signal. |
|  |  | Transmit time mask | It may not be necessary for backscattering types of AIoT device (1 and 2a), but specified for the CW node. |
|  |  | Minimum output power | Shall be specified for all device types. Consider the same level as NR and LTE for all AIoT device types, e.g., -40 dBm. - Further study should be conducted on the corresponding CW power level in this case. |
|  | Transmit signal quality | Frequency error | Transmit signal quality requirements shall be specified for all AIoT device types to ensure the device's nonlinearity component can operate adequately. - The frequency shift function of device 2a needs to be studied, as it may affect the frequency accuracy of the performance. - EVM is necessary for all device types. EVM is the most fundamental measure of the device that can modulate the signal properly. Therefore, regardless of whether it is a backscattering or active type device, the EVM needs to be specified. - For device type 1, even if there is no amplification component, signal quality requirements, at least EVM are still necessary. Otherwise, the OOK/PSK modulation quality cannot be guaranteed. - The IBE and carrier leakage requirements are necessary for device 2a to ensure the reflection amplifier can perform adequately. |
|  |  | EVM |  |
|  |  | In-band emissions |  |
|  |  | Carrier leakage |  |
|  | Output RF spectrum emissions | Occupied bandwidth | Output RF and spectrum emissions requirements shall be specified for all AIoT device types. - The ACLR level will depend on the co-existence. - The Occupied bandwidth is needed for all device types to ensure the transmission of devices is concentrated within the specific bandwidth, and no further spectrum grows after the backscattering due to the nonlinearity response of the AIoT devices. - The transmission intermodulation is needed for devices 2a and 2b at least, considering the usage of reflection amplifier and PA. |
|  |  | SEM |  |
|  |  | ACLR |  |
|  |  | Spurious emissions |  |

# References

- R4-2412834   Topic summary for [112][132] FS_Ambient_IoT_solutions_part2       Moderator(Huawei)

- R4-2414305   WF on RF requirements for A-IoT	Huawei

- TR 38.869	     Study on low-power Wake-up Signal and Receiver for NR

- R1-2407274    RAN1#118 FL Summary #2 for 9.4.1.2 Ambient IoT Device Architecture     Moderator (Qualcomm)

- RP-240116 "View on Ambient IoT scope". Qualcomm.