3GPP TSG-RAN WG4 Meeting #112bis				                   R4-2415583

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	Supporting 3MHz BW for Redcap/eRedcap NTN

Agenda item:	6.8.3.2

Document for:	Discussion

# Background

In RAN#103, a new WID: enhanced requirements and test methodology for NR and IoT NTN has been approved [1]. From core requirement-wise, the target of this WID is to support HPUE for both NR and IoT NTN, and also introduce the 3MHz channel BW into the NTN system.

In this contribution, we discuss the possibility of including the Redcap/eRedcap to be supported by 3MHz channel BW in the NTN system.

# Discussion

Supporting 3MHz channel BW for Redcap/eRedcap NTN was briefly discussed in RAN4#112 [2][3], and the major focus point is whether such a proposal is in the scope of WI.

3MHz channel BW in NR has been specified for TN operation since Rel-18 to support fragmented spectrums, especially for some re-farmed bands. In Rel-19, it is proposed to add this 3MHz channel BW for NR NTN operation as well to facilitate more efficient usage of spectrum for NTN bands, with the detailed motivation as below:

In Rel-18, the feature to support less than 5MHz for NR is defined for TN to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID “NR support for dedicated spectrum less than 5 MHz for FR1” by extending support for NTN.

This feature will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited. This feature should be built on the work done in the Rel-18, i.e., WI of NR support for dedicated spectrum less than 5 MHz for FR, as much as possible by extending support for NTN.

In Rel-19, in addition to normal NR NTN devices, Redcap NTN devices are also introduced and are expected to operate under the same network as normal NR NTN devices. Therefore, it may become problematic for the network to utilize the 3MHz channel BW to efficiently use the spectrum if the Redcap NTN device can’t be supported.

Observation 1: as Redcap NTN and NR NTN can operate under the same network, it will be a challenge for the network to efficiently use the spectrum if 3MHz channel BW is not supported by Redcap NTN.

One concern from the RAN4#112 discussion is that the Redcap feature is not included for 3MHz work in the TN case. We admit the fact here, but per our understanding that the Redcap was precluded from 3MHz TN work in Rel-18 is mainly due to the fact that additional work in RAN4 would be needed to specify the RF requirement for the 3MHz channel BW, and thus, it is precluded from the Rel-18 work. In addition, the 3MHz channel was motivated by a particular deployment scenario in TN, where the Redcap support may not be very relevant. However, the current work for NTN is for a more general purpose, as we understand, and thus Redcap shall not be precluded.

Observation 2: Different from the original motivation to introduce the 3MHz in the TN case, the current work for NTN is for a more general purpose, and thus Redcap shall not be precluded.

From the WI perspective, although the current WI requires leveraging the work done for TN, it does not preclude adding any features that do not exist for TN cases into the NTN network. The detailed objective is shown below:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

Observation 3: the current WI aims to leverage the work done for TN but does not preclude any new feature that was not introduced for NTN.

After RAN4#112 and RAN4#111bis, the majority of RF requirements for Redcap NTN have been agreed with only HD-FDD REFSENS left and some ambiguity regarding GNSS operation. Therefore, it should be feasible to include the Redcap NTN in the scope of the work.

Observation 4: The major part of the core requirements for Redcap NTN UE has been settled.

From a technical perspective, we understand that the major changes in the UE implementation required for 3MHz channel BW is on the baseband, where it should be able to decode the punctured SSB. However, very little change is needed in terms of RF implementation.

Observation 5: From the RF perspective, it is feasible for Redcap NTN UE to support a smaller BW.

In addition, it would make sense for Redcap devices to operate in a smaller bandwidth so their deployment can be more flexible to improve spectral efficiency. Power consumption can also be reduced with a narrow bandwidth. Therefore, it is proposed that the Redcap/eRedcap NTN in Rel-19 be considered to support the 3Mhz channel BW.

Observation 6: Supporting smaller BW can facilitate power consumption and deployment flexibility for Redcap NTN UE.

Proposal 1: RAN4 can focus on studying if there is any additional impact on the physical layer to support Redcap NTN with 3MHz channel bandwidth.

Proposal 2: It is proposed that the 3Mhz channel BW for Redcap/eRedcap NTN in Rel-19 be specified as long as there is no impact on physical layer design.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: as Redcap NTN and NR NTN can operate under the same network, it will be a challenge for the network to efficiently use the spectrum if 3MHz channel BW is not supported by Redcap NTN.

Observation 2: Different from the original motivation to introduce the 3MHz in the TN case, the current work for NTN is for a more general purpose, and thus Redcap shall not be precluded.

Observation 3: the current WI aims to leverage the work done for TN but does not preclude any new feature that was not introduced for NTN.

Observation 4: The major part of the core requirements for Redcap NTN UE has been settled.

Observation 5: From the RF perspective, it is feasible for Redcap NTN UE to support a smaller BW.

Observation 6: Supporting smaller BW can facilitate power consumption and deployment flexibility for Redcap NTN UE.

Proposal 1: RAN4 can focus on studying if there is any additional impact on the physical layer to support Redcap NTN with 3MHz channel bandwidth.

Proposal 2: It is proposed that the 3Mhz channel BW for Redcap/eRedcap NTN in Rel-19 be specified as long as there is no impact on physical layer design.

# References

- RP-240857 New WID: Enhanced requirements and test methodology for NR and IoT NTN

- R4-2413412	Topic summary for [112][312] NR_IoT_NTN_less_than_5MHz_UERF	Moderator (Xiaomi)

- R4-2413528	Way Forward for [112][312] NR_IoT_NTN_less_than_5MHz_UERF	Xiaomi