3GPP TSG-RAN WG4 Meeting #112bis				                   R4-2415584

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Sony

Title:	OCC scheme on the RF requirements for IoT NTN

Agenda item:	6.27.2

Document for:	Discussion

# Background

In Rel-19, uplink capacity enhancement is one of the objects for IoT NTN enhancement [1], with the detail as below:

- Support of Capacity enhancements for uplink

  - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4]

    - Multi-tone support for 15 kHz SCS should also be considered

    - Specify necessary signalling, if needed

    - Update RF requirements accordingly, if needed

Note: Impact of impairment shall be taken into account

In addition, the design of OCC scheme for IoT NTN has been discussed over a few meetings in RAN1, with the latest agreements from RAN1#118[1][2] as below:

Agreement

RAN1 studies whether the following types of UL transmission gap will impact the design of OCC for IoT-NTN when considering e.g. phase continuity

- UL gaps for synchronization (from Rel-13)

- Gaps around NPRACH occasions

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

- TDM DMRS that are muted

- Guard periods for 3.75kHz UL transmissions

Agreement

The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1:

- Option 1: OCC2, Symbol-level, TDM DMRS

- Option 2: OCC2, Symbol-level, CDM DMRS with new pattern

- Option 3: OCC2, Slot-level, TDM DMRS

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 6: OCC4, Symbol-level, CDM DMRS with new pattern

The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1:

- Option 1: OCC2, Symbol-level, TDM DMRS

- Option 3: OCC2, Slot-level, TDM DMRS

- Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern

- Option 5: OCC4, Symbol-level, TDM DMRS

- Option 7: OCC4, Slot -level, TDM DMRS

- Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern

Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS.

Note 2: Companies to report DMRS sequence applied.

Agreement

For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

From RAN4 perspective, applying the OCC on the UE NPUSCH may affect the the RF requirements on IoT NTN,where the following agreement has been made in RAN4#112 [4]:

NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH.

Issue 2-1: UE RF requirement impact for NPUSCH with OCC feature

- Agreement:

  - No UE RF requirement impact from symbol-level for NPUSCH

  - FFS on UE RF requirement impact from slot-level for NPUSCH

Issue 2-2: UE RF requirement impact for NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions on OCC feature before RAN4 evaluates the UE RF impact.

Issue 2-3: SAN RF requirement impact for NPUSCH/NPRACH with OCC feature

- Agreement:

  - Wait RAN1 reach conclusions for OCC feature before RAN4 evaluates the SAN RF impact.

Since RAN1 has not concluded on the OCC scheme yet, we share some initial analysis on the potential impact on UE RF requirement to support OCC operation for IoT NTN in this paper.

# Discussion

The basic principle of OCC is to apply a mask code on top of the repetiton pattern of  UE NPUSCH, where such mask codes are orthogonal between different UEs, so those UEs can be scheduled on the same time and frequency resources to transmit simultaneously to increase the channel capacity.

Per our understanding, it is neccesery for the UE to be able to maintain phase countinuty and minimise differential phase rotation between UEs over the the repetitions when OCC mask codes are applied to ensure the orthgonality between UEs that are multiplexed.

Observation 1: UE needs to maintain a phase countinuty and minimise differential phase rotation between UEs over reptitions when OCC masek code applies.

Currently, two schemes of OCC are under discussion in RAN1 for single-tone transmissions, which is symbol level OCC and slot level OCC. In more detail, two OCC lengths, OCC2 and OCC4, are considered for symbol and slot level,. In RAN4#112, it has been agreed that there will be no UE RF requirement impact from symbol-level for NPUSCH. It is our understanding that this is okay for OCC2 with symbol level due to the short time span of it. In this case, it can be expected that the UE is capable of maintain a sufficiently stable phase over the reptitions with the existing RF requirement.

Observation 2: pending on RAN1 design, if OCC-2 symbol level would be specified, no additional RF requirements are needed due to the short time span this scheme.

However, when it comes to OCC symbol level-4 and even slot level OCC, this may not be the case due to the increased time span of the OCC code. Due to the impact of CFO of UE and the stability of active componenets like PA over the time span of the OCC code, the phase countinuty may not be maintained, or there may be excessive phase rotation, and thus RF requirements may need to be specified to limit those effects. Therefore, RAN4 shall further study this aspect when RAN1 has made decisions.

Observation 3: Due to the impact of the CFO of the UE and the stability of active componenet like PA over the time span of the OCC code The UE may not maintain sufficient phase continuity within the time span of the OCC scheme if OCC4  symbol-level or OCC2/4 slot level would be adopted.

Proposal 1: Pending on RAN1 design, RAN4 may consider the potential RF requirement impact for OCC4  symbol-level or OCC2/4 slot level

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: UE needs to maintain a phase countinuty and minimise differential phase rotation between UEs over reptitions when OCC masek code applies.

Observation 2: pending on RAN1 design, if OCC-2 symbol level would be specified, no additional RF requirements are needed due to the short time span this scheme.

Observation 3: Due to the impact of the CFO of the UE and the stability of active componenet like PA over the time span of the OCC code The UE may not maintain sufficient phase continuity within the time span of the OCC scheme if OCC4  symbol-level or OCC2/4 slot level would be adopted.

Proposal 1: Pending on RAN1 design, RAN4 may consider the potential RF requirement impact for OCC4  symbol-level or OCC2/4 slot level.

# References

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R1-2407296	FL Summary #1 for IoT-NTN	Moderator (Sony)

- R1-2407297	FL Summary #2 for IoT-NTN	Moderator (Sony)

- R4-2413530  Way Forward for [112][316] IoT_NTN_Ph3    MediaTek Inc.