3GPP TSG-RAN WG4 Meeting #112 bis	R4-2415587

Hefei, China,  14 – 18 Oct 2024

Source:	Charter Communications, Cable Labs

Title:	SBFD minimum channel bandwidth and SBFD as a band-specific feature

Agenda item:	6.20.1	[NR_duplex_evo-Core]

Document for:	approval

Introduction

In TSG-RAN WG4 Meeting #112, a way forward [1] was approved with the following FFS:

- SBFD is a feature with requirements can potentially be applied to TDD bands, under the following considerations:

  - Option 1: Declaration based method.

    - The band supported for SBFD shall be base station (BS) vendor declaration based.

    - The channel bandwidth supported for SBFD shall be manufacturer declaration based.

      - The supported channel bandwidth can be impacted by the sub band/guard band size discussion.

  - Option 2: General principle but with consideration(s) for a specific band

    - General principle: The channel bandwidth shall be larger than X MHz

      - FFS the value of X.

        - X can be different for high and middle TDD bands

      - For a specific band, the following further restriction(s) can be considered:

        - Certain restrictions can be provided by the operator(s) for certain bands.

        - Other restrictions are not precluded.

In this contribution, we will discuss the reasons why we believe SBFD should have restrictions for bands with a minimum of 100MHz channel bandwidths and why SBFD should be a band-specific feature.

Discussion

2.1 SBFD Restrictions for Carrier Bandwidth

The following diagram shows the guard band between the DL and UL subbands of a 100MHz SBFD carrier, versus a 50 MHz SBFD carrier. The guard band is required to reduce the risk of blocking the UL receiver because of the high-power DL transmitter.

Figure 1: The guard band in a DUD SBFD configuration for 100 and 50 MHz channel bandwidth.

Observation 1: Figure 1 shows that the overhead of the 100 MHz guard band is smaller relative to the 50 MHz by a factor of 2.

Observation 2: This means that the 100 MHz SBFD carrier will have always a higher and better spectral efficiency than the 50 MHz carrier. For this reason, 100 MHz becomes our 1st priority relative to 50 MHz.

Proposal 1:   The channel bandwidth for a Subband Full Duplex carrier should be no less than 100 MHz.

2.2 SBFD as a band-specific feature

Figure 2 shows that in North America the CBRS band (known as n48) is in between two adjacent bands: AMBIT and C-Band (n78).

Figure 2: Band n48 and n77 spectrum allocation in the USA and EIRP requirements

Observation 3: Figure 2 shows that there are no guard bands between the AMBIT band and the CBRS band, as well as there are no guard bands between the CBRS band and the C-band.

Observation 4: Figure 2 shows that both the AMBIT band C-band can transmit at 75 dBm /10MHz EIRP, but the CBRS band is limited to 30 dBm/10MHz only. This means that the CBRS band EIRP is 45 dB smaller than band n77. Therefore, legacy TDD operations in the USA CBRS band will suffer much higher risk from adjacent-band SBFD deployments.

Figure 3:  n77 for USA

Therefore, we offer the following proposals. Since deploying SBFD near legacy TDD can severely interfere with the TDD operation, as shown in [3] during the Rel-18 SI phase, we strongly propose to exclude both n48 and n77 as our 1st priority:

Proposal 2: To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n50, n78, n79, n90, n96, n102, n104 for FR1, excluding n48 and n77.

As a compromise, we can also accept Proposal 3 instead:

Proposal 3: To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: 
n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1, 
except for the following bands in the USA:  n48 and n77.

Conclusion

The following Observations and Proposals were made:

Observation 1: Figure 1 shows that the overhead of the 100 MHz guard band is smaller relative to the 50 MHz by a factor of 2.

Observation 2: This means that the 100 MHz SBFD carrier will have always a higher and better spectral efficiency than the 50 MHz carrier. For this reason, 100 MHz becomes our 1st priority relative to 50 MHz.

Observation 3: Figure 2 shows that there are no guard bands between the AMBIT band and the CBRS band, as well as there are no guard bands between the CBRS band and the C-band.

Observation 4: Figure 2 shows that both the AMBIT band and C-band can transmit at 75 dBm/10MHz EIRP, but the CBRS band is limited to 30 dBm/10MHz only. This means that the CBRS band EIRP is 45 dB smaller than band n77. Therefore, legacy TDD operations in the USA CBRS band will suffer much higher risk from adjacent-band SBFD deployments.

Proposal 1:

Priority 1: The channel bandwidth of a Subband Full Duplex carrier should be no less than 100 MHz

Priority 2: The channel bandwidth of a Subband Full Duplex carrier should be no less than 50 MHz

Since deploying SBFD near legacy TDD can severely interfere with the TDD operation, as shown in [3] during the SI phase, we strongly propose to exclude both n48 and n77 as our 1st priority:

Proposal 2: To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n50, n78, n79, n90, n96, n102, n104 for FR1, excluding n48 and n77.

As a compromise, we can also accept Proposal 3 instead:

Proposal 3: To include the following text in Clause 5.2 in TS 38.104: Sub-band full duplex can be applied to the following TDD bands given in Table 5.2-1: 
n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1, 
except for the following bands in the USA:  n48 and n77.

- References

- R4-2413507 Way Forward for [112][307][308] NR duplex evo, Samsung

- TS 38104-i60.docx

- TR 38.858-i10.docx