3GPP TSG-RAN WG4 Meeting#112bis		R4-2415588

Hefei, China, Oct 14th -18th, 2024

Agenda item:	6.21.3.2

Source: 	Spreadtrum Communications

Title: 	Discussion on RF requirements impact for ambient IoT device

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follow.

Discussion

TX requirements

Agreement in RAN4#112: - Requirements potentially needed: - Transmit OFF power for device 2b - Power control requirement for device 2b - ON/OFF time mask for device 2b - Frequency error for device 2b - In band emissions (IBE) for device 2b - Carrier leakage for device 2b - [SEM] - [ACLR] - [Spurious emissions] - FFS whether requirements are needed or not: - Maximum output power - EVM - Transmit OFF power for device 1/2a - Transmit time mask - Minimum output power - Frequency error for device 2a - In band emissions (IBE) for device 2a - Carrier leakage for device 2a - Occupied bandwidth - Transmit intermodulation - Requirements not needed: - Power control requirement for device 1/2a - ON/OFF time mask for device 1/2a - Frequency error for device 1 - In band emissions (IBE) for device 1 - Carrier leakage for device 1

Maximum output power

The transmit output power is up to CW signal for device 1 and device2a. However, these device need to meet the coverage target of 10 meters and related regulatory requirements. In our understanding, it is necessary to define maximum output power for device1/2a/2b. Of course, different devices have different power consumption. Hence, different maximum output power for different devices.

Proposal 1: It is necessary to define maximum output power for device1/2a/2b.

EVM

The full name of EVM is Error Vector Magnitude. Literally, we can understand it as the vector error between the ideal error-free reference signal and the actual transmitted signal at a given moment, and this vector error can comprehensively measure the amplitude error and phase error of the modulation signal. From the point of view of this definition, an ideal reference signal is needed for EVM testing. It is observed that there is no any reference signal to be defined up to now. Maybe we can use preamble/midamble/postamble or other signal to be reference signal. However, the reference signal is ambiguous. In our understanding, whether EVM testing is feasible needs to be further studied.

Proposal 2: Whether EVM testing is feasible needs to be further studied.

On the other hand, we assume that EVM testing is feasible. According to RAN1#118 meeting agreement, OOK and Binary PSK for baseband modulation are feasible for D2R for all devices. In current spec, pi/2 BPSK EVM is 30%. On the whole, it is needed to define EVM for devices. However, device1 has ultra-low complexity and cost. The number of kinds of RF requirements should be less. In our understanding, EVM is not needed to consider for device1 in view of testing cost and testing time. In addition, as for device 2a and device2b, these two devices have more power consumption and cost. Maybe with higher positioning accuracy and higher data carrying capacity in the future release is needed. It is necessary to define EVM to ensure the related performance.

Proposal 3: EVM is not needed to define for device1. If EVM testing is feasible, EVM is needed to define for device2a and device2b.

Transmit off power

It is observed that excessive transmitter off power may increase the Rise over Thermal (RoT), thus reducing the cell coverage area of other UEs. If the off power of UE is too high, it means that the system floor noise of the whole cell has been raised. In order to meet a certain SNR requirement, the terminal receiving service will have to raise the transmission power, which in turn will cause higher noise for other UEs. As a result, the system capacity of the whole cell decreases. According to RAN1’s conclusion, D2R is supported for FDMA for all devices, which means that a cell has many devices. Excessive transmitter off power may cause SNR degradation for working devices. In addition, device2a has a backscatter amplifier and device2b has a power amplifier, these amplifiers need to be controlled. On the whole, to define transmit off power needs to be considered for device2a and device2b. For device1, with regard to complexity and cost. Perhaps we need to sacrifice performance for cost. Hence, there is no need to define transmit off power for device1.

Proposal 4: Transmit off power is not needed to define for device1. To define transmit off power needs to be considered for device2a and device2b.

Minimum output power

The purpose of defining the minimum power is to ensure the performance and SNR prevent from being drowned by noise. For device1, with regard to complexity and cost. Perhaps we need to sacrifice performance for cost. Hence, there is no need to define transmit off power for device1and device2a. For device2b, in our understanding, we need to consider.

Proposal 5: Minimum output power is not needed to define for device1 and device2a. To define minimum output power needs to be considered for device2b.

Frequency error

Frequency error is not only includes carrier frequency error, but also includes the impact from power, temperature and so on. For device1 and 2a, there is no carrier frequency error due to carrier from CW node. At the same time, device1 and device2a with no module (e.g., AFC) can adjust the frequency error caused by temperature and other factors. Hence, there is no need to define frequency error for device1 and device2a.However, device2b with LO will cause carrier frequency error. It is necessary to define frequency error for device2b.

Proposal 6: Frequency error is not needed to define for device1 and device2a; Frequency error is needed to define for device2b.

In band emission

According to the definition of in-band emission (IBE), IBE includes three parts: general, IQ image and carrier leakage. Carrier leakage will be discussed below, not here for the time being. For device1 with no large frequency shift, there is no IQ image. General requirement for device1 could be ensured by guard RB(s). Similarly, if device2a supports large frequency shift, IQ image needs to be defined. General requirement for device2a could also be ensured by guard RB(s). However, in order to improve frequency efficiency, maybe general requirement needs to be processed by device2b due to device2b having LPF and so on to suppress these emissions. On the whole, IBE is not needed to define for device1.For device2a, IBE is needed to define if it supports large frequency shift. IBE is needed to define for device2b.

Proposal 7: In band emission is not needed to define for device1; In-band emission is needed to define if it supports large frequency shift for device2a; in band emission is needed to define for device2b.

Carrier leakage

For device1 and device2a, the carrier is from CW node. Even though some circuit design factors lead to carrier leakage, which could be completely detected and eliminated by Reader since the LO location is known to Reader. Hence, there is no need to define carrier leakage for device1 and device2a. To the contrary, carrier is produced by itself for device2b. Too high carrier leakage will cause SNR degradation and needs to be avoided. It is necessary to define carrier leakage for device2b.

Proposal 8: Carrier leakage is not needed to define for device1 and device2a. Carrier leakage is needed to define for device2b.

Transmit intermodulation

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its nonlinear elements caused by the presence of the wanted signal and an interfering signal reaching the transmitter via the antenna. According to RAN1’s conclusion, CW can be single tone or two single tones with gap. For device1 with no backscattering amplifier, which is a nonlinear element. In our understanding, there is no need to consider transmit intermodulation for device1. For device2a, if CW is two single tones, new interference components are generated by intermodulation of two tones through nonlinear backscattering amplifier. Hence, transmit intermodulation needs to be considered for device2a. Similarly, device2b has a nonlinear element. From my point of view, device2b needs to consider transmit intermodulation.

Proposal 9: Transmit intermodulation is not needed to define for device1. Transmit intermodulation is needed to define for device2a and device2b.

RX requirements

Agreement in RAN4#112: - Requirements potentially needed: - Reference sensitivity power level - Maximum input power - Receiver intermodulation - FFS whether requirements are needed or not: - ACS - In-band blocking - Out-of-band blocking - Narrow band blocking - Receiver spurious response - Rx spurious emission - Requirements not needed: - ICS

ACS/In-band blocking

According to the architecture for device1 and device2a, just with BB LPF to suppress related interference. We can depend on the co-existence study. Even though we can use guard RB to ensure the ACS and In-band blocking replace to define requirements to save testing cost for device1 and device2a.In our understanding, device2b could consider ACS and in-band blocking.

Proposal 10: ACS/in-band blocking are not needed to define for device1 and device2a. ACS/in-band blocking need to be considered for device2b.

Out-of-band blocking

According to the architecture for device1 and device2a, could be with RF BPF to suppress related out-of-band blocking. Similarly, we need to sacrifice performance for cost. In our understanding, out-of-band blocking is not needed to define for device1 and device2a. For device2b, we could consider out-of-band blocking.

Proposal 11: Out-of-band blocking is not needed to define for device1 and device2a. Out-of-band blocking needs to be considered for device2b.

Receiver spurious response/ RX spurious emission

Similarly, with out-of-band blocking, we need to sacrifice performance for cost. In our understanding, receiver spurious response/RX spurious emission are not needed to define for device1 and device2a. For device2b, we could consider receiver spurious response/RX spurious emission.

Proposal 12: Receiver spurious response/ RX spurious emission are not needed to define for device1 and device2a. Receiver spurious response/ RX spurious emission need to be considered for device2b.

Conclusions

In this paper, we provide our views on RF requirements for ambient IoT device, we have made the following proposals.

Proposal 1: It is necessary to define maximum output power for device1/2a/2b.

Proposal 2: Whether EVM testing is feasible needs to be further studied.

Proposal 3: EVM is not needed to define for device1. If EVM testing is feasible, EVM is needed to define for device2a and device2b.

Proposal 4: Transmit off power is not needed to define for device1. To define transmit off power needs to be considered for device2a and device2b.

Proposal 5: Minimum output power is not needed to define for device1 and device2a. To define minimum output power needs to be considered for device2b.

Proposal 6: Frequency error is not needed to define for device1 and device2a; Frequency error is needed to define for device2b.

Proposal 7: In band emission is not needed to define for device1; In-band emission is needed to define if it supports large frequency shift for device2a; in band emission is needed to define for device2b.

Proposal 8: Carrier leakage is not needed to define for device1 and device2a. Carrier leakage is needed to define for device2b.

Proposal 9: Transmit intermodulation is not needed to define for device1. Transmit intermodulation is needed to define for device2a and device2b.

Proposal 10: ACS/in-band blocking are not needed to define for device1 and device2a. ACS/in-band blocking need to be considered for device2b.

Proposal 11: Out-of-band blocking is not needed to define for device1 and device2a. Out-of-band blocking needs to be considered for device2b.

Proposal 12: Receiver spurious response/ RX spurious emission are not needed to define for device1 and device2a. Receiver spurious response/ RX spurious emission need to be considered for device2b.

Reference

- R4-2414305, WF  on impacts of A-IoT on RF requirements, Huawei, TSG RAN WG4 meeting #112