3GPP TSG-RAN WG4 Meeting# 112bis								                        R4-2415591

Hefei, China, October 14th – 18th, 2024

Agenda item:			6.21.3.2

Source:	Spreadtrum communications

Title:	TP for TR 38.769 on device 1 RF requirements

Document for:	Approval

Introduction

This document proposes TP for TR 38.769 related to Device 1 RF requirements. The TP is based on the device 1 RF requirements related agreements and conclusions in previous RAN4 meetings, i.e., RAN4#111 and RAN4#112 (copied in appendix).

TP for TR 38.769 on device 1 RF requirements

The following primary TP for TR 38.769 (based on TR 38.769 v1.0.0) on device 1 RF requirements is proposed.

## 6.7 RF requirements study ### 6.7.4 Ambient IoT Device #### 6.7.4.1 Device 1 For device 1, Tx and Rx RF requirements are studied. The Tx and Rx RF requirements are shown in table 6.7.4.1-1 and table 6.7.4.1-2, respectively. For device 1, OTA test is the baseline. Table 6.7.4.1-1 Tx RF Requirement for A-IoT device 1 Tx requirement; Transmit output power; Maximum output power. Output power dynamic; Minimum output power. Transmit ON/OFF power; Transmit OFF power. Transmit signal quality; EVM. Output RF spectrum emissions; Occupied bandwidth. SEM. ACLR. Spurious emissions. Transmit intermodulation. Table 6.7.4.1-2 Rx RF Requirement for A-IoT device 1 RX requirement; Reference sensitivity. Maximum input power. ACS. ASCS. In-band blocking. Out-of-band blocking. Receiver intermodulation. Rx spurious emission. #### 6.7.4.2 Device 2a #### 6.7.4.3 Device 2b

Summary

In this contribution, we proposed TP for TR 38.769 on device 1 RF requirements.

Reference

[1] Chairman’s notes, RAN4#111

[2] Chairman’s notes, RAN4#112

Appendix

Device 1 RF requirements related agreements:

- General

Agreement in RAN4#112:

- Different RF requirement for Ambient IoT Device 1, Device 2a and Device 2b can be specified

- TX(D2R)

Agreement in RAN4#111:

- Use the following table as starting point for RF requirements impact study. The table is for information.

  - Encourage companies to provide views on RF requirements impact for different device types.

| RF Requirement for A-IoT device- TX part |  |  |  |
| --- | --- | --- | --- |
| Tx requirement | Transmit output power | Maximum output power |  |
|  | Output power dynamic | Transmit OFF power |  |
|  |  | Transmit time mask |  |
|  |  | Minimum output power |  |
|  |  | Power control requirement |  |
|  | Transmit ON/OFF power | Transmit OFF power |  |
|  |  | ON/OFF time mask |  |
|  | Transmit signal quality | Frequency error |  |
|  |  | EVM |  |
|  |  | In band emissions |  |
|  |  | Carrier leakage |  |
|  | Output RF spectrum emissions | Occupied bandwidth |  |
|  |  | SEM |  |
|  |  | ACLR |  |
|  |  | Spurious emissions |  |
|  | Transmit intermodulation |  |  |

Agreement in RAN4#112:

- Requirements potentially needed:

  - Transmit OFF power for device 2b

  - Power control requirement for device 2b

  - ON/OFF time mask for device 2b

  - Frequency error for device 2b

  - In band emissions (IBE) for device 2b

  - Carrier leakage for device 2b

  - [SEM]

  - [ACLR]

  - [Spurious emissions]

- FFS whether requirements are needed or not:

  - Maximum output power

  - EVM

  - Transmit OFF power for device 1/2a

  - Transmit time mask

  - Minimum output power

  - Frequency error for device 2a

  - In band emissions (IBE) for device 2a

  - Carrier leakage for device 2a

  - Occupied bandwidth

  - Transmit intermodulation

- Requirements not needed:

  - Power control requirement for device 1/2a

  - ON/OFF time mask for device 1/2a

  - Frequency error for device 1

  - In band emissions (IBE) for device 1

  - Carrier leakage for device 1

- RX(R2D)

Agreement in RAN4#111:

- Use the following table as starting point for RF requirements impact study. The table is for information.

  - Encourage companies to provide views on RF requirements impact for different device types.

| RF Requirement for A-IoT device- RX part |  |  |
| --- | --- | --- |
| RX requirement | Reference sensitivity |  |
|  | Maximum input power |  |
|  | ACS |  |
|  | ASCS |  |
|  | In-band blocking |  |
|  | Out-of-band blocking |  |
|  | Receiver intermodulation |  |
|  | Rx spurious emission |  |

Agreement in RAN4#112:

- Requirements potentially needed:

  - Reference sensitivity

- FFS whether requirements are needed or not:

  - Maximum input power

  - Receiver intermodulation

  - Rx spurious emission

  - ACS

  - ACSC

  - In-band blocking

  - Out-of-band blocking

- Requirements not needed:

  - Spurious response

- Testability

Agreement: in RAN4#111:

- FFS on whether conducted conformance test is feasible for AIOT devices.

- FFS on OTA test method, performance metric, etc.

Agreement in RAN4#112:

- Take OTA test as the baseline for at least Devices 1 and 2a