3GPP TSG-RAN WG4 Meeting#112-bis		R4-2415592

Hefei, China, Oct 14th -18h, 2024

Agenda item:	6.6.2

Source: 	Spreadtrum Communications

Title: 	Discussion on methods for reducing the number of UE RX chains

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved. In this contribution, we continue to provide some initial views on methods for reducing the number of UE RX chains.

Topic #2: Methods for reducing the number of UE Rx chains #### Issue 2-1-1: Applicability and clarification on the scope Online agreements: Agreement: - The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain - The two non-contiguous CCs is still under the current DL non-contiguous CA framework - Each fragment is treated as an individual CC - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz. - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands. - Provide the details related to gap between fragmented DL blocks] Topic #3: Impacts on UE RF requirements and DL performance #### Issue 3-1-4: Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements Recommended WF - Proposal 1: put off the signalling related discussions until the UE performance aspects are clear. - Appendix: Open issues Topic #2: Methods for reducing the number of UE Rx chains #### Issue 2-1-2: Reference architecture Recommended WF: - The existing requirements for DL NCCA was specified based on partially-shared Rx architecture according to TR36.823. - FFS on following options for reducing number of Rx chains for DL non-contiguous CCs: - Option 1: Only the “fully shared Rx chain architecture” needs to be studied - Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz - Other options are not precluded - Companies are encouraged to provide baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study - FFS on whether a single FFT are expected to cover the fragmented carriers or separate FFT are expected for each fragmented carrier. #### Issue 2-1-4: Study on power spectral density difference between carriers of co-located adjacent channel operators Recommended WF - FFS on following options in next meeting - Option 1: Prioritize the “co-location” deployment (meaning only location is shared, but not infrastructure) - For evaluation purpose and for simplicity, assume the two gNBs of two co-location operators are both transmitting at maximal allowed power level - equal PSD between two co-location operators could be assumed from gNB transmisstion point of view for evaluation purpose, as the starting point - The PSD difference from UE reception point of view could vary as per the different antenna type, antenna pointing direction, etc between operators - Option 2: For inter-operator deployment scenario, both shared RRU case and separate RRU case are considered - Other options are not precluded #### Issue 2-1-6: Others FFS following proposals in next meeting: - Proposal 1 - The impact of UE switching between common RF chain and separate RF chains, including potential interruption, should be studied. If there is RRM impact, RAN4 should consider it. - Proposal 2 - The new CA configuration supported only by adjusted RF requirements could be reported to the NW together with other default supported CA configuration. - Proposal 3 - gNB trigger UE to measure gap could be considered.

Discussion

UE reference architectures

The motivation of this SI is to increase the maximum supported number of component carriers (CCs) by reducing the number of UE RX chains. In the last meeting, the part of UE reference architecture was not discussed due to the limited time. Therefore, FL has given a recommended way forward to this reference architecture. The existing requirements for DL NCCA were specified based on partially-shared Rx architecture according to TR36.823. Option1 is that only the “fully shared RX chain architecture” needs to be studied. Option2 is that RAN4 also studies the possibility of supporting only 2RX in the mandatory 4RX band below 2700MHz. Besides that other options are not precluded.

However, whether a fully shared RX chain architecture is equivalent to a single RX chain needs to be further clarified. In our understanding, a single RX chain of two intra-band non-contiguous carriers means shared RF chain, shared analogue BB and shared digital BB. To be honest, different shared chains will save different costs and chip areas also face different challenges.

Shared RF chain and shared analogue BB is that shared antenna, LNA, LO, LPF and ADC. The in-gap interference which is caused by adjacent channel operator falls into this architecture between the two non-contiguous carriers can’t be filtered by LPF. In addition, TX leakage and TX image interference will aslo fall into this architecture. These interferences will cause the degradation of ACS and in-band blocking requirements and maybe exceed the dynamic range of ADC and lead to the deterioration of SNR. In our understanding, the DL performance of “fully shared RX chain” and “partially shared RX chain” is comparable. If the in-gap interference is too large, it will lead to performance degradation due to insufficient ADC dynamics.

To be further, shared digital BB includes that shared digital filter and a single FFT. If shared digital BB filter is to be considered, out of the boundary of two carriers rejection can’t be achieved. In our understanding, shared digital BB filter needs to be careful consideration. Besides, a single FFT is able to process when two CCs are aligned with SCS. However, a single FFT can’t process OFDM data when two CC with different SCSs are used. In existing spec, different SCS with two CCs is not limited. A single FFT will limit the scope of existing spec. Therefore, a single FFT should not be considered. Additionally, two non-contiguous carriers to be received by a single digital BB together, the cost and complexity of digital BB with the Gap bandwidth between two non-contiguous carriers increase compared to separate digital BB.

According to the above analysis, “fully shared RX chain architecture” with shared LNA, shared LO and LPF with separate ADC and Digital BB architecture needs to be studied priority below Figure 1. Whether this architecture with shared ADC and shared digital BB needs to be further studied.

Figure1: Shared LNA, shared LO and LPF with separated ADC and Digital BB architecture

Proposal 1: The concept of “fully shared RX chain architecture” and “a single RX chain” needs to be clarified.

Proposal2: the UE reference architecture that from shared antenna to shared LPF architecture, shown in figure1, is proposed.

Proposal 3: Separate FFTs are expected for each fragmented carrier.

Semi-statically switch hardware resources

The SID [2] mentioned that the ability to semi-statically switch hardware resources means that UE is needed to switch between fully shared RX chain and separate RX chains for DL NCCA( two intra-band non-contiguous carriers). It is observed that the performance (i.e., ACS, In-band blocking requirements, SNR) of fully shared RX chain is degraded compared to partially shared RX chain. From our perspective, it is hardly possible to maintain the ACS and in-band blocking requirements between fully shared RX chain and separate RX chain. Therefore, it is necessary to define the switching criteria between separate RX chains and fully shared RX chain for DL NCCA. The switching criteria can be SNR, power level of in-gap interference or others. With regard to how to get these results, gNB could trigger or indicate UE to measure when gNB needs to configure the UE with more multiple CCs or UE could do some measurements periodically autonomously.

Proposal 4: To define the switching criteria between separate RX chains and fully shared RX chain for DL NCCA should be considered.

Proposal 5: gNB trigger or indicate UE or UE do some measurements autonomously could be considered.

In addition, the potential impact of switching hardware resources needs to be considered, which maybe interruption, the degradation, and so on.

Proposal 6: Potential impact of switching hardware resources needs to be considered.

Conclusions

In this paper, we provide our views on methods for reducing the number of UE RX chains, we have made the following proposals.

Proposal 1: The concept of “fully shared RX chain architecture” and “a single RX chain” needs to be clarified.

Proposal2: the UE reference architecture that from shared antenna to shared LPF architecture, shown in figure1, is proposed.

Proposal 3: Separate FFTs are expected for each fragmented carrier.

Proposal 4: To define the switching criteria between separate RX chains and fully shared RX chain for DL NCCA should be considered.

Proposal 5: gNB trigger or indicate UE or UE do some measurements autonomously could be considered.

Proposal 6: Potential impact of switching hardware resources needs to be considered.

- Reference

- R4-2414457,WF on fragmented DL carriers study, WF on fragmented DL carriers study.TSG-RAN WG4 112 meeting

- RP-241669, New SID: Study on NR FR1 DL Fragmented Carries, Media Tek Inc. TSG-RAN Plenary 104 meeting