3GPP TSG-RAN WG4 Meeting #112-bis						R4-2415593

Hefei, Anhui, China 14th – 18th  October, 2024

Agenda Item:	6.5.2.2

Source: 	OPPO

Title: 	On the non-collocated UE behavior and capability

Document for:	Approval

- Introduction

In RAN4#112 meeting, the intra-band non-collocated UE behaviour and capability has been discussed and the WF [1] has been agreed. In this paper, we try to give further discussion on this topic.

- Discussion

During the last RAN4#112 meeting, the agreement for UE capabilities has been captured as below from the WF[1].

< Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA >

[Agreement:]

- Define one capability for NR CA and one UE capability for EN-DC

- Differentiate Type 4a and 4b by UE capability for EN-DC.

- Specific UE Capabilities’ design depends on RAN2.

It can be observed from the above captured agreements, the different understanding at least starts from whether type 4a UE can support NR-CA. We have looked back to the WID and previous agreements, from the WID [2], it has captured that “Note: UE types are defined in R4-2217734 (WF on NonCol_intraB_ENDC_NR_CA for New Type UE).”  In such case we refer to the UE types captured in WF [3] and the agreements are captured as below:

  - Sub-topic 2-1: UE RF architecture on new Type UE

< Issue 2-1-1: Possible UE RF architecture candidates on new Type UE>

Agreement:

- Agree the following table as UE RF architecture candidates on new Type UE for non-collocated non-contiguous intra-band NR-CA and inter-band EN-DC

  - Check whether there are other restrictions except for power imbalance. If some other restrictions are found, moderator will add them to the table immediately

- Prioritize Type 3a/3b discussion for 4MIMO layer in Rel-18.

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

< Issue 2-1-2: UE RF architecture on new Type UE>

Agreement:

- Discuss the following items in case of Type 3a/3b with 4 shared antennas and 4 shared LNAs (shared LNA AGC) for smartphone,

  - Necessary limitations or exceptions on dynamic range, REFSENS and blocking

  - FFS if imbalance < 25dB would allow larger dynamic range and whether large RTD will cause impact on the shared LNA.

  - Type 3a enabling 4Rx in one CC and 2Rx in the other CC for EN-DC

  - Type 3b enabling 4Rx in one both CCs for NR-CA and EN-DC

From the UE type table as captured above, at that time, the UE type definition for type 3a is for ENDC only since as also captured in the following agreement that for “ype 3a enabling 4RX in one CC and 2RX in the other CC for EN-DC.” From number of CCs perspective, also as limit the current WID scope to LTE band 42 and NR band n77, n78, it seems at least 8RX can be supported for NR-CA.

However, as currently this is intra-band CA for non-collocated which means this is for single band. A 6RX FWA who support 6 layers in both band n77 and n78 can declare it mandatory support single band n77 and single band n78 with mandatory 4RX required by the specification. This means for single band the number of RX is 4 as mandatory while the FWA has already equipped with 6. Then it comes to the intra-band non-contiguous NR CA as the maximum capability of the FWA is only 6 and hence it can only support 4+2 or even 3+3 although not defined currently in the spec. From our understanding, current definition as precluding the NRCA for type 4a leads to preventing the 6RX to be deployed for intra-band non-collocated NR CA. This is for sure not the result we want to see

Observation 1: A 6RX FWA who support 6 layers in both band n77 and n78 can declare it mandatory support single band n77 and single band n78 with mandatory 4RX required by the specification.

Observation 2: A 6RX FWA can only support 4+2 for NR CA.

Observation 3: Current definition as precluding the NRCA for type 4a leads to preventing the 6RX to be deployed for intra-band non-collocated NR CA.

Based on the analysis and observations above, it is our understanding that for type 4a should also include 6RX NR CA.

Proposal 1: For type 4a, include 6RX NR CA.

Based on the agreement in the last meeting as captured above, to define one capability for NR CA and one capability for EN DC, in such case this one capability for NR CA needs to differentiate the 6RX and 8RX. Although it is common understanding that RAN4 would leave RAN2 to further define the capability, it is our responsibility to tell RAN2 how this signalling needs to tell the capability.

During the last meeting, our contribution has investigated the type 2 signalling which is reported in the CA-ParametersNR which has been captured from TS 38.331 as below:

– CA-ParametersNR

The IE CA-ParametersNR contains carrier aggregation and inter-frequency DAPS handover related capabilities that are defined per band combination.

CA-ParametersNR information element

CA-ParametersNR-v1800 ::= SEQUENCE {

intraBandNR-CA-non-collocated-r18 ENUMERATED {supported} OPTIONAL

}

Also captured in TS 38.306 the signalling is defined as a per BC reported capability.

The type 2 UE capability tells the NW whether the UE support type 2 capability or not hence only 1 bit is enough. For the type 4a/4b UE capability, as only one capability will be defined as agreed in the last meeting, then we need to inform RAN2 that different from the type 2 UE capability, type 4 UE capability should at least include {4a, 4b, not supported} 3 different status.

Proposal 2: The one UE capability for type 4 NR CA should at least include {4a, 4b, not supported}.

The above discussion is for NR CA, while considering EN DC, another topic for signalling is that reusing the current LTE signalling for indicating MIMO layer as 2 or 4. This has been proposed by different companies to differentiate the type 4a and 4b for EN-DC. Firstly, we would like to clarify these signalling has no relationship with the non-collocated deployment scenarios since at the time they are designed it is only for collocated scenarios. Hence these signalling can only indicate the max MIMO layer capability of LTE for collocated scenarios. For non-collocated scenarios, new capability signalling is needed to specifically indicate the 2 layer or 4 layer support for a LTE band.

Proposal 3: New capability signalling is needed to indicate the 2 layer or 4 layer support for a LTE band in non-collocated scenarios.

Similar with the capability required for NR CA, then the one UE capability for type 4 EN DC should at least include {4a, 4b, not supported}.

Proposal 4: The one UE capability for type 4 EN DC should at least include {4a, 4b, not supported}.

Secondly, compared to type 2 capability, the NW signalling is used to configure the UE working at type 2 non-collocated and it is configured in the CellGroupConfig IE. The detail IE description is captured as below:

– CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels

with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the

configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::= SEQUENCE {

nonCollocatedTypeNR-CA-r18 ENUMERATED { true } OPTIONAL, -- Need R

uplinkTxSwitchingMoreBands-r18 SetupRelease { UplinkTxSwitchingMoreBands-r18 } OPTIONAL -- Need M

]]

During the last meeting, for the type 4 network signalling has been discussed and the agreements are captured as below:

< Issue 2-3-3:  New BS Signaling to switch between Type 4a/4b and Type 2 >

Agreement:

- No new NW signaling needed from Type-4 to Type-2 switch and maxMIMO-layer can be used.

< Issue 2-3-4: New BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated)>

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

< Issue 2-3-5:  New BS Signaling to switch between Type 4a/4b and Type 16L/8L/CC(collocated) >

Way Forward:

- First, conclude discussions of whether to add new UE capability(s) for Type 4.

- And then, continue further discussion on BS signalling in the next meeting.

Firstly, it has been agreed that no new NW signalling needed from Type 4 to Type 2 switch and maxMIMO-layer can be used. This is quite feasible to make the signalling clear and easy. From our understanding, the new BS signalling should also differentiate type 4a and 4b.

Proposal 5: The new BS signalling should also differentiate type 4a and 4b.

Due to the agreement with to use maxMIMO-layer for the switching from type 4a/4b to type 2, hence below table 1 is provided to clarify the relationship between UE capability, NW signalling and maxMIMO-layer.

Proposal 6: To agree on below relationship between UE capability, NW signalling and maxMIMO-layer.

Table 1

| intraBandNR-CA-non-collocated-type4 (New UE capability) | nonCollocatedTypeNR-CA-type4 (New BS signalling) | CC1 maxNumberMIMO-LayersPDSCH1 | CC2 maxNumberMIMO-LayersPDSCH2 | Requirement UE needs to fulfil |
| --- | --- | --- | --- | --- |
| a | a | 2 | 4 | Type 4a |
| a | a | 2 | 2 | Type 2 |
| b | b | 4 | 4 | Type 4b |
| b | b | 2 | 4 | Type 4a |
| b | b | 2 | 2 | Type 2 |

3	Conclusions

In this paper, we give further discussion on Type 4 UE behaviour and capability, the observations and proposals are as below:

Observation 1: A 6RX FWA who support 6 layers in both band n77 and n78 can declare it mandatory support single band n77 and single band n78 with mandatory 4RX required by the specification.

Observation 2: A 6RX FWA can only support 4+2 for NR CA.

Observation 3: Current definition as precluding the NRCA for type 4a leads to preventing the 6RX to be deployed for intra-band non-collocated NR CA.

Proposal 1: For type 4a, include 6RX NR CA.

Proposal 3: New capability signalling is needed to indicate the 2 layer or 4 layer support for a LTE band in non-collocated scenarios.

Proposal 4: The one UE capability for type 4 EN DC should at least include {4a, 4b, not supported}.

Proposal 5: The new BS signalling should also differentiate type 4a and 4b.

Proposal 6: To agree on below relationship between UE capability, NW signalling and maxMIMO-layer.

Table 1

| intraBandNR-CA-non-collocated-type4 (New UE capability) | nonCollocatedTypeNR-CA-type4 (New BS signalling) | CC1 maxNumberMIMO-LayersPDSCH1 | CC2 maxNumberMIMO-LayersPDSCH2 | Requirement UE needs to fulfil |
| --- | --- | --- | --- | --- |
| a | a | 2 | 4 | Type 4a |
| a | a | 2 | 2 | Type 2 |
| b | b | 4 | 4 | Type 4b |
| b | b | 2 | 4 | Type 4a |
| b | b | 2 | 2 | Type 2 |

4 References

[1] R4-2414453 	WF on Rel-19 non-collocated scenario, KDDI

[2] RP-240838 	New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

[3] R4-2217734 	WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI