3GPP TSG-RAN WG4 Meeting #112-bis						R4-2415594

Hefei, Anhui, China 14th – 18th October, 2024

Agenda Item:	6.5.2.1

Source: 	OPPO

Title: 	On the non-collocated UE RF requirements

Document for:	Approval

- Introduction

In RAN4#112 meeting, the intra-band non-collocated UE RF requirements have been fully discussed and due to the discussion on the UE capability and NW signalling has not been finalized yet, the RF requirements are still left open. Hence in this paper we try to give further discussion.

- Discussion

The detail requirement is captured below from TS 38.101-1. From our understanding, there are two parts for the requirement, one is the exact requirement that UE should fulfil and the other one is the situation describing the UE capability and NW signalling.

For the requirements itself, it has been agreed that to use “25dB power imbalance for 4RX REFSENSE +1dB for Type 4 UE” in the RAN4#110bis meeting. However, there are still applicability issue to be discussed in detail. Firstly, the 2RX RFSENS requirement should apply for 2 RX carrier while the 4RX REFSENS requirement should apply for 4 RX carrier. This also apply for EN-DC part as for EN-DC, still there are 2RX+4RX and 4RX+4RX for the LTE CC and NR CC.

Proposal 1: For both NR CA and EN DC, 2RX REFSENS requirement apply for 2RX carrier and 4RX REFSENS requirement apply for 4RX carrier.

Besides the RX port clarification, the UE capability and NW signalling should also be included in the requirement description. Currently there is an on-going issue which is to introduce the UE type of intra-band non-collocated deployment scenarios which is discussed in Rel-18 TEI. At least if there is any update of type 2 UE as introducing the type 2 terminology, then the type 4 terminology can also use the same description. In such case, it is no hurry to settle down the type 4 UE RF requirement wording but to first clarify the requirement relationship between the UE capability and NW signalling,

Proposal 2: To first clarify the requirement relationship between the UE capability and NW signalling,

7.10A	Power imbalance for CA

7.10A.1	General

Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal.

Power imbalance requirement in this subclause is only applicable for a UE capable of intraBandNR-CA-non-collocated-r18 and is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

7.10A.2	Minimum requirement

For the test parameters in table 7.10A.2-1, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1).

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. Four antenna port reference sensitivity allowance ΔRIB,4R in table 7.3.2-2 needs to be considered for UE equipped with more than 4 antenna port. NOTE 5: Void. |  |  |  |  |

For a UE capable of intraBandNR-CA-non-collocated-r18 for the following CA band combinations in Table 7.10A.2-2, the Rx requirements for two Rx ports are applicable for each component carrier if it is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2.

For a UE capable of [Rel-19 capability] for the following CA band combinations in table 7.10A.2-2, the RX requirements for two RX ports are applicable for the component carrier if it is provided with [Rel-19 NW signalling type 4a] and RX requirements for four RX ports are applicable for the component carrier if it is provided with [Rel-19 NW signalling type 4b].

Table 7.10A.2-2: NR CA combinations

CA combination

CA_n77(2A)1

CA_n78(2A)1

NOTE 1: The applicability is specified in clause 4.2 d)

In our companion paper of UE capability and NW signalling, we have fully discussed this topic and here we would like to further clarify the relationship between them. Below table 1 is to summarize different requirement applicability when different UE capability reported with different NW signalling configured. Detail can be seen in below table.

Table 1 Capability and NW signalling

| intraBandNR-CA-non-collocated-type4 | intraBandNR-CA-non-collocated-r18 | UE type can be configured | nonCollocatedTypeNR-CA-type4 | nonCollocatedTypeNR-CA-r18 | Requirement apply |
| --- | --- | --- | --- | --- | --- |
| a | Supported | 1，2，4 | a | Not provided | Type 4a |
|  |  |  | a | true | Type 4a |
|  |  |  | Not provided | Not provided | Type 2 |
|  |  |  | Not provided | true | Type 1 |
| a | Not reported | 1，4 | a | Network should not use this filed | Type 4a |
|  |  |  | Not provided |  | Type 1 |
| b | Supported | 1，2，4 | b | Not provided | Type 4b |
|  |  |  | b | true | Type 4b |
|  |  |  | Not provided | Not provided | Type 2 |
|  |  |  | Not provided | true | Type 1 |
| b | Not reported | 1，4 | b | Network should not use this filed | Type 4b |
|  |  |  | Not provided |  | Type 1 |
| Not reported | Supported | 1,2 | Network should not use this filed | true | Type 2 |
|  |  |  |  | Not provided | Type 1 |
| Not reported | Not reported | 1 | Network should not use this filed |  | Type 1 |

Proposal 3: To agree on above table 1 the relationship between UE capability and NW signalling and the UE RF requirement applicability.

3	Conclusions

In this paper, we give further discussion on Type 4 UE RF requirements, the observations and proposals are as below:

Proposal 1: For both NR CA and EN DC, 2RX REFSENS requirement apply for 2RX carrier and 4RX REFSENS requirement apply for 4RX carrier.

Proposal 2: To first clarify the requirement relationship between the UE capability and NW signalling,

Proposal 3: To agree on above table 1 the relationship between UE capability and NW signalling and the UE RF requirement applicability.

4 References

[1] R4-2414453 	WF on Rel-19 non-collocated scenario, KDDI

[2] RP-240838 	New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

[3] R4-2217734 	WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI