3GPP TSG-RAN WG4 Meeting #112-bis						R4-2415597

Hefei, Anhui, China 14th – 18th October, 2024

Agenda Item:	6.21.3.4

Source: 	OPPO

Title: 	On CW cancellation capability

Document for:	Approval

- Introduction

In RAN4#112 meeting, the CW cancellation capability has been raised during the simulation discussion. Also in the RAN#105 meeting that RAN4 will further discuss the CW cancellation capability. Hence in this paper we give further discussion on the CW cancellation.

- Discussion

During the last RAN1#118 meeting, the FL summary on CW waveform characteristics for A-IoT has been provided [1]. The CW interference suppression capability has been discussed and the interference suppression is captured as below.

Table 2.1.3.2-1 Interference suppression for different interference types at D2R receiver

| Interference type | Interference suppression methods |
| --- | --- |
| Self-interference | - Spatial domain cancellation [19] [23][25] [26] [34] [40] - antenna isolation [19] [23] [26] [34] [40], additional circulator[34] [40] or directional coupler [26] [40], duplexer[23], Physical shielding or cross-polarization [25] - Analog cancellation [19] [25] [26] - RF-IC suppression [19] [26] [34], BB filtering [26] [34], BB/IF self-interference cancellation [26] , BPF [23] - Digital cancellation [19] [23][25] [26] - channel estimation[25], high pass filter [26] [34], and/or spatial signalling processing [26] - Frequency shift for PDRCH at device, if supported [26] [40] [18], [25], [27], |
| Cross-link interference | - Spatial domain cancellation [19] [23][25] [26] [40] - antenna separation/ isolation [19][25] [26], antenna design [26], circulator [34] - Analog cancellation [19] [26] [34] [40] - BB filtering [26], BB/IF interference cancellation [26] - Digital cancellation [19] [26] [40] - high pass filter [26] [34], digital BB interference cancellation [26] - Design for channel estimation and frequency & timing offset estimation [25][15][33] - Frequency shift for PDRCH at device, if supported [26] [40] [18], [25], [27] |

As can be seen the interference suppression cancellation capability can be divided at least into 4 parts which is antenna isolation, Analog cancellation, Digital cancellation and others. A very similar scenario to be reference is the SBFD interreference cancellation scheme which has been captured in TR 38.858. Similar parts has been discussed separately.

Observation 1: The interference suppression cancellation capability can be divided into 4 parts which is antenna isolation, Analog cancellation, Digital cancellation and others.

From this perspective, it seems the BS and UE should have different CW cancellation capability. For example, the antenna isolation for base station can be achieved easier than UE and also the analogue cancellation for UE is harder to achieve.

Proposal 1: For the CW cancellation capability, BS and UE should have different capability.

For the BS perspective, as refer to the SBFD interference cancellation capability, the spatial isolation is around 80dBc. Considering the Beam nulling technology, further 10dBc cancellation can be applied. The RF IC is around 10 to 15dBc while even some of the company thinks as low as low to 0dBc; digital IC is around 15 to 20dBc. In such case, around 115 to 125dBc total cancellation is considered. This can be the up to date technology for interference cancellation. Furthermore, the free space pathloss is also considered for the CW cancellation. Consider the UE antenna design is much harder than the BS, the isolation cannot reach as high as 80dBc and also the RF IC for UE is usually not feasible, a total of 120dBc cancellation for UE is considered.

Proposal 2: 120dBc is used in co-existence simulation for CW cancellation for A2 for intermediate UE.

3	Conclusions

In this paper, we give further discussion on Type 4 UE behaviour and capability, the observations and proposals are as below:

Observation 1: The interference suppression cancellation capability can be divided into 4 parts which is antenna isolation, Analog cancellation, Digital cancellation and others.

Proposal 1: For the CW cancellation capability, BS and UE should have different capability.

Proposal 2: 120dBc is used in co-existence simulation for CW cancellation for A2 for intermediate UE.

4 References

[1] R1-2407544	Final FL summary on CW waveform characteristics for A-IoT, Moderator (Spreadtrum Communications)