3GPP TSG-RAN WG4 Meeting #112-bis						R4-2415598

Hefei, Anhui, China 14th – 18th October, 2024

Agenda Item:	6.21.3.2

Source: 	OPPO

Title: 	On device RF requirements

Document for:	Approval

- Introduction

In RAN4#112 meeting, the device RF requirements have been fully discussed. Due to the study phase, it is not urgent to make decision on the exact RF requirements. However, a first glance is quite important for further study. Hence in this paper we give further analysis on device RF requirements.

- Discussion

For the TX requirement, it has been mentioned in our previous paper about regulation, only TX power is required. From one point, this is to limit the output power of device to not interfere other service and from the other point, it is also important to guarantee the device have enough output power to be received. However, when it comes to the 3GPP specification, we want to have more RF performance to be observed and hence more core requirements are defined besides the output power. During the last meeting discussion, the agreements is captured as below:

Agreement in RAN4#112:

- Requirements potentially needed:

  - Transmit OFF power for device 2b

  - Power control requirement for device 2b

  - ON/OFF time mask for device 2b

  - Frequency error for device 2b

  - In band emissions (IBE) for device 2b

  - Carrier leakage for device 2b

  - [SEM]

  - [ACLR]

  - [Spurious emissions]

- FFS whether requirements are needed or not:

  - Maximum output power

  - EVM

  - Transmit OFF power for device 1/2a

  - Transmit time mask

  - Minimum output power

  - Frequency error for device 2a

  - In band emissions (IBE) for device 2a

  - Carrier leakage for device 2a

  - Occupied bandwidth

  - Transmit intermodulation

- Requirements not needed:

  - Power control requirement for device 1/2a

  - ON/OFF time mask for device 1/2a

  - Frequency error for device 1

  - In band emissions (IBE) for device 1

  - Carrier leakage for device 1

Firstly, the max output power is needed. As described in the first part, the regulation has requirement for the tag radiated power and the test is OTA method. It is illustrated as below:

Also as explained previously, there are two meaning for the test of max output power, one is for limiting the emission and the other one is to guarantee the TX power.

Observation 1: There are two meaning for the test of max output power, one is for limiting the emission and the other one is to guarantee the TX power.

Hence, we believe the max output power is needed and quite important to be defined. A little bit more complex might be that for device 2b, the requirement might be defined differently, i.e. not OTA but conducted. However, this can be further discuss in the future phase.

Proposal 1: To define max output power for all type of devices.

For other requirements, it is summarized as below table 1:

Table 1 Summary of other TX requirements

| EVM | Not needed for device 1/2a, potentially needed for 2b |
| --- | --- |
| Transmit OFF power for device 1/2a | Not needed |
| Transmit time mask | Not needed for device 1/2a, potentially needed for 2b |
| Minimum output power | Not needed for device 1/2a, potentially needed for 2b |
| Frequency error for device 2a | Needed as device 2a is frequency offset sensitive |
| In band emissions (IBE) for device 2a | Needed to be further discussed together with co-existence |
| Carrier leakage for device 2a | Not needed |
| Occupied bandwidth | Needed |
| Transmit intermodulation | Not needed for device 1/2a, potentially needed for 2b |

Proposal 2: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

| EVM | Not needed for device 1/2a, potentially needed for 2b |
| --- | --- |
| Transmit OFF power for device 1/2a | Not needed |
| Transmit time mask | Not needed for device 1/2a, potentially needed for 2b |
| Minimum output power | Not needed for device 1/2a, potentially needed for 2b |
| Frequency error for device 2a | Needed |
| In band emissions (IBE) for device 2a | Needed to be further discussed together with co-existence |
| Carrier leakage for device 2a | Not needed |
| Occupied bandwidth | Needed |
| Transmit intermodulation | Not needed for device 1/2a, potentially needed for 2b |

For the RX requirements, below table 2 is provided:

Table 2 Summary of other RX requirements’

| Maximum input power | Not needed for device 1/2a, potentially needed for 2b |
| --- | --- |
| Receiver intermodulation | Not needed for device 1/2a, potentially needed for 2b |
| Rx spurious emission | Needed |
| ACS | Potentially Needed |
| ACSC | Needed to be further discussed together with co-existence |
| In-band blocking | Potentially Needed |
| Out-of-band blocking | Potentially Needed |

For the RX spurious emission, it is regulation required and hence needed for all three types of devices.

For ACS, it is potentially needed to see the adjacent channel interference while the ACSC is not used for device 1 and 2a for the co-existence study, however, this can be further study together with the outcome of co-existence.

For in-band and out-of-band blocking it is potentially needed so that the receiver will not be blocked.

Proposal 3: To agree on the following table for RX requirements:

Table 2 Summary of other RX requirements’

| Maximum input power | Not needed for device 1/2a, potentially needed for 2b |
| --- | --- |
| Receiver intermodulation | Not needed for device 1/2a, potentially needed for 2b |
| Rx spurious emission | Needed |
| ACS | Potentially Needed |
| ACSC | Needed to be further discussed together with co-existence |
| In-band blocking | Potentially Needed |
| Out-of-band blocking | Potentially Needed |

3	Conclusions

In this paper, we give further discussion on device RF requirement the observations and proposals are as below:

Observation 1: There are two meaning for the test of max output power, one is for limiting the emission and the other one is to guarantee the TX power.

Proposal 1: To define max output power for all type of devices.

Proposal 2: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

| EVM | Not needed for device 1/2a, potentially needed for 2b |
| --- | --- |
| Transmit OFF power for device 1/2a | Not needed |
| Transmit time mask | Not needed for device 1/2a, potentially needed for 2b |
| Minimum output power | Not needed for device 1/2a, potentially needed for 2b |
| Frequency error for device 2a | Needed |
| In band emissions (IBE) for device 2a | Needed to be further discussed together with co-existence |
| Carrier leakage for device 2a | Not needed |
| Occupied bandwidth | Needed |
| Transmit intermodulation | Not needed for device 1/2a, potentially needed for 2b |

Proposal 3: To agree on the following table for RX requirements:

Table 2 Summary of other RX requirements’

| Maximum input power | Not needed for device 1/2a, potentially needed for 2b |
| --- | --- |
| Receiver intermodulation | Not needed for device 1/2a, potentially needed for 2b |
| Rx spurious emission | Needed |
| ACS | Potentially Needed |
| ACSC | Needed to be further discussed together with co-existence |
| In-band blocking | Potentially Needed |
| Out-of-band blocking | Potentially Needed |

4 References

[1] R4-2414305	WF on RF requirements for A-IoT, Huawei