3GPP TSG-RAN WG4 Meeting #112-bis						R4-2415599

Hefei, Anhui, China 14th – 18th October, 2024

Agenda Item:	6.21.3.3

Source: 	OPPO

Title: 	On intermediate UE RF requirements

Document for:	Approval

- Introduction

In RAN4#112 meeting, the device RF requirements have been fully discussed. Due to the study phase, it is not urgent to make decision on the exact RF requirements. However, a first glance is quite important for further study. Hence in this paper we give further analysis on device RF requirements.

- Discussion

For the TX requirement, it has been mentioned in our previous paper about regulation, only TX power is required. From one point, this is to limit the output power of device to not interfere other service and from the other point, it is also important to guarantee the device have enough output power to be received. However, when it comes to the 3GPP specification, we want to have more RF performance to be observed and hence more core requirements are defined besides the output power. During the last meeting discussion, the agreements is captured as below:

Agreement in RAN4#112:

- Requirements potentially needed:

  - Maximum output power

  - Transmitted signal quality (Frequency error, EVM)

  - Occupied bandwidth

  - Spectrum emission mask

  - Transmitter Intermodulation

  - ACLR

  - Operating band unwanted emissions

  - Transmitter spurious emissions

- FFS whether requirements are needed or not:

  - Output power dynamics

  - Transmit ON/OF power

  - Transmit ON/OFF time mask

  - IBE

- Requirements not needed:

  - Transmission times

Agreement in RAN4#112:

- Requirements potentially needed:

  - Reference sensitivity power level

  - Maximum input power

  - Receiver intermodulation

- FFS whether requirements are needed or not:

  - ACS

  - In-band blocking

  - Out-of-band blocking

  - Narrow band blocking

  - Receiver spurious response

  - Rx spurious emission

  - Requirements not needed:

  - ICS

For other requirements, it is summarized as below table 1:

Table 1 Summary of other TX requirements

| Output power dynamics | Potentially needed |
| --- | --- |
| Transmit ON/OF power | Potentially needed |
| Transmit ON/OFF time mask | Potentially needed |
| IBE | Needed to be further discussed together with co-existence |

Proposal 1: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

| Output power dynamics | Potentially needed |
| --- | --- |
| Transmit ON/OF power | Potentially needed |
| Transmit ON/OFF time mask | Potentially needed |
| IBE | Needed to be further discussed together with co-existence |

For the RX requirements, below table 2 is provided:

Table 2 Summary of other RX requirements’

| ACS | Needed to be further discussed together with co-existence |
| --- | --- |
| In-band blocking | Potentially Needed |
| Out-of-band blocking | Potentially Needed |
| Narrow band blocking | Potentially Needed |
| Receiver spurious response | Potentially Needed |
| Rx spurious emission | Needed |

Proposal 2: To agree on the following table for RX requirements:

Table 2 Summary of other RX requirements’

| ACS | Needed to be further discussed together with co-existence |
| --- | --- |
| In-band blocking | Potentially Needed |
| Out-of-band blocking | Potentially Needed |
| Narrow band blocking | Potentially Needed |
| Receiver spurious response | Potentially Needed |
| Rx spurious emission | Needed |

3	Conclusions

In this paper, we give further discussion on intermediate UE RF requirement the observations and proposals are as below:

Proposal 1: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

| Output power dynamics | Potentially needed |
| --- | --- |
| Transmit ON/OF power | Potentially needed |
| Transmit ON/OFF time mask | Potentially needed |
| IBE | Needed to be further discussed together with co-existence |

Proposal 2: To agree on the following table for RX requirements:

Table 2 Summary of other RX requirements’

| ACS | Needed to be further discussed together with co-existence |
| --- | --- |
| In-band blocking | Potentially Needed |
| Out-of-band blocking | Potentially Needed |
| Narrow band blocking | Potentially Needed |
| Receiver spurious response | Potentially Needed |
| Rx spurious emission | Needed |

4 References

[1] R4-2414305	WF on RF requirements for A-IoT, Huawei