3GPP TSG-RAN WG4 Meeting #112-bis						R4-2415601

Hefei, Anhui, China 14th – 18th October, 2024

Agenda Item:	6.3.2.2

Source: 	OPPO

Title: 	on intra-band NCCA MPR simulation results

Document for:	Approval

- Introduction

In RAN4#112 meeting, the intra-band NCCA MPR simulation has been discussed and a WF[1] has been agreed. Also we realized that some of the requirements are interpreted differently among companies. With further alignment, we have done more simulation and the simulation results are provided as well as some consideration on the IQ image and DC suppression.

- Discussion

During the last RAN4#112 meeting, the agreement for UE capabilities has been captured as below from the WF[1].

Issue 2-2-2: PSSCH/PSCCH with 2x20dBm PA + 1LO

Agreement:

- use PSSCH/PSCCH with 2x20dBm PA for PC3 sidelink non-contiguous CA evaluation.

The detail configuration can be found as below table 1.

Table 1, channel bandwidth and RB configuration

| Ca se | LC RB1 | RBST ART1 | LC RB2 | RBST ART2 | SC S | CB W1 | CB W2 | GA P | Ca se | LC RB1 | RBST ART1 | LC RB2 | RBST ART2 | SC S | CB W1 | CB W2 | GA P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 10 | 0 | 10 | 42 | 15 | 10 | 20 | 49 | 1 | 10 | 0 | 10 | 42 | 15 | 10 | 50 |
| 2 | 2 | 10 | 18 | 10 | 22 | 15 | 10 | 20 | 50 | 2 | 10 | 18 | 10 | 22 | 15 | 10 | 50 |
| 3 | 3 | 10 | 19 | 10 | 22 | 15 | 10 | 20 | 51 | 3 | 10 | 19 | 10 | 22 | 15 | 10 | 50 |
| 4 | 4 | 10 | 27 | 10 | 14 | 15 | 10 | 20 | 52 | 4 | 10 | 27 | 10 | 14 | 15 | 10 | 50 |
| 5 | 5 | 10 | 33 | 10 | 7 | 15 | 10 | 20 | 53 | 5 | 10 | 33 | 10 | 7 | 15 | 10 | 50 |
| 6 | 6 | 10 | 34 | 10 | 7 | 15 | 10 | 20 | 54 | 6 | 10 | 34 | 10 | 7 | 15 | 10 | 50 |
| 7 | 7 | 10 | 42 | 10 | 0 | 15 | 10 | 20 | 55 | 7 | 10 | 42 | 10 | 0 | 15 | 10 | 50 |
| 8 | 8 | 25 | 0 | 25 | 27 | 15 | 10 | 20 | 56 | 8 | 25 | 0 | 25 | 27 | 15 | 10 | 50 |
| 9 | 9 | 25 | 19 | 25 | 8 | 15 | 10 | 20 | 57 | 9 | 25 | 19 | 25 | 8 | 15 | 10 | 50 |
| 10 | 10 | 25 | 20 | 25 | 8 | 15 | 10 | 20 | 58 | 10 | 25 | 20 | 25 | 8 | 15 | 10 | 50 |
| 11 | 11 | 25 | 27 | 25 | 1 | 15 | 10 | 20 | 59 | 11 | 25 | 27 | 25 | 1 | 15 | 10 | 50 |
| 12 | 12 | 30 | 0 | 30 | 22 | 15 | 10 | 20 | 60 | 12 | 30 | 0 | 30 | 22 | 15 | 10 | 50 |
| 13 | 13 | 30 | 19 | 30 | 3 | 15 | 10 | 20 | 61 | 13 | 30 | 19 | 30 | 3 | 15 | 10 | 50 |
| 14 | 14 | 30 | 19 | 30 | 2 | 15 | 10 | 20 | 62 | 14 | 30 | 19 | 30 | 2 | 15 | 10 | 50 |
| 15 | 15 | 30 | 22 | 30 | 1 | 15 | 10 | 20 | 63 | 15 | 30 | 22 | 30 | 1 | 15 | 10 | 50 |
| 16 | 16 | 50 | 2 | 50 | 0 | 15 | 10 | 20 | 64 | 16 | 50 | 2 | 50 | 0 | 15 | 10 | 50 |
| 17 | 17 | 10 | 0 | 20 | 86 | 15 | 10 | 30 | 65 | 17 | 10 | 0 | 20 | 86 | 15 | 10 | 40 |
| 18 | 18 | 10 | 18 | 20 | 44 | 15 | 10 | 30 | 66 | 18 | 10 | 18 | 20 | 44 | 15 | 10 | 40 |
| 19 | 19 | 10 | 19 | 20 | 44 | 15 | 10 | 30 | 67 | 19 | 10 | 19 | 20 | 44 | 15 | 10 | 40 |
| 20 | 20 | 10 | 27 | 20 | 28 | 15 | 10 | 30 | 68 | 20 | 10 | 27 | 20 | 28 | 15 | 10 | 40 |
| 21 | 21 | 10 | 33 | 20 | 14 | 15 | 10 | 30 | 69 | 21 | 10 | 33 | 20 | 14 | 15 | 10 | 40 |
| 22 | 22 | 10 | 34 | 20 | 14 | 15 | 10 | 30 | 70 | 22 | 10 | 34 | 20 | 14 | 15 | 10 | 40 |
| 23 | 23 | 10 | 42 | 20 | 2 | 15 | 10 | 30 | 71 | 23 | 10 | 42 | 20 | 2 | 15 | 10 | 40 |
| 24 | 24 | 25 | 0 | 50 | 56 | 15 | 10 | 30 | 72 | 24 | 25 | 0 | 50 | 56 | 15 | 10 | 40 |
| 25 | 25 | 25 | 19 | 50 | 16 | 15 | 10 | 30 | 73 | 25 | 25 | 19 | 50 | 16 | 15 | 10 | 40 |
| 26 | 26 | 25 | 20 | 50 | 16 | 15 | 10 | 30 | 74 | 26 | 25 | 20 | 50 | 16 | 15 | 10 | 40 |
| 27 | 27 | 25 | 27 | 50 | 2 | 15 | 10 | 30 | 75 | 27 | 25 | 27 | 50 | 2 | 15 | 10 | 40 |
| 28 | 28 | 30 | 0 | 60 | 46 | 15 | 10 | 30 | 76 | 28 | 30 | 0 | 60 | 46 | 15 | 10 | 40 |
| 29 | 29 | 30 | 19 | 60 | 6 | 15 | 10 | 30 | 77 | 29 | 30 | 19 | 60 | 6 | 15 | 10 | 40 |
| 30 | 30 | 30 | 19 | 60 | 4 | 15 | 10 | 30 | 78 | 30 | 30 | 19 | 60 | 4 | 15 | 10 | 40 |
| 31 | 31 | 30 | 22 | 60 | 2 | 15 | 10 | 30 | 79 | 31 | 30 | 22 | 60 | 2 | 15 | 10 | 40 |
| 32 | 32 | 50 | 2 | 100 | 0 | 15 | 10 | 30 | 80 | 32 | 50 | 2 | 100 | 0 | 15 | 10 | 40 |
| 33 | 33 | 10 | 0 | 10 | 41 | 30 | 10 | 30 | 81 | 33 | 10 | 0 | 10 | 41 | 30 | 10 | 40 |
| 34 | 34 | 10 | 2 | 10 | 22 | 30 | 10 | 30 | 82 | 34 | 10 | 2 | 10 | 22 | 30 | 10 | 40 |
| 35 | 35 | 10 | 6 | 10 | 22 | 30 | 10 | 30 | 83 | 35 | 10 | 6 | 10 | 22 | 30 | 10 | 40 |
| 36 | 36 | 10 | 8 | 10 | 14 | 30 | 10 | 30 | 84 | 36 | 10 | 8 | 10 | 14 | 30 | 10 | 40 |
| 37 | 37 | 10 | 10 | 10 | 7 | 30 | 10 | 30 | 85 | 37 | 10 | 10 | 10 | 7 | 30 | 10 | 40 |
| 38 | 38 | 10 | 12 | 10 | 7 | 30 | 10 | 30 | 86 | 38 | 10 | 12 | 10 | 7 | 30 | 10 | 40 |
| 39 | 39 | 10 | 14 | 10 | 0 | 30 | 10 | 30 | 87 | 39 | 10 | 14 | 10 | 0 | 30 | 10 | 40 |
| 40 | 40 | 12 | 0 | 25 | 26 | 30 | 10 | 30 | 88 | 40 | 12 | 0 | 25 | 26 | 30 | 10 | 40 |
| 41 | 41 | 12 | 4 | 25 | 8 | 30 | 10 | 30 | 89 | 41 | 12 | 4 | 25 | 8 | 30 | 10 | 40 |
| 42 | 42 | 12 | 8 | 25 | 8 | 30 | 10 | 30 | 90 | 42 | 12 | 8 | 25 | 8 | 30 | 10 | 40 |
| 43 | 43 | 12 | 12 | 25 | 0 | 30 | 10 | 30 | 91 | 43 | 12 | 12 | 25 | 0 | 30 | 10 | 40 |
| 44 | 44 | 15 | 0 | 30 | 21 | 30 | 10 | 30 | 92 | 44 | 15 | 0 | 30 | 21 | 30 | 10 | 40 |
| 45 | 45 | 15 | 3 | 30 | 3 | 30 | 10 | 30 | 93 | 45 | 15 | 3 | 30 | 3 | 30 | 10 | 40 |
| 46 | 46 | 15 | 7 | 30 | 2 | 30 | 10 | 30 | 94 | 46 | 15 | 7 | 30 | 2 | 30 | 10 | 40 |
| 47 | 47 | 15 | 9 | 30 | 0 | 30 | 10 | 30 | 95 | 47 | 15 | 9 | 30 | 0 | 30 | 10 | 40 |
| 48 | 48 | 24 | 0 | 50 | 0 | 30 | 10 | 30 | 96 | 48 | 24 | 0 | 50 | 0 | 30 | 10 | 40 |

For intra-band non-contiguous CA, both PC3 and PC2 will be defined in Rel-19 timeline. Below we would like to discuss them separately.

2.2 PC3 intra-band non-contiguous CA

2PA 1LO architecture:

Figure 1 PC3 2PA 1LO architecture

We have found that in case 7, the LO has fall inside the GAP ACLR region and also the allocated RBs are quite near the GAP ACLR region. In such case, even with small LCRB length, still the MPR are quite large. For -13dBm/MHz, the MPR range from 5 to 10dB and for -30dBm/MHz range, the MPR range from 7 to 10dB.

Observation 1: The MPR needed is large in case the LO has fall inside the GAP ACLR region and also the allocated RBs are quite near the GAP ACLR region.

Observation 2: For PC3 2PA 1LO architecture, for -13dBm/MHz, the MPR range from 5 to 10dB and for -30dBm/MHz range, the MPR range from 7 to 10dB.

2PA 2LO architecture:

Figure 2 PC3 2PA 2LO architecture

It is quite obvious when 2LO applies, the LO and IQ image influence are quite small and hence much better MPR for 2LO compared to 1LO can be seen from figure 2. The MPR range is from 2dB to 8dB for -13dBm/MHz and from 5 dB to 9dB for -30dBm/MHz.

Observation 3:For the PC3 2PA 2LO architecture The MPR range is from 2dB to 8dB for -13dBm/MHz and from 5 dB to 9dB for -30dBm/MHz.

1PA 1LO architecture:

For the 1PA 1LO architecture, the MPR are from 2dB to 12dB for -13dBm/MHz and 2dB to 14dB for -30dBm/MHz. We observe such large MPR required and further in some cases such as case 7, 23 and 39, due to the non-linearity and the RB allocation, only apply MPR cannot help to solve the ACLR problem and this might need further study.

Observation 4: For the PC3 1PA 1LO architecture, the MPR are from 2dB to 12dB for -13dBm/MHz and 2dB to 14dB for -30dBm/MHz.

Observation 5: In some cases such as case 7, 23 and 39, due to the non-linearity and the RB allocation, only apply MPR cannot help to solve the ACLR problem and this might need further study.

Figure 3 PC3 1PA 1LO architecture

In the NR CA MPR requirement definition, the granularity is around 1 to 1.5dB and similar approach can apply to NR SL intra-band NCCA at least for 2PA 2LO and 2PA 1LO architecture but for 1PA 1LO architecture, further study might be needed.

2.3 PC2 intra-band non-contiguous CA

2PA 1LO architecture:

Figure 4 PC2 2PA 1LO architecture

The MPR is from 3dB to 13dB for -13dBm/MHz and 4dB to 15dB for -30dBm/MHz. For some cases as case 7, 23. 39 similar problem as PC3 has occurred.

Observation 6: For PC2 2PA 1LO architecture, The MPR is from 3dB to 13dB for -13dBm/MHz and 4dB to 15dB for -30dBm/MHz.

2PA 2LO architecture:

Figure 5 PC2 2PA 2LO architecture

For 2PA 2LO architecture, the MPR is from 2dB to 8dB for -13dBm/MHz and 3 to 10dB for -30dBm/MHz.

Observation 7: For 2PA 2LO architecture, the MPR is from 2dB to 8dB for -13dBm/MHz and 3 to 10dB for -30dBm/MHz.

1PA 1LO architecture:

Figure 6 PC2 1PA 1LO architecture

For 1PA 1LO architecture, the MPR is from 4dB to 11dB for -13dBm/MHz and 5 to 12dB for -30dBm/MHz.

Observation 8: For 1PA 1LO architecture, the MPR is from 4dB to 11dB for -13dBm/MHz and 5 to 12dB for -30dBm/MHz.

For PC2 intra-band NCCA, the problem is quite similar to PC3. For some cases whose RB allocation are quite near to the GAP region, the MPR is very hard to achieve a proper number.

3	Conclusions

In this paper, we give further discussion on PC3 and PC2 intra-band NR SL NCCA MPR simulation, the observations and proposals are as below:

Observation 1: The MPR needed is large in case the LO has fall inside the GAP ACLR region and also the allocated RBs are quite near the GAP ACLR region.

Observation 2: For PC3 2PA 1LO architecture, for -13dBm/MHz, the MPR range from 5 to 10dB and for -30dBm/MHz range, the MPR range from 7 to 10dB.

Observation 3:For the PC3 2PA 2LO architecture The MPR range is from 2dB to 8dB for -13dBm/MHz and from 5 dB to 9dB for -30dBm/MHz.

Observation 4: For the PC3 1PA 1LO architecture, the MPR are from 2dB to 12dB for -13dBm/MHz and 2dB to 14dB for -30dBm/MHz.

Observation 5: In some cases such as case 7, 23 and 39, due to the non-linearity and the RB allocation, only apply MPR cannot help to solve the ACLR problem and this might need further study.

Observation 6: For PC2 2PA 1LO architecture, The MPR is from 3dB to 13dB for -13dBm/MHz and 4dB to 15dB for -30dBm/MHz.

Observation 7: For 2PA 2LO architecture, the MPR is from 2dB to 8dB for -13dBm/MHz and 3 to 10dB for -30dBm/MHz.

Observation 8: For 1PA 1LO architecture, the MPR is from 4dB to 11dB for -13dBm/MHz and 5 to 12dB for -30dBm/MHz.

4 References

[1] R4-2414312	WF on NR sidelink for intra-band non-contiguous CA, OPPO