3GPP TSG-RAN WG4 Meeting#112-bis		R4-2415609

Hefei, China, Oct 14th -18th, 2024

Agenda item:	6.26.2.1

Source: 	Spreadtrum Communications

Title: 	Discussion on NTN (e)Redcap UE RF requirements

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

REFSENS for NTN HD-FDD Redcap

Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement

According to the agreement, we can start the discussions on the RF requirements for NTN Redcap with HD-FDD. The one important of RF requirements is REFSNES.As we all know, the REFSENS requirements for NR FDD and NR NTN band are band dependent.

We can follow R17 Redcap REFSENS, adopt the same scale for NTN Redcap REFNSENS requirements.

Table1: HD-FDD REFSENS tightening from FD-FDD for NR

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -94.2 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

According to the table1, and Table 7.3.2-1, it can be found that n256 for 2RX HD-FDD Redcap REFSENS could be tightening 0.5 dB compared with 2RX FD-FDD REFSENS, n255 for 2RX HD-FDD Redcap REFSENS could be tightening 0 dB compared with 2RX FD-FDD REFSENS. Based on the amylase above, it is proposed:

Proposal 1: Adopting the following REFSENS for NTN HD-FDD Redcap UE:

- n255: Existing n255 2RX FD-FDD RESENS-0 dB

Maximum output power

As is known to all, FD-FDD with duplex filter. If HD-FDD with two simple filters, one for TX and the other for RX, there is no obvious difference in insertion loss (e.g., 0.2dB) between duplex filter and simplex filter. If HD-FDD reuses the duplex filter to save cost, there is no reason to increase the maximum output power. If HD-FDD with no filter for TX, the insertion loss could be reduced by 1dB or more. Obviously, the reduction of insertion loss is up to UE implementation. Therefore, we should keep PC3 power level is the same for non-Redcap NR NTN UE and (e)Redcap NR NTN UE. However, some companies proposed that the tolerance of PC3 for HD-FDD bands should be modified. According to current spec, tolerance of PC3 for TN (e)Redcap NR UE is the same as tolerance of PC3 for TN normal NR UE. Hence, in our view, we should still keep the same tolerance of PC3 for HD-FDD NTN (e)Redcap UE.

Proposal 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE.

Proposal 3: Keep the same tolerance of PC3 for HD-FDD NTN (e)Redcap UE.

Simultaneous operation with GNSS

## Issue 1-4: Simultaneous operation with GNSS Agreement: Further study NR NTN UE simultaneous operation with GNSS considering e.g. - Operating bands with UL close to GNSS frequencies (n254, n255) - Emissions on GNSS frequencies - Whether transmission gaps enabling successful GNSS reception are present - Potential impact on field performance - Potential need for specification updates, e.g. frequency error

In the last meeting, an open issue was identified that the range of GNSS frequency from 1559 to 1610MHz for NR NTN Redcap UE(n254,n255) cannot be simultaneous operation with GNSS.As we all known that, NTN (e)Redcap UE is a lower device which is similar to IoT NTN. In IoT NTN spec, it introduces GNSS gap in order to solve the issue that cannot be simultaneous operation with GNSS due to shared component for these two functions as follows. In the same way, GNSS gap also can be introduced in NR NTN (e)Redcap spec. Of course, it is necessary to trigger LS to RAN1 and RAN2 in order to get involved in this issue.

## 16.10 GNSS measurement gap related procedures For a NB-IoT UE in a NTN FDD serving cell, when the UE receives a GNSS Measurement Command MAC CE in a NPDSCH ending in DL subframe n, - if the UE shall not provide HARQ-ACK information for the HARQ process associated with the transport block in the NPDSCH carrying GNSS Measurement Command MAC CE, - the UE shall assume the start of the measurement gap in subframe n+13 - otherwise, - the UE shall assume the start of the measurement gap in subframe k+2, where k is the first DL subframe after the end of the transmission of the NPUSCH carrying ACK/NACK response for the HARQ process associated with the transport block in the NPDSCH. For a NB-IoT UE in a NTN FDD serving cell, the UE is not required to monitor NPDCCH within the GNSS measurement gap duration, until it reacquires GNSS position and a contention based Random Access is performed as specified in TS 36.321 [8].

Observation1: IoT NTN introduces GNSS gap to enable successful GNSS reception.

Proposal 4: GNSS gap could be introduced for NTN (e)Redcap UE.

Proposal 5: It is necessary to trigger LS to RAN1 and RAN2 in order to get involved in this issue.

Conclusions

In this paper, we provide our views on on NTN (e)Redcap UE RF requirements, we have made the following

proposals.

Proposal 1: Adopting the following REFSENS for NTN HD-FDD Redcap UE:

- n255: Existing n255 2RX FD-FDD RESENS-0 dB

Proposal 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE.

Proposal 3: Keep the same tolerance of PC3 for HD-FDD NTN (e)Redcap UE.

Observation1: IoT NTN introduces GNSS gap to enable successful GNSS reception.

Proposal 4: GNSS gap could be introduced for NTN (e)Redcap UE.

Proposal 5: It is necessary to trigger LS to RAN1 and RAN2 in order to get involved in this issue.

- Reference

- R4-2413527, WF on NR_NTN_Ph3_UERF, Qualcomm INC.TSG-RAN4#112 meeting