3GPP TSG-RAN WG4 Meeting#112bis		R4-2415611

Hefei, China, Oct 14th -18th, 2024

Agenda item:	6.23.2.1

Source: 	Spreadtrum Communications

Title: 	Discussion on systems parameters for LP-WUR

Document for:	Discussion

Introduction

In RAN4#112meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

Performance metric for Rx RF requirements

Issue 1-1-2: Performance metric for Rx RF requirements Agreement: - 1% MDR for evaluation [and core requirements] - FFS on the testability issue.

In last meeting, some companies proposed that using higher MDR (e.g. 5%) for testing requirements compared with 1% MDR for core requirements in order to save testing cost and complexity. If we define different value for core requirements and testability requirements, especially lower MDR for testability requirements. Assume the MDR value of testing requirements is 5% and the MDR value of core requirements is 1%.In such conditions, even though terminal pass the testing requirements, how can we ensure that the terminal must meet the core requirements? If we can’t guarantee that the terminal meets core requirements, the definition of core requirements will be meaningless. In our understanding, MDR value of core requirements should be the same as testing requirements. In addition, from the point of view of system performance, higher MDR maybe means lower power consumption and overhead. Thus, we can reuse the existing PDCCCH requirements. The performance metric is 1% probability of miss-detection of the downlink scheduling grant for PDCCH, thus we could reuse 1% MDR is suitable for LP-WR RX RF requirements.

Proposal 1: MDR value of core requirements and testing requirements should be same.

Proposal 2: Use 1% MDR as the metric for LP-WUR RX RF requirements.

Channel raster for LP-WUR

Issue 1-2-1: Channel raster for LP-WUR Agreements: - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS. - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful.

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The centre of the channel bandwidth configured by gNB for UE should be aligned with the channel raster. Therefore, it is necessary for gNB to know the channel raster. According to the previous agreement, we focus on IMT bands. No matter whether LR and MR use the same frequency band or different bands, these bands belong to the IMT bands. Based on the above analysis, gNB could know the channel raster based on existing channel raster for IMT bands. Thus, the existing channel raster could be reused for LP-WUR. No new channel raster is needed for LP-WUR. In addition, from RAN1’s perspective, the same grid of serving cells can be shared by MR and LP-WUR. Hence, so no new channel raster is needed for LP-WUR.

Proposal 3: Existing channel raster could be reused for LP-WUR, no new channel raster is needed for LP-WUR.

With regard to obtaining the frequency domain position of LP-WUS, the indication process of RAN1 is as follows. It is similar to the indication process of CORESET0.By indicating the starting position of LP-WUS frequency domain and an offset of the lower boundary of LP-WUS frequency domain in SIB1, the frequency domain position of LP-WUS can be obtained. The frequency position of LP-WUS is composed of the initial RB position and the number of RB.

Observation 1: RAN1 does not indicate the centre operation frequency of LP-WUS.

Observation 2: The frequency position of LP-WUS is composed of the initial RB position and the number of RB, which is indicated by SIB1.

Conclusions

In this paper, we provide our views on system parameters for LP-WUR, we have made the following proposals and observations.

Proposal 1: MDR value of core requirements and testing requirements should be same.

Proposal 2: Use 1% MDR as the metric for LP-WUR RX RF requirements.

Proposal 3: Existing channel raster could be reused for LP-WUR, no new channel raster is needed for LP-WUR.

Observation 1: RAN1 does not indicate the centre operation frequency of LP-WUS.

Observation 2: The frequency position of LP-WUS is consist of initial RB which is indicated by SIB1 and RB number.

- Reference

- R4-2414309, WF on LP-WUS UE RF requirements, vivo, TSG-RAN WG4 112 meeting