3GPP TSG-RAN WG4 Meeting #112bis	  R4-2415621

Hefei, China, October 14th – 18th, 2024

Agenda Item:	6.17.2.1

Source:	Huawei, HiSilicon

Title:	Discussion on testability and interoperability issues in AIML CSI compression

Document for:	Discussion and Decision

- Introduction

According to WF in RAN4 #112 [1], we continue to discuss the remaining issues related to the test for AIML enabled CSI feedback enhancement.

- Test metric for AI CSI compression

TR 38.843: For metrics for CSI requirements/tests, the following test metrics are identified: - Option 1: Throughput/relative throughput - Option 2: SGCS, NMSE - Option 3: CSI prediction accuracy Option 1 should be used as baseline. For option 3, further discuss is needed on the feasibility to define the CSI prediction accuracy in WI. For metrics for CSI monitoring, further discussion is needed in WI.

The motivation of introducing AIML-enabled CSI compression is to achieve performance gain compared to legacy Type II codebook-based PMI reporting. Since the SGCS and NMSE cannot directly reflect the eventual throughput gain, Option 2 can be deprioritized for defining baseline requirements.

However, the SGCS/NMSE can be used as an intermediate result for aligning the test encoder/decoder or reference encoder/decoder.

Proposal 1: Use throughput as test metric for AIML-enabled CSI compression.

- Simulation results based on refined parameters

According to [1] and [2], the parameters are aligned as follows. Parameter/Configurations; Options/Values. Channel/data-set related. UE antenna element assumptions; Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; Aim to use the dataset around 600K. At least 100K sample to be used.. Dataset quantization; Float32. Other data-set characteristics/statistics; Companies to propose. Model-related. Quantization of encoder output; - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; Not to use drop-out layers. Training loss function, use of regularization; Use (1-SGCS) as loss function as reference. Optimizer; Use Adam. Learning rate; Fixed learning rate of 10^-4. Batch size; 256. (Max) Number of epochs; 150 epochs. Stopping criteria; No improvement over 25 epochs.. Weight initialization; Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); Default.

Based on the refined simulation parameters in [1] and [2], RAN4 will perform a round of alignment on the SGCS performance.

From our simulations, the SGCS performance of MLP, CNN and Transformer under the refined parameters are listed as below.

Table 1: SGCS performance of MLP, CNN and Transformer

|  | MLP | CNN | Transformer |
| --- | --- | --- | --- |
| AI | 0.71 | 0.70 | 0.71 |
| R16 Type-II | 0.66 |  |  |

It is observed that MLP and Transformer are slightly better than CNN in terms of SGCS.

Observation 1: MLP is superior to CNN in terms of SGCS.

The model structures of MLP and CNN are shown in the following figure, according to RAN4 WF and post email discussion.

MLP: CNN:

The complexity of MLP and CNN compared to R16 eType-II are analyzed as follows.

Table 2: Complexity of MLP, CNN and Transformer

|  | MLP | CNN | Transformer |
| --- | --- | --- | --- |
| AI | 6 MFLOPs | 94 MFLOPs | 23MFLOPs |
| R16 Type-II | 0.37 MFLOPs |  |  |

Since we have no aligned model structure of Transformer, the model structure of the evaluated Transformer in Table 1 and Table 2 are shown in the following.

Transformer:

It is observed that the complexity of MLP is lower than that of CNN and Transformer.

Observation 2: The complexity of MLP is lower than that of CNN.

We understand that at current stage, the feasibility is evaluated and the model under investigated will not be reused or has to be revised for defining requirement if AI CSI turns to WI. From simulation results, it is observed that the complexity of AI is much higher than that of R16 eType-II. Different from the purpose of RAN1 study, the complexity of the reference model has to be considered in RAN4 for defining requirements.

Proposal 2: Principle of reference model design – complexity of reference model is one of the metrics for selecting reference model structure.

For reducing the workload in the feasibility verification stage, the model structures of encoder and decoder are assumed to be the same. While for defining requirements, there is no need to follow this design. For example, we can use MLP for encoder, while CNN/Transformer for decoder.

Proposal 3: Different model structures of encoder and decoder can be considered for reference model design, e.g., MLP for encoder while Transformer for decoder.

- Feasibility verification of potential testing Option 3 and Option 4

RAN4 #112 WF [1]: Option 3 next steps - Check on performance alignment -> see simulation results from contributing companies - repeat simulations until good alignment is achieved - move to next step after alignment - Share models(encoder or decoder or both)/datasets (training/testing/inference) - framework to share data among companies(dataset/model) to be discussed - Select one or more decoder for further analysis - selection criteria to be discussed - Each company brings results for training of “own encoder” with selected decoder(s) - performance alignment to be checked/discussed - using “own” data or data shared by other companies - Conclude on overall feasibility of Option 3 - consider the conditions under which Option 3 is feasible if found feasible - FFS on what the feasibility criteria Note: some steps could be reused for Option 4, FFS on what can be reused

It is observed that we are at step 1 where simulations are performed with refined parameters. If the good alignment is achieved, we can move forward. However, to what extent the results are aligned is defined as a good alignment has not be discussed yet.

There are two options for defining the good alignment.

- Option 1: The SGCS margin is within x

  - FFS: the value of x

- Option 2: The throughput margin is within y

  - FFS: the value of y

Proposal 4: Two options are identified for defining the alignment in step 1.

- Option 1: The SGCS margin is within x

  - FFS: the value of x

- Option 2: The throughput margin is within y

  - FFS: the value of y

Since the eventual goal is to verify whether RAN4 can achieve an aligned AI model for defining requirement of AI-CSI compression, the model that we are trying to align should fulfill the baseline requirement regarding to RAN4 test metric. Otherwise, even if we can achieve an aligned AI model, the feasibility can still not be justified because the aligned model cannot be used for requirement definition. Therefore, we suggest to consider the throughput performance gain at step 1 to avoid a waste of time.

Proposal 5: Take throughput performance gain into account for defining the criteria of overall feasibility for potential testing options.

RAN4 #112 WF [1]: Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

From our understanding, the alignment work is the same between option 3 and option 4. The difference between these two options is that the decoder is specified in option 3 while encoder or the dataset derived from the pair of models is specified in option 4. Therefore, the feasibility of option 4 is the same as that of option 3. We can first verify the feasibility of aligning a pair of models which can fulfil RAN4 requirement and then decide which part would be specified in RAN4 if AI CSI compression turns to WI.

Observation 3: The feasibility of option 4 is the same as that of option 3.

Proposal 6: If the overall feasibility of potential testing options can be verified, RAN4 will further discuss which part of the model would be specified if AI CSI compression turns to WI.

Conclusions

According to the discussion, following proposals and observations are provided:

Proposal 1: Use throughput as test metric for AIML-enabled CSI compression.

Observation 1: MLP is superior to CNN in terms of SGCS.

Observation 2: The complexity of MLP is lower than that of CNN.

Proposal 2: Principle of reference model design – complexity of reference model is one of the metrics for selecting reference model structure.

Proposal 3: Different model structures of encoder and decoder can be considered for reference model design, e.g., MLP for encoder while Transformer for decoder.

Proposal 4: Two options are identified for defining the alignment in step 1.

- Option 1: The SGCS margin is within x

  - FFS: the value of x

- Option 2: The throughput margin is within y

  - FFS: the value of y

Proposal 5: Take throughput performance gain into account for defining the criteria of overall feasibility for potential testing options.

Observation 3: The feasibility of option 4 is the same as that of option 3.

Proposal 6: If the overall feasibility of potential testing options can be verified, RAN4 will further discuss which part of the model would be specified if AI CSI compression turns to WI.

References

- R4-2410570, “WF on AI/ML”, 3GPP RAN4 #112, May 20th – 24th, 2024.

- R4-2414447, “Alignment of model training and data set parameters for AI/ML-based CSI compression”, 3GPP RAN4 #112, May 20th – 24th, 2024.