3GPP TSG-RAN WG4 Meeting #112bis	  R4-2415625

Hefei, China, October 14th – 18th, 2024

Agenda Item:	6.17.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on testability and interoperability issues in AIML Beam management

Document for:	Discussion and Decision

- Introduction

According to WF in RAN4 #112 [1], we continue to discuss the remaining issues related to the test for AIML enabled beam management.

- Test setup for AI-BM

RAN4 #112 WF [1]: Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

For AI BM Case 1 and Case 2, measured RSRP is the model input. Both AoA and AoD are not considered as the model input. Therefor in RAN4, only the RSRP related to different beam needs to be emulated.

Observation 1: AoA and AoD are not used as model input in AI BM Case 1 and Case 2.

Proposal 1: Use TDL as the channel model for AI BM testing.

RAN4 #112 WF [1]: Issue 2-4: Test setup needs Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered

The selection of the number of set A/B Tx beams need to take account of the configuration of set A/set B from other WGs. RAN4 can wait more progress of RAN1.

RAN4 #112 WF [1]: Issue 2-6: Measurement error impact Agreement: RAN4 will study the impact of measurement error for the input to inference and dataset for training(set B measurement error) on prediction accuracy, companies should bring proposals on how to proceed with such a study. impact on different prediction accuracy metrics should be taken into account studies already performance by other groups(RAN1) could also be taken into account Note: intention of the study is just to understand the impact of measurement error on prediction accuracy.

In practical, only the measured RSRP rather than the ideal RSRP can be obtained by DUT. Therefore, the measurement error should be considered when defining the requirement of prediction accuracy. RAN4 can first evaluate the relationship between SNR and RSRP measurement error. Then measurement error will be captured in the model input during model training. With these assumptions, companies can perform simulations for alignment and requirement definition.

Proposal 2: To consider the measurement error impact, the relationship between SNR and RSRP measurement error can be used to generate model input during model training.

It is observed that the performance of the model may also be impacted by other factors, including the gNB-side beampattern (which is gNB-side proprietary information), the channel conditions (e.g., the scatter in the physical environment), the receiving gain at UE (i.e., different UE implementation, UE rotation). Therefore, whether and how to define a common training/testing dataset is an open issue.

If the training dataset is not specified, how to ensure the consistency between training and testing needs to be studied. One possible solution is to let the DUT to collect training dataset and complete the model training, followed by performing model test. However, according to R18 study outcome, the training procedure is not considered in RAN4 test due to many reasons, such as test cost/time.

Even if RAN4 revisits the training procedure, how to ensure the following principle identified in R18 is also an open issue:

The design of test should ensure performance is guaranteed and avoid that a UE can pass the test but perform poorly in the field. [2]

This is due to the reason that the parameters considered in the test set up which limit the model generalization may be totally different from real deployment. In this case, a DUT has an AI model to pass RAN4 test does not mean it can work well in the field.

Proposal 3: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to avoid that a UE can pass the test but performs poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

After achieving clear answers of the aforementioned questions, we can start to discuss how to define the setup, where the exiting FR2 OTA test set up should be considered as the baseline. Any enhancements on top of existing fancy enough framework should be justified.

Proposal 4: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

- Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: AoA and AoD are not used as model input in AI BM Case 1 and Case 2.

Proposal 1: Use TDL as the channel model for AI BM testing.

Proposal 2: To consider the measurement error impact, the relationship between SNR and RSRP measurement error can be used to generate model input during model training.

Proposal 3: Before investigating how to set up the test environment, the following questions have to be answered:

- How to ensure the consistency between model training and model testing

  - Whether and how to define a common training/testing dataset.

    - If define, how to capture the different UE implementations and UE behavior when constructing the dataset

  - Whether UE is expected to collect training dataset and train the model before performing model performance test.

    - If yes, how to resolve the test cost/time issue.

- How to avoid that a UE can pass the test but performs poorly in the field, considering that some parameters used in the test set up which limit the model generalization may totally be different from that in real deployment.

Proposal 4: For test set up in AI-BM, taking the existing FR2 OTA test set up as baseline, any enhancements on top of which should be justified.

References

- R4-2414323, “WF on AI/ML”, 3GPP RAN4 #112, 19th – 23rd August, 2024.