3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415626

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on RRM core requirements in AIML Positioning

Source: 	Huawei, HiSilicon

Agenda item:	6.17.5.1

Document for:	Discussion

Introduction

RRM core requirements for AI/ML based positioning are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the earlies discussion, the RRM requirements are mainly considered for data collection, model inference and LCM related requirements. Our views on requirements for model inference are provided in the companion paper on testability and interoperability issues in AI/ML based positioning.

In this paper, we will provide our views on data collection and LCM related requirements for AI/ML based positioning.

Discussion

Data collection

In RAN1#118 the following agreements are reached related to data collection.

Agreement For training data collection of AI/ML based positioning case 3b, for time stamp of channel measurement, - For channel measurement generated by TRP/gNB, existing IE “Time Stamp” in TS 38.455 can be reused from RAN1 perspective - Note: Purpose, such as above “training data collection", will not necessarily be specified in RAN 1 specifications Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Data collection function provides input data to model training, management and inference function. If new measurement metric is introduced in other WGs, new requirements can be considered accordingly, otherwise we do not see the need for new RRM requirements on data collection. Based on the RAN1 progress so far, we have not observed any RRM impact.

Proposal 1: RAN4 to wait for RAN1 conclusion before discussing new RRM requirements for data collection for AI/ML positioning.

LCM related requirements

In RAN1#118 the following agreements are reached related to performance monitoring.

Agreement For AI/ML positioning Case 3a, for performance monitoring metric calculation in label-based monitoring, from RAN1 perspective, Option A and Option B are feasible, - Option A. NG-RAN node performs monitoring metric calculation for its own model. - Option B. LMF performs monitoring metric calculation for the model located at the NG-RAN node. Note: Final selection of Option A and Option B is out of RAN1 scope. Potential support of Option A and/or Option B is pending RAN3 confirmation. Note: Exact method to perform monitoring metric calculation is up to implementation. Note: For Option A, RAN1 assumes that user data privacy needs to be preserved. Conclusion For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - Option A-4 can be realized by implementation in a manner transparent to specification specification if the PRU sends information to the target UE side in a proprietary method. No further discussion on Option A-4.

In LCM procedure, the functionality/model activation/deactivation/fallback to non-AI function are to be considered. If the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., triggered by LMF via LPP signaling, the delay of activating/deactivating one function can be defined. Otherwise if these functionality/model changes are autonomously performed by the assisting node (UE or gNB) with AL/ML model, the assisting node would need to monitor the function performance, then the requirements would be related with performance monitoring requirements.

If performance monitoring is mainly executed at the assisting node, some metrics are to be defined so that UE itself can make decision of activation, deactivation, or fallback to non-AI. Then from RRM requirement perspective, the requirements of monitoring periodicity or monitoring delay when certain criterion is met may need to be specified.

Based on RAN1 progress so far, there are many open schemes for performance monitoring. Similar to other use cases, whether and how to define requirements for performance monitoring depends on the selected schemes which will be defined separately for each positioning sub-case. Also, it may depend on what kind of requirements are defined for inference for each positioning sub-case.

Proposal 2: RAN4 to discuss requirements for LCM procedure especially performance monitoring for AI/ML positioning, based on RAN1 conclusion on performance monitoring schemes and also RAN4 conclusion on the requirements for inference.

Conclusions

In this paper we provided provide our views on data collection and LCM related requirements for AI/ML based positioning.

Proposal 1: RAN4 to wait for RAN1 conclusion before discussing new RRM requirements for data collection for AI/ML positioning.

Proposal 2: RAN4 to discuss requirements for LCM procedure especially performance monitoring for AI/ML positioning, based on RAN1 conclusion on performance monitoring schemes and also RAN4 conclusion on the requirements for inference.

Reference

- R4-2414323, WF on AI/ML, Qualcomm Incorporated.