3GPP TSG-RAN WG4 Meeting #112bis	R4-2415629

# Hefei, China, Oct 14th – 18th, 2024

Agenda item:			4.4.1

Source:	Huawei, HiSilicon

Title:	Updated simulation assumptions for introduction of FR2 Power Class 8

Document for:	Approval

- Introduction

Way forward R4-2414420 [1] on introduction of new FR2 PC8 is approved in last RAN4#112 meeting. Updated simulation assumptions are provided in this contribution to correct typos and add other more suitable assumptions for evaluation.

- System simulation assumptions

Table 1: FR2 new PC8 UL/DL System Simulation Assumptions

| Parameters | Value |
| --- | --- |
| Scenario | UMa |
| Site layout |  |
| UE deployment | 100% Outdoor |
| Pathloss model | TR 38.901 Table 7.4.1-1: Pathloss models |
| LOS Probability | TR 38.901 Table 7.4.2-1 LOS probability |
| Traffic type | UL/DL: Full buffer DL: FTP Model 3 (4Mb payload every 200ms) UL: FTP Model 3 (1Mb payload every 200ms) |
| Scheduling type | Proportional Fair |
| CQI/MCS table | 256QAM |
| BS antenna heights (m) | 25 |
| UT antenna heights (m) | 1.5 |
| h_E (m) | 1 |
| Number of UE | 10 per TRxP |
| SCS( kHz) | 120 |
| Carrier frequency (GHz) | 26 |
| Occupied channel bandwidth for PDSCH Per CC (MHz) | [100 for comparison], 200, 400 |
| Number of CC | 1 |
| occupied PRB Per CC | [66],132, 264 |
| ISD(m) | 100,150,200 |
| max layer | 2 |
| TDD pattern | DDDSU with S = 10:2:2 |
| DL Symbols with PDCCH | 1 |
| DM-RS configuration | Type 1: 1+1 (DL: Multiplexing with PDSCH, UL: Non- multiplexing with PUSCH) |
| xOverhead | 6 |
| Max transmission number | 4 |
| UL Codebook type | ULCB |
| DL codebook type | SRS based codebook selection |
| BS antenna radiation pattern [NOTE 1] | UPA, [m,n,p,mg,ng,mp,np] = [16,16,2,1,1, 1,1] [dh,dv] = [0.5λ, 0.5λ], beam_theta = [90:5:155]° beam_phi = [-55:5:55]° polar_angle = [45, -45] element gain_pattern: General3D = { theta_3db = 65.0°, phi_3db = 65.0°, sla_v = 30.0dB, a_m = 30.0dB, max_element_gain = 8.0dBi } |
| UE antenna radiation pattern [NOTE 1] | UPA = { [m, n, p, mg, ng, mp, np] = [2, 1, 2, 1, 1, 1, 1], [dh,dv] = [NA, 0.5λ,], beam_theta = [90.0]°, beam_phi = [0.0]° } polar_angle = [0, 90]° element gain pattern: General3D = { theta_3db = 90.0°, phi_3db = 90.0°, sla_v = 25.0dB, a_m_db = 25.0dB, max_element_gain = 5.0dBi } |
| UE EIRP (dBm) | 16.4 |
| BS EIRP (dBm) | 70 |
| Thermal noise density (dBm/Hz) | -174 |
| BS Noise figure (dB) | 7 |
| UE Noise figure (dB) | 10 |
| UE velocity | 3km/h |
| UL Power control parameter | P0 = -86, alpha = 0.9 |
| Criteria for selection for serving TRxP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered |
| Criteria for analog beam selection for serving TRxP | Select the best beam pair among the limited set of DFT analog beams, based on the criteria of maximizing receive power after beamforming. |
| Criteria for analog beam selection for interfering TRxP | Based on the analog beam selection according to scheduling results of non-serving TRxP |
| NOTE 1: Refer to ITU R-REP-M.2412-2017-PDF-E Table 9 and Table 11 respectively |  |

- Reference

- R4-2414420 WF on introduction of new FR2 PC8, RAN4#112, Huawei, HiSilicon