3GPP TSG-RAN WG4 Meeting #112bis	R4-2415633

# Hefei, China, 14th – 18th Oct., 2024

Title: 	Discussion on NTN testing for NGSO channel model

Source: 	Huawei, HiSilicon

Agenda item:	6.8.4

Document for:	Discussion

Background

During RAN4#112 meeting, Way Forward [1] for NTN_testing_NGSO_channel_model was approved. In this contribution, we share our view on NTN testing for NGSO channel model.

Discussion

Methodology for Time varying Doppler and Delay shifts modelling

- Agreement - RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options: - Option 1: Eckstein-Hechler model - Option 2: Keplerian model - Option 3: Existing model from TR 38.811 5.3.4.3 - Option 4: Proposal 2 from R4-2412552 - Option 5: Proposal 2 from R4-2412783 - Other options not precluded Note: Companies are encouraged to bring more information for option 1 and option2. - Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model - FFS on the initial ephemeris information and parameters

Here we provide the simulation results for different channel models comparing to the predefined files. Eckstein-Hechler model, Keplerian model and circular model are evaluated. Since the epoch is indicated by SFN, the maximum interval between two different ephemeris is limited by 10.24s. In the following Figure 2.1-1, the ephemeris is transmitted with the period of 10.24s using the information indicated in the predefined files. We can observe that either Eckstein-Hechler model or Keplerian model can achieve rather small Doppler and delay error comparing to the circular model.

Figure 2.1-2 Ephemeris is with the period of 10.24s

For Eckstein-Hechler model and Keplerian model, the maximum Doppler error can be about 5Hz and the maximum delay error can be about 1.5e-8s while ephemeris is transmitted with the period of 10.24s.

For circular model, the maximum Doppler error can be about 10Hz and the maximum delay error can be about 8e-7s while ephemeris is transmitted with the period of 10.24s.

To judge whether the channel model is accurate enough, some criteria should be established. In our view, a channel model is OK to say accurate enough if the Doppler error and the delay error not exceed the given tolerance (e.g., far less than frequency error requirements 0.1 ppm = 200Hz@2GHz and timing error requirements 29*64*Tc = 9.44e-7s) within at most 10.24s.

The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Table 7.1C.2-1: Te_NTN Timing Error Limit Frequency Range; SCS of SSB signals (kHz); SCS of uplink signals (kHz); Te_NTN. FR1-NTN; 15; 15; 29*64*Tc. 30; 24*64*Tc. 60; N/A. 30; 15; 24*64*Tc. 30; 22*64*Tc. 60; N/A. Note 1: Tc is the basic timing unit defined in TS 38.211 [6].

The Doppler error and the delay error brought by the channel model should not exceed the given tolerance, i.e. frequency error and timing error shall be less than 5% of the 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s respectively within at most 10.24s.

Based on above analysis, either Eckstein-Hechler model or Keplerian model is accurate enough. However, from TE implementation point of view, Keplerian model is simpler than Eckstein-Hechler model. To simplify the test, we propose to consider Keplerian model for NTN testing for NGSO.

Consider Keplerian model for NTN testing for NGSO.

Parameters for TE-emulated channel model

- Agreement - RAN4 further discuss the necessary parameters with associated channel model, following parameters list for information purpose: - Satellite altitude: - Option 1: LEO -600km only - Option 2: Both LEO-600km and LEO-1200km Elevation angle range - UE position - Whether to consider a non-zero time-varying feeder link delay - Other parameters not precluded

Based on rough geometrical relation calculation, considering 10° as the minimum value of alpha (elevation angle), we can see that the Doppler shift variation rate is maximum when UE is at the nadir; while the Doppler shift value, the delay shift variation rate and the delay shift value is maximum when the alpha = 10 degrees, as Table 2.2-1 shows. In addition, alpha=30° is more typically used in Rel-16 and Rel-17 NTN evaluation.

Table 2.2-1 Doppler and delay for LEO

| Scenario | LEO-600 | LEO-1200 |
| --- | --- | --- |
| Doppler shift variation rate at nadir | 0.291ppm/s | 0.123ppm/s |
| Doppler shift variation rate when alpha = 10 degrees | ~0ppm/s | ~0ppm/s |
| Doppler shift value at nadir | ~0ppm | ~0ppm |
| Doppler shift value when alpha = 10 degrees | 22.7ppm | 20.1ppm |
| delay shift variation rate at nadir | ~0 | ~0 |
| delay shift variation rate when alpha = 10 degrees | 2.27e-5 | 2.01e-5 |
| delay shift value at nadir | 0.00200s | 0.00400s |
| delay shift value when alpha = 10 degrees | 0.00644s | 0.0104s |

In our view, since the test purpose (e.g., to test maximum Doppler shift variation or test maximum delay shift variation or typical scenario) is different in different session, the satellite altitude and the UE position selection is more appropriate to be discussed during the requirements definition phase based on the demand of different session.

The satellite altitude and the UE position selection is more appropriate to be discussed during the requirements definition phase based on the demand of different session.

For the feeder link delay, we think the zero value should be more typical. In our view, the test purpose for NTN NGSO testing is to verify NTN UE performance under dynamic conditions. The feeder link delay has no impact on the test purpose, a typical value should be configured. Also in case of non-zero time-varying feeder link delay, UE may suffer additional delay error not caused by UE itself but by common TA configuration. So, we propose to not consider a non-zero time-varying feeder link delay.

Do not consider a non-zero time-varying feeder link delay.

From TE implementation point of view, the TE update granularity of the Doppler and timing offset needs to be further discussed. For example, if TE update Doppler and timing offset every 640ms, the maximum Doppler and timing offset can be about 0.18ppm and 1.47e-5s respectively for LEO-600 that is not acceptable. Also due to satellite motion during the test, the sampling rate variation may cause phase jumping within a slot that should be avoid. Therefore, to achieve accurate enough Doppler and timing offset, we propose that TE should update Doppler and timing offset per slot.

TE should update Doppler and timing offset per slot.

For the application scenario, whether to consider UE moving need to be further discussed, such as airborne and vehicle-mounted scenario, which may cause GNSS aging rapidly.

Whether to consider UE moving need to be further discussed.

For the test procedure, based on TS38.509, it is feasible for TE to inform UE position by using UE Positioning test mode procedures. Also based on UE receiving ephemeris from TE by SIB periodically, UE can perform the Doppler and delay compensation. The detailed test procedure is shown as following:

- Enter UE Positioning test mode, inform UE position that is randomly selected by TE from a pre-defined position set (including latitude and longitude).

- Based on above UE position, TE calculates initial satellite position and generates ephemeris information that is accordance with the Keplerian channel model.

- TE sends the ephemeris information to the UE via SIB19 with the period of 10.24s, and add Doppler and delay to the transmitting signal based on above channel model.

- Check whether UE can meet the requirements.

Consider above test procedure for NGSO test.

Proposals

In this contribution, we share our view on NTN testing for NGSO for demodulation performance requirements. Our observations and proposals are:

- For Eckstein-Hechler model and Keplerian model, the maximum Doppler error can be about 5Hz and the maximum delay error can be about 1.5e-8s while ephemeris is transmitted with the period of 10.24s.

For circular model, the maximum Doppler error can be about 10Hz and the maximum delay error can be about 8e-7s while ephemeris is transmitted with the period of 10.24s.

- The Doppler error and the delay error brought by the channel model should not exceed the given tolerance, i.e. frequency error and timing error shall be less than 5% of the 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s respectively within at most 10.24s.

Consider Keplerian model for NTN testing for NGSO.

The satellite altitude and the UE position selection is more appropriate to be discussed during the requirements definition phase based on the demand of different session.

Do not consider a non-zero time-varying feeder link delay.

TE should update Doppler and timing offset per slot.

Whether to consider UE moving need to be further discussed.

Consider following test procedure for NGSO test.

- Enter UE Positioning test mode, inform UE position that is randomly selected by TE from a pre-defined position set (including latitude and longitude).

- Based on above UE position, TE calculates initial satellite position and generates ephemeris information that is accordance with the Keplerian channel model.

- TE sends the ephemeris information to the UE via SIB19 with the period of 10.24s, and add Doppler and delay to the transmitting signal based on above channel model.

- Check whether UE can meet the requirements.

Reference

R4-2413516, Way Forward for [112][329] NTN_testing_NGSO_channel_model, RAN4#112, Samsung